2. Subspace Identification (N4SID, MOESP): Subspace ID methods like N4SID and MOESP let you take a large block of real plant data (inputs, outputs, disturbances), automatically distill it to the handful of true “state variables” governing your process, and extract a clean linear state-space model. This model plugs directly into observers and predictive controllers, giving you an MPC that inherently “looks ahead” across all disturbance channels—far outperforming a bank of decoupled PIDs when the plant has many interacting variables.

**When to Use**:

- Multivariable (MIMO) systems: Ideal when you have multiple inputs, outputs, and disturbance channels interacting—common in industrial plants (e.g., distillation columns, heat-exchanger networks). Don’t need to hand-choose polynomials or worry about which lag-terms to include; subspace methods discover the state dimension and system order from data.

- Black-box modeling without structural priors: When you lack a first-principles model or when the physics are too complex, subspace ID automatically builds a minimal state-space realization (A,B,C,D). Works “out of the box” without you defining basis functions or tuning model orders manually (beyond selecting the projection horizon).

- Robustness to noise and disturbances: By projecting onto dominant singular-value subspaces, these methods inherently filter measurement noise. You can incorporate measured disturbances as extra inputs and get disturbance-to-state mappings directly in B.

- Computational efficiency for large data sets: Heavy lifting reduces to QR-factorizations and SVDs of Hankel matrices—operations that are well-optimized in numerical libraries. Scales gracefully to long I/O histories and many channels, unlike iterative nonlinear optimizers.

- Good foundation for predictive control & observers: Delivers a state-space model ready for Kalman filters and linear-MPC solvers—no extra conversion step needed. Facilitates fast updates (e.g., via fast SVD or rank-updates) if you need to re-identify online.

**Key equations**: xk+1 = Axk + Buk, yk = Cxk + Duk.

**Variables defined**:

- xk+1: State vector at time k+1, in Rn, where n is system order (e.g., internal states like fluid levels).

- xk: State vector at time k, in Rn.

- A: State transition matrix, in Rn×n, describes state dynamics.

- uk: Input (MV) at time k, in Rm (m inputs, e.g., control signals).

- B: Input matrix, in Rnxm, maps inputs to state updates.

- yk: Output (PV) at time k, in R p (p outputs, e.g., sensor measurements).

- C: Output matrix, in R pxn, maps states to outputs.

- D: Feedthrough matrix, in R pxm, direct input-to-output effect (often zero in process control).

N4SID extracts A, B, C, D by projecting Hankel matrices of past inputs/outputs onto dominant subspaces via SVD. MOESP minimizes output-prediction error via orthogonal projections.

**HREF**: [https://en.wikipedia.org/wiki/Subspace_identification](https://en.wikipedia.org/wiki/Subspace_identification)

Implementing Subspace ID Algorithms (N4SID & MOESP):

Prerequisites and Installation: Python Packages:

- control: For subspace ID models n4sid & moesp.

- numpy: Numeric arrays & linear algebra.

- pandas: I/O dataset storage (csv, hdf5, etc.).

- matplotlib: Plotting for validation.

- joblib: Model persistence (save / load objects).

- osqp: Example QP solver for deployment.

Collecting and Storing I/O Datasets: When collecting from a live plant, accumulate timestamps + channel names into a pandas.DataFrame and periodically flush to HDF5 or CSV.

Python Example (Simulate Data for Demonstration): Python

import numpy as np

import pandas as pd

from control import rss, forced_response

import matplotlib.pyplot as plt

Simulate a random stable MIMO system for demonstration: Python

n, l, m = 4, 2, 2  # States, outputs, inputs

sys = rss(n, l, m)  # Random state-space system

Ts = 0.1

T_total = 1000

time = np.arange(0, T_total * Ts, Ts)

# PRBS inputs

U = (np.random.rand(m, len(time)) > 0.5).astype(float)

# Simulate outputs

_, Y, _ = forced_response(sys, T=time, U=U)

# Package into a DataFrame and save

cols = [f'u{i+1}' for i in range(m)] + [f'y{j+1}' for j in range(l)]

df = pd.DataFrame(np.vstack((U, Y)).T, columns=cols)

df.to_csv('io_data.csv', index=False)

Loading and Wrapping Data for Subspace ID Algorithms: Python

import numpy as np

import pandas as pd

import control

# Load raw CSV

df = pd.read_csv('io_data.csv')

m, l = 2, 2 # Inputs, outputs

T = len(df)

# Extract U, Y arrays (shape: channels x time)

U = df[[f'u{i+1}' for i in range(m)]].values.T  # (m, T)

Y = df[[f'y{j+1}' for j in range(l)]].values.T  # (l, T)

# Wrap in python-control's iddata (time x channels)

Ts = 0.1

data = control.iddata(y=Y.T, u=U.T, tsamp=Ts)

Choose a Model Order: Run a quick grid-search on one-step MSE – Python

def one_step_mse(sys_ss, U, Y, Ts):

time = np.arange(Y.shape[1]) * Ts

_, Y_pred = forced_response(sys_ss, T=time, U=U)

# Ensure shape match

Y_pred = Y_pred.T if Y_pred.ndim == 1 else Y_pred

return np.mean((Y - Y_pred)**2)

best_order = None

best_mse = np.inf

for n in range(1, 11):

sys_n4 = control.n4sid(data, n)

mse = one_step_mse(sys_n4, U, Y, Ts)

if mse < best_mse:

best_mse, best_order = mse, n

print(f"Optimized model order: {best_order} (MSE={best_mse:.3e})")

model_order = best_order

Identify with N4SID: Python

sys_n4sid = control.n4sid(data, model_order)

A_n4, B_n4, C_n4, D_n4 = sys_n4sid.A, sys_n4sid.B, sys_n4sid.C, sys_n4sid.D

Identify with MOESP: Python

sys_moesp = control.moesp(data, model_order)

A_mo, B_mo, C_mo, D_mo = sys_moesp.A, sys_moesp.B, sys_moesp.C, sys_moesp.D

Testing and Validation: Ensure your identified subspace-ID model captures the true process dynamics and generalizes to new data—rather than overfitting the training set. This ensures successful MPC deployment.

One-step-ahead vs. full-horizon simulation: In MPC design, use one-step error to choose model order (sensitive to missing dynamics) but rely on multi-step simulation to confirm accuracy over the prediction horizon - Python

import numpy as np

import matplotlib.pyplot as plt

from control import forced_response

# Time vector for plotting

time = np.arange(T) * Ts

# One-step-ahead prediction (uses true past outputs)

_, Y_pred = forced_response(sys_n4sid, T=time, U=U)

Y_pred = Y_pred.T if Y_pred.ndim == 1 else Y_pred  # Shape match

# Full-horizon “open-loop” simulation (only inputs)

_, Y_sim = forced_response(sys_n4sid, T=time, U=U)

Y_sim = Y_sim.T if Y_sim.ndim == 1 else Y_sim

# Plot primary output (y1)

plt.figure(figsize=(8,3))

plt.plot(time, Y[0], 'k', label='Measured y1')

plt.plot(time, Y_pred[0], 'r--', label='1-step-ahead')

plt.plot(time, Y_sim[0], 'b:', label='Full-horizon sim')

plt.xlabel('Time [s]')

plt.ylabel('y1')

plt.legend()

plt.tight_layout()

plt.show()

#Key checks: Residual whiteness - Python

e = Y - Y_pred

# Compute autocorrelate (example for y1 residuals)

from scipy.signal import correlate

autocorr = correlate(e[0], e[0], mode='full')

plt.plot(autocorr)

plt.title('Residual Autocorrelation')

plt.show()  # Peaks only at zero lag indicate whiteness

Persisting the Identified Model: Save for reuse, reproducibility, and fast deployment: Python

import joblib

# Save entire StateSpace object

joblib.dump(sys_n4sid, 'n4sid_model.pkl')

# Or save matrices

np.savez('model_matrices.npz', A=A_n4, B=B_n4, C=C_n4, D=D_n4)

# Load example

sys_loaded = joblib.load('n4sid_model.pkl')

data_loaded = np.load('model_matrices.npz')

A_loaded = data_loaded['A']

Deploying in an MPC: Turn your identified (A,B,C,D) model into finite-horizon QP matrices, solve in real time for control moves: Python

import numpy as np

import osqp

from scipy import sparse

n, m, p = A_n4.shape[0], B_n4.shape[1], C_n4.shape[0]

lookahead_time = 10.0

N = int(lookahead_time / Ts)  # Prediction horizon

# PHI (pN x n)

PHI = np.vstack([C_n4 @ np.linalg.matrix_power(A_n4, i+1) for i in range(N)])

# GAMMA (pN x mN)

GAMMA = np.zeros((p*N, m*N))

for i in range(N):

for j in range(i+1):

GAMMA[i*p:(i+1)*p, j*m:(j+1)*m] = C_n4 @

np.linalg.matrix_power(A_n4, i-j) @ B_n4

# Cost weights

Q = np.eye(p) * 1.0

R = np.eye(m) * 0.01

Q_bar = sparse.block_diag([Q]*N)

R_bar = sparse.block_diag([R]*N)

H = 2 * (GAMMA.T @ Q_bar @ GAMMA + R_bar)  # Quadratic term

# Example real-time loop (simplified; add constraints as needed)

x_k = np.zeros(n)

for k in range(T - N):

# State update (or use Kalman filter)

u_k = U[:, k]

x_k = A_n4 @ x_k + B_n4 @ u_k

# Build linear term f = 2 * GAMMA.T @ Q_bar @ PHI @ x_k (for zero SP)

f = 2 * (GAMMA.T @ Q_bar @ (PHI @ x_k))

# Setup and solve QP (no constraints here; add A, l, u for real use)

prob = osqp.OSQP()

prob.setup(P=sparse.csc_matrix(H), q=f, A=None, l=None, u=None, verbose=False)

res = prob.solve()

delta_u = res.x[:m]  # Apply first input move

# Send u_cmd = u_k + delta_u to PLC / actuator

Ongoing Maintenance:

- Online re-identification: Periodically rerun n4sid on a sliding window of recent data.

- State estimation: Wrap (A,B,C,D) in a Kalman filter (control.kalman) for noisy outputs.

- Order selection: Monitor singular-value decay and prediction MSE to adapt model_order.

3. Eigensystem Realization Algorithm (ERA):

When to Use:

- For reduced-order models from impulse/step response data; useful in vibration-heavy manufacturing (e.g., aerospace).

- Predicting process disturbances before they occur.

- Think of ERA + the SVD projection step as one streamlined recipe for “learning the handful of key motion-patterns in your process—including all your disturbance channels—and then discovering exactly how those patterns march forward in time,” so your MPC can truly “look ahead” rather than just react.

A. Collect and Build - From response data, build block-Hankel matrix:

Create matrix of past responses (measuring the Markov parameters Y(k)):

H(0) = [\begin {bmatrix} Y(1) & Y(2) & \cdots & Y(j) \\ Y(2) & Y(3) & \cdots & Y(j+1) \\ \vdots & \vdots & \ddots & \vdots \\ Y(i) & Y(i+1) & \cdots & Y(i+j-1) \end{bmatrix} \]

Perform Singular-Value Decomposition (SVD) on H(0) to reveal the dominant patterns: H(0) = U \Sigma VT, where U ∈ ℝi×i is orthonormal (UT U = I), Σ ∈ ℝi×j is diagonal with nonnegative entries σ1 ≥ σ2 ≥ ⋯, and V ∈ ℝj×j is orthonormal (VT V = I).

Form Hankel matrix: Collect time series Y(1), Y(2), …, Y(T) into an i x j Hankel matrix H(0).

Compute the Gram matrices:

Compute H(0) H(0)T, a i×i symmetric matrix.

Compute H(0)T H(0), a j×j symmetric matrix.

Solve the eigen-problems:

Find eigenvectors and eigenvalues of H(0) H(0)T: H(0) H(0)T uk = λk uk, k=1, …, i. Normalized uk form columns of U.

Find eigenvectors of H(0)T H(0): H(0)T H(0) vk = λk vk, k=1, …, j. These vk form columns of V.

Since H(0) H(0)T and H(0)T H(0) share non-zero eigenvalues λk, sort in descending order λ1 ≥ λ2 ≥ …, λk.

Build singular-value matrix: Define singular values σk = √λk, place on diagonal of Σ in descending order. If H(0) is i x j, Σ is diagonal with zeros for padding.

Verify H(0) = U Σ VT by construction: H(0) = ∑_{k=1}^{min(i,j)} σk uk (vk)T

Truncate for model reduction (optional): See Appendix 1.1.2 ‘Why Truncate SVD’ – required if reduced order extraction is needed. Often keep only first r largest singular values/vectors: H(0) ≈ Ur Σr (Vr)T = ∑_{k=1}r (σk uk (vk)T.

Interpret the factors:

U: Basis for space spanned by rows of H(0) (left singular vectors).

Σ: Gains indicating how “energetic” each mode is.

V: Basis for space spanned by columns of H(0) (right singular vectors).

Using subspace identification, split Ur = [U_{1,1} U_{2,1}], (Σr (Vr)T = [M1 M2], to recover estimates of system’s A,B,C,D matrices. But the core SVD step is the factorization H(0) = U Σ VT, computed via eigen-decompositions of the two Gram matrices, sorting by magnitude, and truncating (optional).

B. Extraction: Moving from truncated SVD factors of H(0) to reduced-order state-space realization (\hat{A}, \hat{B}, \hat{C}).

Start with truncated SVD: Keep n largest singular values/vectors of H(0) ≈ Un Σn (Vn)T, where Un ∈ ℝi×n (first n left singular vectors), Σn ∈ ℝn×n (diagonal of top n singular values), Vn ∈ ℝj×n (first n right singular vectors). This produces a rank-n approximation capturing dominant dynamics.

Create time-shifted Hankel matrix H(1): H(1) = \begin{bmatrix} Y(2) & Y(3) & \cdots & Y(j+1) \\ Y(3) & Y(4) & \cdots & Y(j+2) \\ \vdots & \vdots & \ddots & \vdots \\ Y(i+1) & Y(i+2) & \cdots & Y(i+j) \end{bmatrix}, using same block dimensions as H(0).

Compute reduced-order state matrix \hat{A} = (Σn)-1/2 UnT H(1) Vn (Σn)-1/2.

Reasoning: (Un)T H(1) Vn projects one-step-ahead data onto n principal subspaces.

Python Example (ERA Instantiation): Python - #!/usr/bin/env python3

"""

ERA Example:

- Build H(0), H(1) block-Hankel from measured Markov parameters Y(k)

- Compute SVD, truncate to order r

- Extract A_hat, B_hat, C_hat, D_hat

- Wrap in python-control StateSpace and validate via simulation

"""

import numpy as np

from scipy.linalg import svd

import control  # python-control

import matplotlib.pyplot as plt

import joblib  # for optional model persistence

# 1. Load Markov parameters (impulse/step responses)

# Assume CSV files Y1.csv, Y2.csv, … each p x T (p outputs x T samples)

# Stack so Y_all shape = (p*m, T)

m = 2 # Inputs/disturbance channels

p = 1 # Outputs

T = 200  # Samples per response

# Load example data (simulate if none)

Y_list = [np.random.rand(p, T) for _ in range(m)]  # Placeholder; replace with real data

Y_all = np.vstack(Y_list)  # (p*m, T)

# 2. Build block-Hankel matrices H0 = H(0) and H1 = H(1)

def block_hankel(Y, i, j):

"""Build block-Hankel matrix of shape (p*i, j) from Y[:, 0:(i+j-1)]."""

p, T_all = Y.shape[0], Y.shape[1]

H = np.zeros((p*i, j))

for row in range(i):

H[row*p:(row+1)*p, :] = Y[:, row:row+j]

return H

i = 10  # Block-rows

j = 40  # Block-cols (must satisfy i+j-1 <= T)

assert i + j - 1 <= T

H0 = block_hankel(Y_all, i, j)  # Past responses

H1 = block_hankel(Y_all[:,1:], i, j)  # One-step-ahead (shifted)

# 3. SVD + Truncate

U, s, Vh = svd(H0, full_matrices=False)

# Decide r by energy (e.g., 95% rule)

energy = np.cumsum(s**2) / np.sum(s**2)

r = np.searchsorted(energy, 0.95) + 1

print(f"Retaining {r} modes to capture 95% energy")

U_r = U[:, :r]  # (p*i, r)

S_r = np.diag(s[:r])  # (r, r)

V_r = Vh.conj().T[:, :r]  # (j, r)

# 4. Form reduced-order A_hat

S_inv_sqrt = np.diag(1.0 / np.sqrt(s[:r]))

A_hat = S_inv_sqrt @ (U_r.T @ H1 @ V_r) @ S_inv_sqrt

# 5. Recover C_hat and B_hat (and D = first Markov parameter)

S_sqrt = np.diag(np.sqrt(s[:r]))

C_hat = (U_r @ S_sqrt)[:p, :]  # (p x r)

B_hat = (S_sqrt @ V_r.T)[:, :m]  # (r x m) # adjust slicing if disturbances included

D_hat = Y_all[:,0].reshape(p, m)  # Direct feedthrough

# 6. Wrap in python-control StateSpace and Validate

sys_era = control.ss(A_hat, B_hat, C_hat, D_hat)  # Continuous-time; set Ts for discrete

# 6.1 One-step prediction vs measured (example validation)

time = np.arange(T)  # Assume unit sampling

U_val = Y_all[:m, :]  # Reuse as input for demo

_, y_pred = forced_response(sys_era, T=time, U=U_val)

y_pred = y_pred.T if y_pred.ndim == 1 else y_pred

residuals = Y_all[:p, :] - y_pred[:p, :]

# Plot (example for y1)

plt.figure(figsize=(8,4))

plt.plot(time, Y_all[0], 'k', label='Measured y1')

plt.plot(time, y_pred[0], 'r--', label='One-step pred')

plt.title("One-Step Prediction vs. Measured")

plt.legend()

plt.show()

# 6.2 Multi-step (free-run) simulation

_, y_sim = forced_response(sys_era, T=time, U=U_val)

y_sim = y_sim.T if y_sim.ndim == 1 else y_sim

plt.figure(figsize=(8,4))

plt.plot(time, Y_all[0], 'k', label='Measured y1')

plt.plot(time, y_sim[0], 'b:', label='Free-run sim')

plt.title("Multi-Step (Open-Loop) Simulation")

plt.legend()

plt.show()

# 7. (Optional) Save model for deployment

joblib.dump(sys_era, "era_model.pkl")

print("Saved ERA model to era_model.pkl")

Comments: Extend by adding noise-robustness via different energy levels, including disturbance channels in Y_all, deploying 'era_model.pkl' in MPC, or comparing ERA vs. N4SID/MOESP with control.n4sid/moesp.

**Variables defined**:

- H(0): Block-Hankel matrix at lag 0, in Rixj, constructed from output response data.

- Y(k): Output response at time k, in Rp (p outputs, e.g., from impulse response).

- i: Number of block rows in Hankel matrix, integer, related to data length.

- j: Number of block columns in Hankel matrix, integer.

- U: Left singular vectors from SVD, in Ri x i, orthogonal matrix.

- :Singular values, diagonal matrix in Ri x j, non-negative.

- V: Right singular vectors, in Rjx j, orthogonal matrix.

- A: Reduced-order state transition matrix, in Rnxn, where n is chosen order.

- n: Truncated singular values, in Rnxn, top n singular values.

- Un: Truncated left singular vectors, in Ri x n.

- Vn: Truncated right singular vectors, in Rj x n .

- H(1): Hankel matrix at lag 1, in Ri x j, shifted version of H(0).

**HREF**: [https://en.wikipedia.org/wiki/Eigensystem_realization_algorithm](https://en.wikipedia.org/wiki/Eigensystem_realization_algorithm)

4. Non-linear ARX Neural Net (NARX): While PID excels in linear, single-loop scenarios but falters with nonlinearities, interactions, multiple disruptive variables, or variable delays which are all common in chemical reactors, distillation columns, or batch processes. NARX provides a means of improving on this; the hybrid NARX-MPC provides superior disturbance rejection and can be implemented via edge computing for low-latency control. Below is a step-by-step blueprint for implementing NARX in Python using a Distillation Column multi-section LWVC + LWC condensing system.

When to Use: For nonlinear dynamics with known delays; extends ARX for processes like distillation condenser loops.

Base equation: y(k) = f(y(k-1), …, y(k-ny), u(k-1), …, u(k-nu)), where f is a feed-forward neural network; delays embed dynamics.

Data Collection and Preparation: Collect historical or real-time data from your facility's SCADA/PLC systems. TIP: Use IT/OT integration to stream data via MQTT to an onsite or cloud database for large-scale training, then deploy lightweight models on edge; monitor BTU trends (delta between cooling water return and supply temps) for fouling alerts.

Python Example (NumPy & scikit-learn): Python

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import MinMaxScaler

# Load data (e.g., from CSV exported from SCADA)

data = np.loadtxt('condenser_data.csv', delimiter=',')

’’’ Columns: time, u_FCV, APV_cw_flow, APV_reflux_flow, APV_cw_return_T, y_vent_T, DV_vapor_P, DV_vapor_T, DV_cw_line_P, DV_cw_supply_T, DV_fac_cw_flow ’’’

u = data[:, 1:2]  # Inputs (m=1, cooling water FCV %)

y = data[:, 5:6]  # Outputs (p=1, condenser vent temp °F)

apv_dv = data[:, 2:5]  # APVs and DVs as features if needed

# Define lags (ny=3, nu=2 based on process knowledge/delays)

ny, nu = 3, 2

# Create lagged features

def create_lagged_data(u, y, nu, ny):

X = []  # Inputs: [y(k-1)...y(k-ny), u(k-1)...u(k-nu)]

Y = []  # Targets: y(k)

for k in range(max(nu, ny), len(y)):

lagged_y = y[k-ny:k][::-1].flatten()  # Past y's

lagged_u = u[k-nu:k][::-1].flatten()  # Past u's

X.append(np.concatenate([lagged_y, lagged_u]))

Y.append(y[k])

return np.array(X), np.array(Y)

X, Y = create_lagged_data(u, y, nu, ny)

# Scale data (normalize to [0,1] for neural stability)

scaler_X = MinMaxScaler()

scaler_Y = MinMaxScaler()

X_scaled = scaler_X.fit_transform(X)

Y_scaled = scaler_Y.fit_transform(Y)

# Split: 70% train, 15% Val, 15% test

X_train, X_temp, Y_train, Y_temp = train_test_split(X_scaled, Y_scaled, test_size=0.3, shuffle=False)

X_val, X_test, Y_val, Y_test = train_test_split(X_temp, Y_temp, test_size=0.5, shuffle=False)

Define the NARX Model: NARX handles nonlinear mappings (e.g., heat transfer in condensers affected by DV vapor pressure) via activation functions like ReLU, embedding delays for lag/deadtime in cw return T (cooling water rtn temp).

Python Example (PyTorch): Python

import torch

import torch.nn as nn

import torch.optim as optim

class NARXNet(nn.Module):

def __init__(self, input_dim, output_dim, hidden_dim=64):

super(NARXNet, self).__init__()

self.fc1 = nn.Linear(input_dim, hidden_dim)  # Input: ny*p + nu*m

self.fc2 = nn.Linear(hidden_dim, hidden_dim)

self.fc3 = nn.Linear(hidden_dim, output_dim)  # Output: p

self.relu = nn.ReLU()

def forward(self, x):

x = self.relu(self.fc1(x))

x = self.relu(self.fc2(x))

return self.fc3(x)

# Dimensions: input_dim = ny * p + nu * m (e.g., 3*1 + 2*1 = 5)

p, m = 1, 1

input_dim = ny * p + nu * m

model = NARXNet(input_dim, p)

# To GPU if available

device = torch.device("cuda" if torch.cuda.is_available() else "cpu")

model.to(device)

Train the Model: Use MSE loss for regression, Adam optimizer. Retrain periodically with OT data streams to adapt to DVs (e.g., cw supply T fluctuations), using IT cloud for hyperparameter tuning and BTU delta analysis for fouling prediction.

Python Example (PyTorch): Python

# Convert to tensors

X_train_tensor = torch.tensor(X_train, dtype=torch.float32).to(device)

Y_train_tensor = torch.tensor(Y_train, dtype=torch.float32).to(device)

# Similarly for val

criterion = nn.MSELoss()

optimizer = optim.Adam(model.parameters(), lr=0.001)

epochs = 200

for epoch in range(epochs):

model.train()

optimizer.zero_grad()

outputs = model(X_train_tensor)

loss = criterion(outputs, Y_train_tensor)

loss.backward()

optimizer.step()

# Validate

model.eval()

with torch.no_grad():

val_outputs = model(torch.tensor(X_val, type=torch.float32).to(device))

val_loss = criterion(val_outputs, torch.tensor(Y_val, dtype=torch.float32).to(device))

print(f"Epoch {epoch+1}, Train Loss: {loss.item():.4f}, Val Loss: {val_loss.item():.4f}")

# Save model

torch.save(model.state_dict(), 'narx_condenser.pth')

Validate and Tune: Simulate predictions; compare against PID baselines using metrics like MSE or FIT%. Use SciPy for residual analysis.

Python Example (SciPy & python-control): Python

from scipy.stats import norm  # For residual whiteness test

import numpy as np

import control

import matplotlib.pyplot as plt

# Predict on test

model.eval()

with torch.no_grad():

Y_pred_scaled = model(torch.tensor(X_test, dtype=torch.float32).to(device)).cpu().numpy()

Y_pred = scaler_Y.inverse_transform(Y_pred_scaled)

# MSE (assume Y_test unscaled as Y_test_unscaled)

mse = np.mean((Y_test_unscaled - Y_pred)**2)

# Residuals

residuals = Y_test_unscaled - Y_pred

# Whiteness test (e.g., autocorrelation for first PV)

from scipy.signal import correlate

autocorr = correlate(residuals[:,0], residuals[:,0], mode='full')

plt.plot(autocorr)

plt.title('Autocorrelation of Residuals')

plt.show()  # Peaks only at zero lag indicate whiteness

# Compare to PID (simulate baseline)

Kp, Ti, Td = 1.0, 1.0, 0.25  # Tune via Z-N

# s from sympy or assume

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate (placeholder; adapt to your data)

time = np.arange(len(Y_test_unscaled))

u_sim = np.ones(len(time))  # Example input

_, y_pid = control.forced_response(pid, T=time, U=u_sim)

# Compare errors, e.g., pid_mse = np.mean((Y_test_unscaled - y_pid)**2)

Deploy in Industrial Facility: Integrate into OT loop: Load model on edge device, predict y(k) in real-time, use as feedforward in advanced control (e.g., adjust FCV based on predictions, factoring DVs like vapor pressure).

Python Example (Real-time Inference): Python

# Load model

model.load_state_dict(torch.load('narx_condenser.pth'))

model.eval()

# Real-time loop (e.g., in PLC gateway script)

while True:

# Get current lagged data from sensors (e.g., via OPC UA)

# Update buffers (vent T, FCV)

current_lagged = np.concatenate([past_y.flatten(), past_u.flatten()])

input_tensor = torch.tensor(scaler_X. Transform([current_lagged]), dtype=torch.float32).to(device)

pred = model(input_tensor).cpu().numpy()

# Predicted PPV (vent T)

pred_y = scaler_Y.inverse_transform(pred)[0]

# Use in control: e.g., adjust FCV if pred_y deviates from 149°F SP

# Send to PLC

time.sleep(Ts)  # Sampling time

Monitor, Retrain, and Innovate: Monitor via plc-gbt analytics (e.g., MSE thresholds trigger alerts). Retrain weekly with new data to handle DVs. Innovations for US Manufacturing: Hybrid NARX-SINDy for interpretable nonlinear terms (use pysindy to sparsify f); integrate with RL (e.g., SAC via stable-baselines3) for policy optimization over NARX predictions, incorporating APV purity and DV vapor temp. This enables autonomous, resilient control—propelling efficiency and dominance.

**Variables defined**:

- yk: Predicted output (PV) at time k, in Rp.

- f: Nonlinear function (neural network), maps inputs to output, unitless.

- y(k-1), …, y(k-ny): Past outputs at times k-1 to k-ny, in Rp.

- ny: Number of lagged outputs, integer.

- u(k-1), …, u(k-nu): Past inputs (MVs) at times k-1 to k-nu, in Rm.

- nu: Number of lagged inputs, integer.

**HREF**: [Nonlinear_autoregressive_model](https://en.wikipedia.org/wiki/Nonlinear_autoregressive_model) & [modeling-and-prediction-with-narx-and-time-delay-networks]( https://www.mathworks.com/help/deeplearning/modeling-and-prediction-with-narx-and-time-delay-networks.html)

5. Recurrent Neural Net (LSTM): LSTM empowers complex control operations with sequence-aware intelligence.

When to Use:

- For sequential data with long dependencies; innovative for predictive maintenance in IT/OT-integrated systems.

- LSTM shines in processes like your alcohol condenser, where historical patterns (e.g., persistent DV impacts from vapor temperature or cooling water line pressure) influence current states, enabling better handling of deadtimes (e.g., in cw return T) than feed-forward models like NARX.

- Use when PID fails on multivariable, time-varying disturbances, such as fouling trends or supply temp variations, to predict vent T deviations and preemptively adjust FCV for stable 149°F SP.

**Equations**:

- Forget gate: ft = σ(Wf [ht-1, xt] + bf)

- Input gate: it = σ(Wi [ht-1, xt] + bi)

- Cell update: ct = tanhWc[ht-1, xt] + bc, ct = ft ⋅ ct-1 + it ⋅ ct

- Output gate: ot = σ(Wo [ht-1, xt] + bo), ht = ot ⋅ tanh(ct), where xt includes past y, u.

**Variables defined**:

- ft: Forget gate activation at time t, in h (h hidden units), range [0,1].

- σ: Sigmoid function, maps to [0,1], unitless.

- Wf: Weight matrix for forget gate, in Rh(h+d), where d is input dimension.

- ht-1: Previous hidden state, in Rh.

- xt: Input at time t, in Rd (includes past y, u).

- bf: Bias for forget gate, in Rh.

- it: Input gate activation, in Rh, range [0,1].

- Wi: Weight matrix for input gate, in Rh(h+d).

- bi: Bias for input gate, in Rh.

- ct: Candidate cell state, in Rh.

- tanh: Hyperbolic tangent function, maps to [-1,1], unitless.

- Wc: Weight matrix for cell update, in Rh(h+d).

- bc: Bias for cell update, in Rh.

- ct: Cell state at time t, in Rh.

- ct-1: Previous cell state, in Rh.

- \odot: Hadamard (element-wise) product, unitless.

- ot: Output gate activation, in Rh, range [0,1].

- Wo: Weight matrix for output gate, in Rh(h+b).

- bo: Bias for output gate, in Rh.

- ht: Hidden state (output) at time t, in Rh.

**HREF**: [Long_short-term_memory](https://en.wikipedia.org/wiki/Long_short-term_memory)

Data Collection and Preparation: Collect historical or real-time data from your facility's SCADA/PLC systems.

Python Example (NumPy & scikit-learn): Python

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import MinMaxScaler

# Load data (e.g., from CSV exported from SCADA)

data = np.loadtxt('condenser_data.csv', delimiter=',')

# Columns: time, u_FCV, APV_cw_flow, APV_reflux_flow, APV_cw_return_T, y_vent_T, DV_vapor_P, DV_vapor_T, DV_cw_line_P, DV_cw_supply_T, DV_fac_cw_flow

u = data[:, 1:2]  # Inputs (m=1, cooling water FCV %)

y = data[:, 5:6]  # Outputs (p=1, condenser vent temp °F)

apv_dv = data[:, 2:5]  # APVs and DVs as features if needed

# Define sequence length (seq_len=10 for long dependencies)

seq_len = 10

# Create sequences

def create_sequences(u, y, seq_len):

X = []  # Sequences: [[x_{t-seq_len+1}, ..., x_t]] where x_t includes u(t), y(t-1), etc.

Y = []  # Targets: y(t+1) for prediction

for t in range(seq_len, len(y)):

# Shape: (seq_len, m + p)

seq_x = np.concatenate([u[t-seq_len:t], y[t-seq_len:t-1]], axis=1)

X.append(seq_x)

Y.append(y[t])

return np.array(X), np.array(Y)

X, Y = create_sequences(u, y, seq_len)

# Scale data

scaler_X = MinMaxScaler()

scaler_Y = MinMaxScaler()

X_scaled = scaler_X.fit_transform(X.reshape(-1, X.shape[-1])).reshape(X.shape)

Y_scaled = scaler_Y.fit_transform(Y)

# Split: 70% train, 15% val, 15% test

X_train, X_temp, Y_train, Y_temp = train_test_split(X_scaled, Y_scaled, test_size=0.3, shuffle=False)

X_val, X_test, Y_val, Y_test = train_test_split(X_temp, Y_temp, test_size=0.5, shuffle=False)

Define the LSTM Model:

Python Example (PyTorch): Python

import torch

import torch.nn as nn

import torch.optim as optim

class LSTMNet(nn.Module):

def __init__(self, input_dim, hidden_dim, output_dim, num_layers=1):

super(LSTMNet, self).__init__()

self.hidden_dim = hidden_dim

self.num_layers = num_layers

# Input: batch x seq_len x input_dim

self.lstm = nn.LSTM(input_dim, hidden_dim, num_layers, batch_first=True)

# Output from last hidden state

self.fc = nn.Linear(hidden_dim, output_dim)

def forward(self, x):

h0 = torch.zeros(self.num_layers, x.size(0), self.hidden_dim).to(x.device)

c0 = torch.zeros(self.num_layers, x.size(0), self.hidden_dim).to(x.device)

out, _ = self.lstm(x, (h0, c0))  # out: batch x seq_len x hidden_dim

out = self.fc(out[:, -1, :])  # Last time step

return out

# Dimensions: input_dim = m + p -1 (e.g., 1 + 1 = 2 for u_FCV and past y_vent_T)

input_dim = 2

hidden_dim = 64

output_dim = 1  # p=1 for vent T

model = LSTMNet(input_dim, hidden_dim, output_dim)

# To GPU if available

device = torch.device("cuda" if torch.cuda.is_available() else "cpu")

model.to(device)

Train the Model:

Python Example (PyTorch): Python

# Convert to tensors

X_train_tensor = torch.tensor(X_train, dtype=torch.float32).to(device)

Y_train_tensor = torch.tensor(Y_train, dtype=torch.float32).to(device)

# Similarly for val

criterion = nn.MSELoss()

optimizer = optim.Adam(model.parameters(), lr=0.001)

epochs = 200

for epoch in range(epochs):

model.train()

optimizer.zero_grad()

outputs = model(X_train_tensor)

loss = criterion(outputs, Y_train_tensor)

loss.backward()

optimizer.step()

# Validate

model.eval()

with torch.no_grad():

val_outputs = model(torch.tensor(X_val, dtype=torch.float32).to(device))

val_loss = criterion(val_outputs, torch.tensor(Y_val, dtype=torch.float32).to(device))

print(f"Epoch {epoch+1}, Train Loss: {loss.item():.4f}, Val Loss: {val_loss.item():.4f}")

# Save model

torch.save(model.state_dict(), 'lstm_condenser.pth')

Validate and Tune:

Python Example (SciPy & python-control): Python

from scipy.stats import norm  # For residual whiteness test

import numpy as np

import control

import matplotlib.pyplot as plt

# Predict on test

model.eval()

with torch.no_grad():

Y_pred_scaled = model(torch.tensor(X_test, dtype=torch.float32).to(device)).cpu().numpy()

Y_pred = scaler_Y.inverse_transform(Y_pred_scaled)

# MSE

# Assume unscaled Y_test as Y_test_unscaled

mse = np.mean((Y_test_unscaled - Y_pred)**2)

# Residuals

residuals = Y_test_unscaled - Y_pred

# Whiteness test (e.g., autocorrelation for first PV)

from scipy.signal import correlate

autocorr = correlate(residuals[:,0], residuals[:,0], mode='full')

plt.plot(autocorr)

plt.title('Autocorrelation of Residuals')

plt.show()  # Peaks only at zero lag indicate whiteness

# Compare to PID (simulate baseline)

Kp, Ti, Td = 1.0, 1.0, 0.25  # Tune via Z-N

# s from sympy or assume

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate (placeholder; adapt to your data)

time = np.arange(len(Y_test_unscaled))

u_sim = np.ones(len(time))  # Example input

_, y_pid = control.forced_response(pid, T=time, U=u_sim)

# Compare errors, e.g., pid_mse = np.mean((Y_test_unscaled - y_pid)**2)

Deploy for Monitoring: Integrate into OT loop: Load model on edge device, predict future vent T in real-time, use as feedforward in advanced control (e.g., adjust FCV based on sequences, factoring DVs like vapor pressure).

Python Example (Real-time Inference): Python

# Load model

model.load_state_dict(torch.load('lstm_condenser.pth'))

model.eval()

# Real-time loop (e.g., in PLC gateway script)

sequence buffer = []  # Maintain rolling sequence of last seq_len steps

while True:

# Get current data from sensors (e.g., via OPC UA): append to buffer

current_xt = np.array([current_u_FCV, prev_y_vent_T])  # xt includes u, past y

sequence_buffer.append(current_xt)

if len(sequence_buffer) > seq_len:

sequence_buffer.pop(0)

input_seq = np.array([sequence_buffer])  # Shape: 1 x seq_len x input_dim

input_tensor = torch.tensor(scaler_X.transform(input_seq.reshape(-1, input_dim)).reshape(1, seq_len, input_dim), dtype=torch.float32).to(device)

pred = model(input_tensor).cpu().numpy()

pred_y = scaler_Y.inverse_transform(pred)[0]  # Predicted PPV (vent T)

# Use in control: e.g., adjust FCV if pred_y deviates from 149°F SP

# Send to PLC

time.sleep(Ts)  # Sampling time

Monitor, Retrain, and Innovate:

-Monitor via plc-gbt analytics (e.g., MSE thresholds trigger alerts).

-Retrain weekly with new data to handle DVs.

-Innovations for Manufacturing: Hybrid LSTM-SINDy for interpretable long-term terms (use pysindy on LSTM features); integrate with RL (e.g., SAC via stable-baselines3) for policy optimization over LSTM predictions, incorporating APV purity and DV vapor temp. This enables autonomous, resilient control—propelling efficiency and optimization

6. Gaussian Process Regression (GPR):

When to Use: For uncertainty quantification in small-data regimes; pairs well with MPC for robust manufacturing. GPR is an apt choice for use cases with sparse data and high uncertainty, as its non-parametric modeling handles nonlinearity and provides variance estimates—e.g., in tower supply water control for predicting pressure with confidence bounds under DVs like basin levels.

Base equation: y(x) ~ GP(m(x), k(x, x'))

Variables:

- y(x): Output function, in R, evaluated at input x (e.g., process output).

- x: Input vector, in Rd (e.g., process conditions, controls).

- GP: Gaussian Process, a distribution over functions, unitless.

- Instead of assuming a fixed functional form (e.g. linear, polynomial), a GP defines a probability distribution over all possible functions that fit the data. Any finite collection of function values follows a multivariate Gaussian distribution. http://www.gaussianprocess.org/gpml/

- m(x): Mean function, in R, prior mean of output at x (often 0).

- k(x, x'): Covariance (kernel) function, in R, measures similarity between inputs x, x' (e.g., squared exponential kernel).

- x': Another input vector, in Rd, for covariance computation.

**HREF**:[Gaussian_process_regression](https://en.wikipedia.org/wiki/Gaussian_process_regression)

Data Collection and Preparation: Collect historical data; for tower: States (pressure, flow, temps, levels), actions (VFD speed).

Python Example (NumPy & scikit-learn): Python

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import StandardScaler

# Load data (e.g., from CSV)

# Columns: time, u_VFD, y_pressure, supply_flow, mash_cooler_flow1, ...

data = np.loadtxt('tower_data.csv', delimiter=',')

u = data[:, 1:2]  # Inputs (m=1, VFD speed %)

y = data[:, 2:3]  # Outputs (p=1, supply pressure psi)

features = data[:, 3:]  # Additional vars/DVs

# Combine inputs: X = [u, features]

X = np.hstack([u, features])  # (samples, input_dim)

# Scale data

scaler_X = StandardScaler()

scaler_Y = StandardScaler()

X_scaled = scaler_X.fit_transform(X)

Y_scaled = scaler_Y.fit_transform(y)

# Split: 70% train, 30% test

X_train, X_test, Y_train, Y_test = train_test_split(X_scaled, Y_scaled, test_size=0.3, random_state=42)

Define the GPR Model:

Python Example (scikit-learn): Python

from sklearn.gaussian_process import GaussianProcessRegressor

from sklearn.gaussian_process.kernels import RBF, ConstantKernel, WhiteKernel

# Define kernel: Constant * RBF + White (for noise)

kernel = ConstantKernel(1.0) * RBF(1.0, length_scale_bounds=(1e-2, 1e3)) + WhiteKernel(noise_level=1e-5)

model = GaussianProcessRegressor(kernel=kernel, n_restarts_optimizer=10, alpha=0.01)

Train the Model:

Python Example (scikit-learn): Python

# Fit model

model.fit(X_train, Y_train)

print(f"Optimized kernel: {model.kernel_}")

Validate and Tune:

Python Example (SciPy & python-control): Python

from scipy.stats import norm  # For confidence intervals

import numpy as np

import control

import matplotlib.pyplot as plt

# Predict on test: mean and std

Y_pred, sigma = model.predict(X_test, return_std=True)

Y_pred_unscaled = scaler_Y.inverse_transform(Y_pred.reshape(-1,1)).flatten()

sigma_unscaled = sigma * scaler_Y.scale_  # Approximate scaling

# MSE

mse = np.mean((Y_test_unscaled - Y_pred_unscaled)**2)

# Coverage: Check if true values within 2*sigma (95% CI)

coverage = np.mean(np.abs(Y_test_unscaled - Y_pred_unscaled) < 2 * sigma_unscaled)

# Residuals

residuals = Y_test_unscaled - Y_pred_unscaled

from scipy.signal import correlate

autocorr = correlate(residuals, residuals, mode='full')

plt.plot(autocorr)

plt.title('Autocorrelation of Residuals')

plt.show()

# Compare to PID (simulate baseline)

Kp, Ti, Td = 1.0, 1.0, 0.25  # Tune via Z-N

# s from sympy or assume

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate (placeholder; adapt to your data)

time = np.arange(len(Y_test_unscaled))

u_sim = np.ones(len(time))  # Example input

_, y_pid = control.forced_response(pid, T=time, U=u_sim)

# Compare errors, e.g., pid_mse = np.mean((Y_test_unscaled - y_pid)**2)

Deploy for monitoring: Fit model on edge (GPR is lightweight), predict pressure with uncertainty, adjust VFD if low confidence (e.g., govern mash cooler flow).

Python Example (Real-time Inference): Python

# Assume model fitted; for real-time, refit periodically

# Real-time loop (e.g., in PLC gateway script)

while True:

# Get current features from sensors (e.g., via OPC UA)

current_X = np.array([current_u_VFD, current_supply_flow, ...])  # All inputs/DVs

input_scaled = scaler_X.transform([current_X])

pred_scaled, sigma_scaled = model.predict(input_scaled, return_std=True)

# Predicted pressure

pred_y = scaler_Y.inverse_transform(pred_scaled.reshape(1,-1))[0][0]

sigma = sigma_scaled[0] * scaler_Y.scale_[0]  # Uncertainty

# Use in control: e.g., adjust VFD if pred_y < SP or sigma > threshold

# Send to PLC

time.sleep(Ts)  # Sampling time

Monitor, Retrain, and Innovate:

-Monitor via plc-gbt analytics (e.g., uncertainty thresholds trigger alerts).

-Retrain daily with new data to handle DVs.

-Innovations for Manufacturing: Hybrid GPR-PILCO for uncertainty-aware RL (use GPy if expanded); integrate with SINDy for sparse dynamics discovery on GPR priors, incorporating process flows for demand forecasting. This enables autonomous, probabilistic control—propelling efficiency and dominance.

7. Sparse Identification of Nonlinear Dynamics (SINDy):

When to Use: For discovering sparse governing equations; innovative for physics-informed ML in automation. SINDy is ideal for dynamical systems like boiler steam pressure control, where data reveals a few key nonlinear terms (e.g., quadratic pressure dependencies or interactions with temperature), enabling interpretable models over black-box alternatives. Use when PID underperforms on complex feedbacks (e.g., saturation effects or load disturbances), to derive equations that inform MPC or fault detection—especially in sparse-data scenarios with physical intuition, like ensuring stable pressure amid varying fuel quality or demand spikes.

Base equation: x = θ(x) ξ

Variables defined:

- x: Time derivative of state vector, in Rn, rate of change (e.g., velocity).

- x: State vector, in Rn (e.g., process states).

- θ(x): Library matrix of candidate functions, in Rnxm (m functions, e.g., polynomials x, x2.

- xi: Sparse coefficient vector, in Rm, selects active terms (mostly zeros).

**HREF**: [SINDy](https://en.wikipedia.org/wiki/SINDy)

Data Collection and Preparation: Collect historical data from your boiler loop: States (pressure, level, temp), MVs (valve %).

Python Example (NumPy & SciPy): Python

import numpy as np

from scipy.signal import savgol_filter  # For smoothing/derivatives

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import StandardScaler

# Load data (e.g., from CSV)

# Columns: time, u_valve, x_pressure, drum_level, temp, fuel_flow, ...

data = np.loadtxt('boiler_data.csv', delimiter=',')

t = data[:, 0]  # Time

x = data[:, 1:4]  # States (n=3, pressure, level, temp)

# Compute derivatives (dot_x) using Savitzky-Golay filter

dt = np.mean(np.diff(t))

dot_x = savgol_filter(x, window_length=5, polyorder=2, deriv=1, delta=dt, axis=0)

# Scale data

scaler_x = StandardScaler()

scaler_dot_x = StandardScaler()

x_scaled = scaler_x.fit_transform(x)

dot_x_scaled = scaler_dot_x.fit_transform(dot_x)

# Split: 80% train, 20% test

x_train, x_test, dot_x_train, dot_x_test = train_test_split(x_scaled, dot_x_scaled, test_size=0.2, shuffle=False)

Define the SINDy Model:

Python Example (pysindy): Python

from pysindy import SINDy

from pysindy.feature_library import PolynomialLibrary

# Define library: polynomials degree 3

library = PolynomialLibrary(degree=3)

# Instantiate model with thresholding optimizer

# Sparse threshold

model = SINDy(feature_library=library, optimizer='STLSQ', optimizer__threshold=0.1)

Train the Model:

Python Example (pysindy): Python

# Fit model

model.fit(x_train, t=dt, x_dot=dot_x_train)  # dt from data

# Print equations

model.print()  # Example output: x0' = a x0 + b x1^2 + ...

Validate and Tune:

Python Example (SciPy & python-control): Python

from scipy.integrate import odeint

import numpy as np

import matplotlib.pyplot as plt

# Simulate with SINDy equations

def sindy_eq(x, t):

return model.predict(x.reshape(1,-1)).flatten()  # Unscaled predict

x_sim = odeint(sindy_eq, x_test[0], t_test)  # Integrate (t_test from test time)

# MSE

mse = np.mean((x_test - x_sim)**2)

# Residuals (for pressure, index 0)

residuals = x_test[:,0] - x_sim[:,0]

from scipy.signal import correlate

autocorr = correlate(residuals, residuals, mode='full')

plt.plot(autocorr)

plt.title('Autocorrelation of Residuals')

plt.show()

# Compare to PID (simulate baseline)

import control

Kp, Ti, Td = 1.0, 1.0, 0.25  # Tune via Z-N

# s from sympy or assume

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate (placeholder; adapt to your data)

time = np.arange(len(x_test))

u_sim = np.ones(len(time))  # Example input

_, y_pid = control.forced_response(pid, T=time, U=u_sim)

# Compare errors, e.g., pid_mse = np.mean((x_test[:,0] - y_pid)**2)

Deploy for monitoring: Load model on edge, predict x for forward simulation, adjust valve if pressure trends unsafe.

Python Example (Real-time Inference): Python

# Assume model fitted; for real-time, refit periodically

# Real-time loop (e.g., in PLC gateway script)

while True:

# Get current states from sensors (e.g., via OPC UA)

current_x = np.array([current_pressure, current_level, current_temp])  # Scaled

dot_x_pred = model.predict(current_x.reshape(1,-1))[0]  # Predicted derivatives

# Simulate short horizon

future_t = np.linspace(0, 60, 10)  # 1-min ahead

future_x = odeint(lambda x,t: model.predict(x.reshape(1,-1)).flatten(), current_x, future_t)

# Use in control: e.g., adjust valve if future pressure > max

# Send to PLC

time.sleep(Ts)  # Sampling time

Monitor, Retrain, and Innovate:

-Monitor via plc-gbt analytics (e.g., equation sparsity for model health).

-Retrain weekly with new data to handle DVs.

-Innovations for Manufacturing: Hybrid SINDy-Koopman for sparse observables (use kooplearn on SINDy terms); integrate with RL (e.g., PETS via ensembles) for policy optimization over SINDy equations, incorporating fuel flow for demand forecasting. This enables autonomous, interpretable control—propelling efficiency and optimization.

8. Koopman Operator Approaches:

When to Use: For linearizing nonlinear systems; enables linear MPC on lifted states for complex dynamics.

Base equation: φ(xk+1) ∼ K φ(xk)

Variables defined:

- φ(xk+1): Observable functions at time k+1, in Rm (m observables, e.g., nonlinear transformations of x).

- φ(xk): Observable functions at time k, in Rm.

- K: Koopman operator, in Rmxm, linear operator in observable space.

- xk: State vector at time k, in Rn.

**HREF**: [Koopman_operator](https://en.wikipedia.org/wiki/Koopman_operator)

Data Collection and Preparation: Collect historical data from your mash cooler loop: States (inlet/mid/outlet temps, mash flow), MVs (FCV 1/2 positions).

Python Example (NumPy & scikit-learn): Python

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import StandardScaler

# Load data (e.g., from CSV)

# Columns: time, x_inlet_T, x_mid_T, x_outlet_T, x_mash_flow, u_FCV1, u_FCV2, DV_cook_cycle, ...

data = np.loadtxt('mash_cooler_data.csv', delimiter=',')

t = data[:, 0]  # Time

x = data[:, 1:5]  # States (n=4, inlet T, mid T, outlet T, mash flow)

u = data[:, 5:7]  # Inputs (m=2, FCV1, FCV2)

# Augment states with u

xu = np.hstack([x, u])  # Extended states

# Scale data

scaler = StandardScaler()

xu_scaled = scaler.fit_transform(xu)

# Split: 70% train, 30% test

xu_train, xu_test = train_test_split(xu_scaled, test_size=0.3, shuffle=False)

Define the Koopman Model:

Python Example (pykoopman): Python

from pykoopman import Koopman

from pykoopman.observables import Polynomial

from pykoopman.regression import EDMD

# Define observables: polynomials degree 3

observables = Polynomial(degree=3)

# Define regressor: EDMD

regressor = EDMD()

# Instantiate model

model = Koopman(observables=observables, regressor=regressor)

Train the Model: Fit Koopman on consecutive state pairs; extract operator K.

What's Happening: We slice training data into pairs: current states xk and next states xk+1. This creates transitions capturing the system's evolution. The model lifts states to observables φ(xk) (e.g., polynomials for nonlinear terms like temp-flow interactions in MC01), then regresses K to approximate φ xk+1 ≈ K φ (xk). Under the hood, it solves a matrix equation via least-squares or SVD, minimizing error. Why? This embeds nonlinearity in lifting, allowing linear propagation in high dimensions—enabling efficient prediction for MC01's dual sections and SP weighting.

Python Example (pykoopman): Python

# Prepare data: x_k and x_{k+1}

x_train = xu_train[:-1]  # Current states

x_next_train = xu_train[1:]  # Next states

# Fit model

model.fit(x_train, x_next_train)

# Access K (linear operator)

K = model.A  # Discrete-time Koopman matrix

print(f"Koopman operator shape: {K.shape}")

Validate and Tune:

Python Example (SciPy & python-control): Python

from scipy.linalg import eig

import numpy as np

import matplotlib.pyplot as plt

# Predict multi-step

x_pred = np.zeros_like(x_test)

x_pred[0] = x_test[0]

for i in range(1, len(x_test)):

psi_current = model.observables.transform(x_pred[i-1].reshape(1,-1))

psi_next = psi_current @ model.A.T  # Linear propagation

x_pred[i] = model.observables.inverse(psi_next)  # Back to original space

# MSE

mse = np.mean((x_test - x_pred)**2)

# Stability: Eigenvalues of K

eigenvalues = eig(model.A)[0]

print(f"Max eigenvalue magnitude: {np.max(np.abs(eigenvalues))}")  # <1 for stability

# Compare to PID (simulate baseline)

import control

Kp, Ti, Td = 1.0, 1.0, 0.25  # Tune via Z-N

# s from sympy or assume

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate (placeholder; adapt to your data)

time = np.arange(len(x_test))

u_sim = np.ones(len(time))  # Example input

_, y_pid = control.forced_response(pid, T=time, U=u_sim)

# Compare errors, e.g., pid_mse = np.mean((x_test[:,0] - y_pid)**2)

plt.plot(time, x_test[:,0], label='True')

plt.plot(time, y_pid, label='PID Sim')

plt.legend()

plt.show()

Deploy for monitoring: Integrate into OT loop: Load model on edge, propagate lifted states for prediction, adjust FCV via linear control on \( \psi \), factoring SP weighting.

Python Example (Real-time Inference): Python

# Assume model fitted; for real-time, refit periodically

# Real-time loop (e.g., in PLC gateway script)

current_x = np.array([current_inlet_T, current_mid_T, current_outlet_T, current_mash_flow, current_FCV1, current_FCV2])  # Scaled

while True:

psi_current = model.observables.transform(current_x.reshape(1,-1))

psi_future = psi_current @ model.A.T  # One-step ahead

future_x = model.observables.inverse(psi_future)[0]  # Predicted states

# Use in control: e.g., linear MPC on psi to adjust FCV for target outlet T, applying SP weighting

# Send to PLC

current_x = future_x  # Update for next (or from sensors)

time.sleep(Ts)  # Sampling time

Monitor, Retrain, and Innovate:

-Monitor via plc-gbt analytics (e.g., eigenvalue stability for model health).

-Retrain weekly with new data to handle DVs.

-Innovations for Manufacturing: Hybrid Koopman-SINDy for sparse observables (use pysindy on lifted space); integrate with RL (e.g., DDPG via stable-baselines3) for policy optimization on linear K, incorporating cook cycle DVs for demand forecasting. This enables autonomous, linearized control—propelling efficiency and dominance.

9. Model-Based Reinforcement Learning (PILCO, PETS):

When to Use: For optimal control under uncertainty; bridges RL with process models for adaptive manufacturing. PILCO and PETS are ideal for dynamic, stochastic loops like tower supply water control, where limited data and high variability (e.g., sudden process calls from condensers or fermenters) demand sample-efficient learning with uncertainty quantification. Use PILCO for Gaussian Process-based modeling in small-data regimes (e.g., probabilistic predictions of pressure under basin level fluctuations); opt for PETS when neural ensembles better capture complex interactions (e.g., trajectory sampling for planning VFD speeds amid temp risks). Apply when PID fails on long-term optimization (e.g., balancing flow across consumers while avoiding over-temp shutdowns), especially in safety-critical setups needing expected return gradients or uncertainty propagation for robust governing.

Why Use: PILCO and PETS shine in distillery operations by addressing PID's shortcomings in uncertain, multivariable systems—providing model-based RL that learns dynamics and policies with few trials, incorporating uncertainty to avoid catastrophic failures (e.g., insufficient cooling leading to lost batches). Sample efficiency: Unlike model-free RL (e.g., SAC), PILCO uses GPR for analytic gradients; PETS leverages ensembles for uncertainty-aware planning. Uncertainty handling: Quantify risks (variances or sampled trajectories) for safe exploration (e.g., conservative VFD ramps). Interpretability and innovation: Allow physics-informed priors, fostering IT/OT hybrids like edge execution for flow allocation. Over SINDy/Koopman, add RL for goal-oriented optimization; vs. GPR alone, close the loop with policy learning—crucial for dominance through autonomous, risk-aware systems.

How to Use: We'll implement PILCO and PETS for tower supply water: Learn dynamics to optimize VFD actions for pressure SP, rewarding flow satisfaction while penalizing temp exceeds, using gymnasium for env.

Data Collection and Preparation:

Python Example (gymnasium & NumPy): Python

import numpy as np

import gymnasium as gym

from gymnasium import spaces

class TowerEnv(gym.Env):

def __init__(self):

super().__init__()

# Normalized VFD speed

self.action_space = spaces.Box(low=0, high=1, shape=(1,))

self.observation_space = spaces.Box(low=np.array([0, 0, 0, 0]), high=np.array([75, 3200, 212, 212]), shape=(4,))  # pressure, flow, supply T, return T

self.state = np.array([50, 1600, 70, 85])  # Initial state

def step(self, action):

vfd = action[0] * 100  # Denormalize

self.state[0] += 0.5 * vfd - 0.1 * (self.state[3] - self.state[2])  # Pressure update

self.state[1] += 10 * vfd  # Flow update

# Track 60 psi, penalize temp >80F

reward = -abs(self.state[0] - 60) - 10 * max(0, self.state[2] - 80)

done = False

return self.state, reward, done, {}

def reset(self):

self.state = np.array([50, 1600, 70, 85]) + np.random.uniform(-10, 10, 4)

return self.state, {}

env = TowerEnv()

# Collect initial data (random policy rollout)

states, actions, next_states, rewards = [], [], [], []

state = env.reset()

for _ in range(1000):

action = env.action_space.sample()

next_state, reward, _, _ = env.step(action)

states.append(state)

actions.append(action)

next_states.append(next_state)

rewards.append(reward)

state = next_state

states = np.array(states)

actions = np.array(actions)

next_states = np.array(next_states)

rewards = np.array(rewards)

Implement PILCO (GPR-Based):

Python Example (GPy & SciPy): Python

import GPy

from scipy.optimize import minimize

# Fit GPR dynamics (one for each state dim)

models = []

for i in range(states.shape[1]):

X = np.hstack([states, actions])

Y = next_states[:, i:i+1] - states[:, i:i+1]  # Delta states

kernel = GPy.kern.RBF(input_dim=X.shape[1], variance=1., lengthscale=1.)

gp = GPy.models.GPRegression(X, Y, kernel)

gp.optimize(messages=False)

models.append(gp)

# Policy (linear; optimize via gradients)

def policy(params, state):

return np.clip(np.dot(state, params[:4]) + params[4], 0, 1)  # Example

def expected_return(params, horizon=10):

state = env.reset()

total_r = 0

for _ in range(horizon):

action = policy(params, state)

mean_next, var_next = [], []

for i, gp in enumerate(models):

inp = np.hstack([state, action]).reshape(1, -1)

m, v = gp.predict(inp)

mean_next.append(state[i] + m[0][0])

var_next.append(v[0][0])

next_state = np.array(mean_next)  # Use mean (or sample for variance)

reward = -abs(next_state[0] - 60) - 10 * max(0, next_state[2] - 80)

total_r += reward

state = next_state

return -total_r  # Minimize negative return

initial_params = np.random.randn(5)

result = minimize(expected_return, initial_params, method='L-BFGS-B')

optimal_params = result.x

Implement PETS (Ensemble-Based): Use the revised snippet from our previous exchange (with PyTorch, as clarified).

Validate and Tune:

Python Example (gymnasium): Python

def evaluate(policy_func, episodes=10):

total_rewards = []

for _ in range(episodes):

state = env.reset()

ep_reward = 0

for _ in range(100):

action = policy_func(state)  # e.g., policy(optimal_params, state) for PILCO

state, reward, _, _ = env.step([action])

ep_reward += reward

total_rewards.append(ep_reward)

return np.mean(total_rewards)

pilco_return = evaluate(lambda s: policy(optimal_params, s))

pets_return = evaluate(lambda s: cem_planning(s))  # From PETS snippet

print(f"PILCO avg return: {pilco_return}, PETS: {pets_return}")

Deploy in Industrial Facility: Load models on edge, compute actions via PILCO gradients or PETS sampling, adjust VFD if uncertainty high.

Python Example (Real-time Loop): Python

while True:

current_state = get_sensor_data()  # e.g., via OPC UA

action = cem_planning(current_state)  # Or PILCO policy

env.step([action])  # Send to PLC

time.sleep(Ts)

Monitor, Retrain, and Innovate:

-Monitor returns/uncertainty via plc-gbt (variance thresholds trigger alerts).

-Retrain episodically.

-Innovations: Fuse PETS with SINDy for sparse priors; integrate PILCO with Koopman for linearized uncertainty—enabling autonomous governing (e.g., throttling mash coolers if temp risk high).

Variables defined:

- f: Dynamics model (GPR for PILCO, neural ensemble for PETS), maps state-action to next state, unitless.

- xt: State at time t, in Rn.

- ut: Action (MV) at time t, in Rm.

- rt: Reward at time t, in R, performance metric (e.g., tracking error).

- For PILCO: See GPR variables (m(x), k(x, x')).

- For PETS: Ensemble predictions, uses p(xt+1 | xt, ut).

**HREF (PILCO)**: [pilco](https://mlg.eng.cam.ac.uk/pilco/)

**HREF(PETS)**: [pets](https://arxiv.org/abs/1805.12114)

10. Policy-Gradient RL (DDPG, TD3, SAC):

When to Use: For continuous-action spaces in uncertain environments; model-free for direct policy learning.

Example, we will use still column distillate proofing control: A highly uncertain process where alcohol concentration (proof) must be maintained (e.g., 100 -160 proof SP) with a significant and variable lag and tau.  Low Wine Average proof setpoint = 130 and High Wine Average proof setpoint = 140. Amid many potential disturbance variables causing varying disruption, like varying beerfeed composition (total mash density, %ABV, recipe differences, etc.), still temperature drifts due to varying steam supply pressure, column pressure fluctuations, column beer bottom level, and others which cause risk of producing off-spec product or energy waste.

Using DDPG/TD3/SAC the model can learn deterministic or stochastic policies to adjust multiple MVs /CVs (e.g., reflux ratios or steam inputs) for optimal proof tracking or weighting to other PID or MPC MV adjustments.  Incorporating an exploration release of these models can allow learning and safe adaptation.

The deadtime and lag between Column vapor temperature and vapor condensing back to liquid then flowing thru the proMass proofing meter is very long and dependent on its own set of disturbance variables such as reflux rate, cooling water flow and temperature, vapor concentration, ect. This algorithm can unlock novel and innovative IT/OT hybrid solutions taking advantage of edge agents adjacent to PLCs for real-time policy execution amid long lags, fused with custom analytics platforms like plc-gbt for handling composition DVs, driving precise, resilient, and optimized US spirit production through emerging RL in IIoT ecosystems.

Base equations: (DDPG/TD3) Actor μ(s | θ^μ), critic \( Q(s, a | θ ^Q). (SAC) E[tr_t + αH(π(⋅ | st))]

Variables defined:

- μ(s | θ^μ): Deterministic policy (actor), maps state s to action a, in Rm.

- s: State, in Rn (e.g., process states and observations).

- θ^μ: Parameters of actor network, in Rp.

- Q(s, a | θ ^Q): Action-value function (critic), in R, estimates expected return.

- a: Action (MV), in Rm.

- θ ^Q: Parameters of critic network, in Rq.

- rt: Reward at time t, in R .

- α: Entropy regularization coefficient, in R, balances exploration.

- H(π(⋅ | st)): Entropy of policy π, in R, measures policy randomness.

- π(⋅ | st): Stochastic policy, distribution over actions given state st.

- E: Expectation operator, unitless.

**HREF (DDPG)**: [DDP_Gradient](https://en.wikipedia.org/wiki/Deep_Deterministic_Policy_Gradient)

**HREF (TD3)**: [td3](https://arxiv.org/abs/1802.09477)

**HREF (SAC)**: [sac](https://arxiv.org/abs/1812.05905)

Data Collection and Environment Setup: Define a custom env simulating proofing with lags.

Python Example (gymnasium & NumPy): Python

import gymnasium as gym

from gymnasium import spaces

import numpy as np

from collections import deque

class ProofingEnv(gym.Env):

def __init__(self, lag_steps=20):  # Simulate long lag/deadtime

super().__init__()

self.action_space = spaces.Box(low=0, high=1, shape=(2,))  # Normalized reflux, steam

# Proof, temp °F, pressure psi, %ABV

self.observation_space = spaces.Box(low=np.array([0, 0, 0, 0]), high=np.array([200, 212, 100, 20]), shape=(4,))

self.state = np.array([130, 180, 50, 10])  # Initial

self.sp_proof = 130  # Low Wine SP (or 140 for High)

self.lag_buffer = deque(maxlen=lag_steps)

self.lag_steps = lag_steps

def step(self, action):

self.lag_buffer.append(action.copy())

if len(self.lag_buffer) < self.lag_steps:

delayed_action = np.zeros(2)

else:

delayed_action = self.lag_buffer[0]

reflux, steam = delayed_action

# Proof update with noise

self.state[0] += 0.5 * reflux + 0.2 * steam - 0.1 * (self.state[1] - 180) + 0.05 * np.random.normal()

# Temp update with %ABV DV

self.state[1] += 0.3 * steam + 0.1 * (self.state[3] - 10)

# Track SP, penalize energy

reward = - (self.state[0] - self.sp_proof)**2 - 0.1 * steam

done = abs(self.state[0] - self.sp_proof) < 5

truncated = False

return self.state, reward, done, truncated, {}

def reset(self, seed=None):

self.state = np.array([130, 180, 50, 10]) + np.random.uniform(-20, 20, 4)

self.lag_buffer.clear()

return self.state, {}

env = ProofingEnv(lag_steps=20)  # 20-step lag for deadtime

Implement DDPG:

Python Example (stable-baselines3): Python

from stable_baselines3 import DDPG

from stable_baselines3.common.noise import NormalActionNoise

n_actions = env.action_space.shape[-1]

action_noise = NormalActionNoise(mean=np.zeros(n_actions), sigma=0.1 * np.ones(n_actions))

model_ddpg = DDPG("MlpPolicy", env, action_noise=action_noise, verbose=1)

model_ddpg.learn(total_timesteps=10000, log_interval=10)

model_ddpg.save("ddpg_proofing")

Implement TD3:

Python Example (stable-baselines3):  Python

from stable_baselines3 import TD3

model_td3 = TD3("MlpPolicy", env, verbose=1)

model_td3.learn(total_timesteps=10000, log_interval=10)

model_td3.save("td3_proofing")

Implement SAC:

Python Example (stable-baselines3): Python

from stable_baselines3 import SAC

model_sac = SAC("MlpPolicy", env, verbose=1)

model_sac.learn(total_timesteps=10000, log_interval=10)

model_sac.save("sac_proofing")

Validate and Tune:

Python Example (stable-baselines3): Python

def evaluate(model, episodes=10):

rewards = []

for _ in range(episodes):

obs, _ = env.reset()

ep_reward = 0

done = False

while not done:

action, _ = model.predict(obs, deterministic=True)

obs, reward, done, _, _ = env.step(action)

ep_reward += reward

rewards.append(ep_reward)

return np.mean(rewards)

ddpg_return = evaluate(model_ddpg)

td3_return = evaluate(model_td3)

sac_return = evaluate(model_sac)

print(f"DDPG avg return: {ddpg_return}, TD3: {td3_return}, SAC: {sac_return}")

Deploy for testing: Load models on edge, query policies for actions; monitor for overestimation.

Python Example (Real-time Loop): Python

model = SAC.load("sac_proofing")  # Or DDPG/TD3

while True:

obs = get_sensor_data()  # e.g., via OPC UA: proof, temp, pressure, %ABV

action, _ = model.predict(obs, deterministic=True)

env.step(action)  # Send to PLC (reflux, steam)

time.sleep(Ts)

Monitor, Retrain, and Innovate:

-Monitor returns/variance via plc-gbt (e.g., TD3 critics for bias alerts).

-Retrain episodically.

-Innovate: Fuse SAC entropy with SINDy for sparse priors; integrate DDPG with Koopman for linearized actors—enabling policy optimization amid deadtimes (e.g., adjusting reflux if proof uncertainty from composition DVs threatens yield).

11. Classical Proportional-Integral-Derivative (PID) Tuning Rules:

When to Use: For single-loop control; baseline before MPC in legacy systems.

- i) Ziegler–Nichols (Z-N): Ultimate gain Ku, period Tu; Kp = 0.6 Ku, Ti = 0.5 Tu, Td = 0.125 Tu. **HREF**: [Ziegler–Nichols_method](https://en.wikipedia.org/wiki/Ziegler–Nichols_method)

- ii) Cohen–Coon: From step response lag τ, time constant T.

**HREF**: [pid-control/cohen-coon-tuning-rules/](https://control.com/textbook/pid-control/cohen-coon-tuning-rules/)

- iii) λ-Tuning (IMC-based): Choose closed-loop λ; (Kp = T / (K (λ + τ)), etc.

**HREF**:[Internal_model_contro_Lambda_tuning](https://en.wikipedia.org/wiki/Internal_model_control#Lambda_tuning)

Variables defined:

- Kp: Proportional gain, unitless or scaled to process units.

- Ku: Ultimate gain, unitless, gain causing sustained oscillations.

- Ti: Integral time, in seconds, for reset action.

- Tu: Ultimate period, in seconds, oscillation period at Ku.

- Td: Derivative time, in seconds, for rate action.

- T: Process time constant, in seconds, from step response.

- K: Process gain, unitless or in output/input units.

- λ (lambda): Closed-loop time constant, in seconds, user-specified.

- τ (tau): Process lag, in seconds, from step response.

12. Internal Model Control (IMC): The IMC framework is a model-based control strategy that leverages a process model to design a controller, offering robustness against model mismatches and disturbances, particularly in processes with significant delays

When to Use: For processes with delays; robust against model mismatch.

Base equations:

Decompose GpType equation here.(s) ≈ G_(s) G+(s) (minimum-phase and all-pass).

Controller: Gc(s) = G-1_(s) ∙ 1 / 1 + λs.

Variables defined:

- Decompose: The process of splitting Gp(s) into minimum-phase (G_(s)) and all-pass (G+(s)) components to simplify controller design.

- Gp(s): Process transfer function, in R (freq domain), maps inputs to outputs. Forms Laplace domain (s) that maps inputs (Manipulated / Control Variables, MVs, like valve positions) to outputs (Process Variables, PVs, like temperature or pressure). It captures the dynamic behavior of the physical system.

- s: Laplace variable, in s-1.

- G_(s): Minimum-phase part of process model, in R.  Example, in a first-order plus dead-time (FOPDT) model common in process control: Gp(s) = Ke−τs / τps + 1. The minimum-phase part is: G_ (s)= K / τps + 1 because the denominator τps + 1 (pole at s = −1/τp) and gain K are stable and invertible. Python lib – control handles this with Python:

import control

K, tau_p = 1.0, 2.0

G_minus = control.tf(K, [tau_p, 1]) # K / (tau_p s + 1)

G_minus_inv = control.tf([tau_p, 1], [K]) # (tau_p s + 1) / K

- G+(s): All-pass part (e.g., delays), in R. Includes non-invertible elements, such as: Time delays (e.g., e−τs e^{-\tau s} e−τs), Right-half-plane zeros, which lead to unstable inverses. For the FOPDT example: G+(s) =e−τs, This has magnitude ∣e−τs∣ = 1 (all-pass property, affecting only phase, not gain). ID of delay or non-minimum-phase element using the python control lib. Python:

tau = 1.0

G_plus = control.tf([1], [1], deadtime=tau) # e^(-tau s)

- Controller:

- G_c(s): Controller transfer function, in R.

- G_-1(s): Inverse of minimum-phase model, in R.

- λ: Filter time constant, in seconds.

**HREF**: [Internal_model_control](https://en.wikipedia.org/wiki/Internal_model_control)

### Implement IMC Model: The IMC framework designs a controller by leveraging a model of the process, making it robust to model mismatches and effective for systems with delays, which is critical for distillery processes (e.g., controlling condenser vent temperature or still side proofing). The equations are listed above.​

-Review of subscripts and decomposition usage:

-Role of G_(s) (Minimum-Phase Component): includes all stable dynamics of Gp(s) that can be inverted. This typically means:

-Poles in the left-half s-plane (stable, with negative real parts).

-Zeros in the left-half s-plane (if any), which are invertible.

For example, in a first-order plus dead-time (FOPDT) model common in process control: Gp(s) = Ke−τs / τps + 1,​ The minimum-phase part is: G_(s) = K / τps + 1 because the denominator τps + 1 (pole at s = −1 / τp = -1 and gain K are stable and invertible.

Why It Matters: The controller inverts G_(s) (i.e., computes G_−1(s) to cancel the process’s invertible dynamics, aiming for perfect setpoint tracking in an ideal case (if the model matches the process exactly).

Manipulation: Compute the inverse: For G_(s) = K / τps + 1​, G_−1(s) = τps + 1 / K This is stable and proper (numerator degree = denominator degree), making it suitable for control.

In practice, use tools like Python’s control library to handle this - Python:

import control

K, tau_p = 1.0, 2.0

G_minus = control.tf(K, [tau_p, 1])# K / (tau_p s + 1)

G_minus_inv = control.tf([tau_p, 1], [K])# (tau_p s + 1)/K

Role of G+(s) (All-Pass Component): includes non-invertible elements, such as: Time delays, which are non-causal if inverted (requiring future inputs). This typically means:

Right-half-plane zeros, which lead to unstable inverses.

For example (in FOPDT): G+(s) = e−τs. This has magnitude ∣e−τs ∣ =1 (all-pass property, affecting only phase, not gain).

Why It Matters: Inverting G+(s) (e.g., eτs) is impractical because it requires future data or introduces instability. Thus, IMC leaves G+(s) out of the controller design, accepting the delay or phase shift as a limitation of the physical system.

Manipulation: Identify the delay or non-minimum-phase elements during model identification (e.g., via step tests or subspace methods like N4SID).

Do not invert G+(s); it remains in the closed-loop response, causing the output to track the setpoint with a delay: y(s) / r(s) ≈ G+(s) ⋅ 1 / 1 + λs = e−τs / 1 + λs.

In Python, model the delay as follows: Python

tau = 1.0

G_plus = control.tf([1], [1], deadtime=tau)  # e^(-tau s)

Decomposition Process: the act of factoring Gp(s) into G_(s) and G+(s) components to separate invertible and non-invertible dynamics. This is critical for designing a stable controller.

How to: Start with the process model Gp(s), obtained from system identification (e.g., step response, ARX, or N4SID, as detailed above in section X).

Analyze the poles and zeros:

Poles/zeros in the left-half plane go to G_(s).

Time delays and right-half-plane zeros go to G+(s).

FOPDT example: Gp(s) = Ke−τs / τps + 1 = (K / τps + 1) ⋅ (e−τs) = G_(s)G+(s).

For complex systems, use numerical tools to factorize. Python:

# G_p(s) = G_-(s) * G_+(s)

G_p = control.series(G_minus, G_plus)

Purpose: This separation ensures the controller Gc(s) = G_−1(s) ⋅ 1 / 1+λs only inverts the stable, causal dynamics, avoiding instability or non-causality.

Controller Gc(s): The controller equation is listed above with a purpose of canceling the minimum-phase dynamics.

1 / 1+λs acts as a low-pass filter with time constant λ (seconds), ensuring:

Stability by attenuating high-frequency noise.

Robustness to model mismatches,

Proper transfer function (finite high-frequency gain).

Manipulation:

Compute G_−1(s), as shown above.

Choose λ based on process dynamics (e.g., λ ≈ τp for balance, or larger for robustness in noisy environments like a distillation column with fluctuating pressures.

Form the controller: Gc(s) = τps + 1 / K ⋅ 1 / 1+λs = τps + 1 / K(1 + λs). Discretize for real-time implementation: Python

lambda_val = 2.0

G_c = control.series(control.tf([tau_p, 1], [K]), control.tf([1], [lambda_val, 1]))

Ts = 0.1

G_c_d = control.c2d(G_c, Ts, method='tustin')

num, den = G_c_d.num[0][0], G_c_d.den[0][0]

Implement in a control loop: Python

u_prev, e_prev = 0, 0

while True:

y = get_sensor_data() #e.g., vent temp via OPC UA

r = 149  # SP in °F

y_tilde = simulate_model(u_prev)#Model prediction

e = r - (y - y_tilde)  # IMC error

u = (num[0]*e + num[1]*e_prev - den[1]*u_prev) / den[0]

send_to_plc(u)  # Adjust flow

u_prev, e_prev = u, e

time.sleep(Ts)

13. Relay Feedback:

When to Use: For auto-tuning PID without open-loop tests. Relay induces limit cycles; infer Ku, Tu for Z-N tuning.

Variables defined:

- K: Ultimate gain, unitless, from relay-induced limit cycle.

- Tu: Ultimate period, in seconds, from limit cycle.

**HREF**: [Relay_autotuning](https://en.wikipedia.org/wiki/Relay_autotuning)

14. Genetic Algorithm (GA):

When to Use: For optimizing non-differentiable objectives, like tuning hybrid controllers. Evolve population Θi via selection, crossover, mutation to minimize J(\theta).

Variables defined:

- JΘ: Objective function, in R, performance metric (e.g., control error).

- Θ: Parameter vector, in Rp, candidate solution (e.g., controller gains).

- Θi: Population of parameter vectors, set of vectors in Rp.

**HREF**: [wiki/Genetic_algorithm](https://en.wikipedia.org/wiki/Genetic_algorithm)

### Revised Draft: OVERVIEW - Algorithms for Process Control:  This draft serves as a comprehensive learning / how-to guide, for controls engineers to gain familiarity with algorithms most relevant to process control, machine learning, and model predictive control.  The intent is to facilitate a transformation from standard OT practice to a more sustainable and long-term IT OT converged practice.  Facilitating the use of well-developed tools  more familiar to the IT space.  Included are how to guides to implement, train, deploy, and monitor algorithms in a highly dynamic system.

# OVERVIEW: Algorithms for Process Control

This mathematically focused practical overview outlines models and algorithms relevant to process control in manufacturing, emphasizing IT/OT convergence. We cover system identification for building dynamic models from data, then transition to control strategies like Model Predictive Control (MPC). References provide deeper dives. The document culminates in a detailed, end-to-end Python-based example for creating an MPC algorithm: from data collection to building, training, testing, and deploying a multivariable controller that outperforms traditional PID loops.  This level of control is not necessary for every control loop in a process, but it provides a far more robust means of dealing with complex processes dependent on  multiple input variables and changing conditions.

Key innovations: Hybrid approaches (e.g., augmenting linear models with neural residuals) enable real-time adaptation on edge devices, bridging OT sensors with IT infra or cloud analytics for resilient US manufacturing.

## Glossary of Acronyms

- ARMAX: AutoRegressive Moving Average with eXogenous inputs

- ARX: AutoRegressive with eXogenous inputs

- CV: Controlled Variable (used as Manipulated Variable in some contexts, synonymous with MV)

- DDPG: Deep Deterministic Policy Gradient

- DV: Disturbance Variable

- ERA: Eigensystem Realization Algorithm

- FIT: Fit Percentage (a validation metric)

- GA: Genetic Algorithm

- GPR: Gaussian Process Regression

- IMC: Internal Model Control

- ISE: Integral of Squared Error

- IT: Information Technology

- LASSO: Least Absolute Shrinkage and Selection Operator

- LSTM: Long Short-Term Memory

- MAE: Mean Absolute Error

- MOESP: Multivariable Output-Error State Space

- MPC: Model Predictive Control

- MSE: Mean Squared Error

- MV: Manipulated Variable

- N4SID: Numerical algorithms for Subspace State Space System IDentification

- NARX: Non-linear AutoRegressive with eXogenous inputs

- OT: Operational Technology

- PETS: Probabilistic Ensembles with Trajectory Sampling

- PID: Proportional-Integral-Derivative

- PILCO: Probabilistic Inference for Learning Control

- PLC: Programmable Logic Controller

- PRBS: Pseudo-Random Binary Sequence

- PV: Process Variable

- QP: Quadratic Program

- RL: Reinforcement Learning

- RLS: Recursive Least Squares

- SAC: Soft Actor-Critic

- SINDy: Sparse Identification of Nonlinear Dynamics

- SP: Set Point

- SVD: Singular Value Decomposition

- TD3: Twin Delayed Deep Deterministic Policy Gradient

- Z-N: Ziegler–Nichols

## Algorithms for Process Control: Each entry includes a brief "When to Use" note for practical application in manufacturing.

### 1. AutoRegressive with eXogenous Inputs (ARX / ARMAX)

When to Use:

- For simple linear processes with known inputs/outputs: Ideal for quick baseline models in stable manufacturing lines. If your process behaves approximately linearly over the range of interest (e.g., small deviations of temperature, flow, or pressure), an ARX/ARMAX model can capture that behavior with just a handful of parameters. Suitable for single-loop or modestly multivariable systems where nonlinearities are mild and can be treated as disturbances.

- For fast, on-line identification needs: ARX models can be estimated via recursive least squares in real time, allowing you to track slow drifts or step changes in process gain, delay, or noise characteristics. Useful when the plant dynamics change over time—e.g., fouling, catalyst deactivation, or equipment aging—and you want to auto-tune your controller.

- Low computational burden: The parameter estimation and prediction computations boil down to linear algebra (matrix multiplications and inversions of small orders), making ARX/M models eminently suitable for implementation directly in PLCs or low-power embedded hardware.

- Explicit noise modeling with ARMAX: When measurement noise or unmeasured disturbances corrupt your outputs, augmenting ARX to ARMAX (adding a moving-average noise model) lets you separate true dynamics from stochastic fluctuations, improving prediction accuracy in MPC or Kalman-filter observers.

- A foundation for simple predictive control: Even a basic receding-horizon controller built on an ARX model often outperforms PID under disturbance, since it can anticipate future outputs based on both current and past disturbance channels.

Model Form (ARX):

y(k) + i=1n_aa_iy(k-i) = j=1n_bb_ju(k-j+1)+e(k)

ARMAX adds a moving-average noise term:

y(k) = B(q-1) / A(q-1) u(k-1) + C(q-1) / A(q-1) e(k), where A, B, C are polynomials in the backward-shift operator q-1.

Variables defined:

- y(k): Output (Process Variable, PV) at discrete time step k, typically a scalar or vector in  R (e.g., temperature, pressure). Represents the measured system response.

- k: Discrete time index, integer (unitless).

- na: Order of autoregressive terms, integer, number of past outputs used (e.g., na = 2 means two lagged outputs).

- ai: ARX coefficients for past outputs, scalar in R, weights for y(k-i), for i = 1, …, na.

- y(k-i): Past output at time k-i, same units as y(k).

- nb: Order of exogenous input terms, integer, number of past inputs used.

- bj: Input coefficients, scalar in R , weights for u(k-j+1), for j = 1, …, nb.

- u(k-j+1): Input (Manipulated Variable, MV) at time k-j+1, scalar or vector in R (e.g., valve position, flow rate).

- e(k): Noise or error term at time k, scalar or vector in R , assumed white noise (zero mean, finite variance).

- B(q-1): Polynomial in backward-shift operator q-1, defined as B(q-1 =  b1 q-1 + b2 q-2 + … + bn_b q-n_b, unitless coefficients.

- A(q-1): Polynomial in backward-shift oper, A(q-1) = 1 + a1 q-1 + … + an_a q-n_a, unitless coefficients.

- C(q-1): Polynomial for moving-average noise, C(q-1) = 1 + c1 (q-1) + … + cn_c q-n_c, unitless coefficients.

- (q-1): Backward-shift operator, shifts signal back by one time step (e.g., q-1 u(k) = u(k-1)).

- u(k-1): Input at previous time step, same as u(k-j+1) for j=1.

**HREF**: https://en.wikipedia.org/wiki/Autoregressive_exogenous_model](https://en.wikipedia.org/wiki/Autoregressive_exogenous_model)

### 2. Subspace Identification (N4SID, MOESP): Subspace ID methods like N4SID and MOESP let you take a large block of real plant data (inputs, outputs, disturbances), automatically distill it to the handful of true “state variables” governing your process, and extract a clean linear state-space model. This model plugs directly into observers and predictive controllers, giving you an MPC that inherently “looks ahead” across all disturbance channels—far outperforming a bank of decoupled PIDs when the plant has many interacting variables.

**When to Use**:

- Multivariable (MIMO) systems: Ideal when you have multiple inputs, outputs, and disturbance channels interacting—common in industrial plants (e.g., distillation columns, heat-exchanger networks). Don’t need to hand-choose polynomials or worry about which lag-terms to include; subspace methods discover the state dimension and system order from data.

- Black-box modeling without structural priors: When you lack a first-principles model or when the physics are too complex, subspace ID automatically builds a minimal state-space realization (A,B,C,D). Works “out of the box” without you defining basis functions or tuning model orders manually (beyond selecting the projection horizon).

- Robustness to noise and disturbances: By projecting onto dominant singular-value subspaces, these methods inherently filter measurement noise. You can incorporate measured disturbances as extra inputs and get disturbance-to-state mappings directly in B.

- Computational efficiency for large data sets: Heavy lifting reduces to QR-factorizations and SVDs of Hankel matrices—operations that are well-optimized in numerical libraries. Scales gracefully to long I/O histories and many channels, unlike iterative nonlinear optimizers.

- Good foundation for predictive control & observers: Delivers a state-space model ready for Kalman filters and linear-MPC solvers—no extra conversion step needed. Facilitates fast updates (e.g., via fast SVD or rank-updates) if you need to re-identify online.

**Key Equations**:

xk+1 = Axk + Buk, yk = Cxk + Duk.

**Variables**:

- xk+1: State vector at time k+1, in Rn, where n is system order (e.g., internal states like fluid levels).

- xk: State vector at time k, in Rn.

- A: State transition matrix, in Rn×n, describes state dynamics.

- uk: Input (MV) at time k, in Rm (m inputs, e.g., control signals).

- B: Input matrix, in Rnxm, maps inputs to state updates.

- yk: Output (PV) at time k, in R p (p outputs, e.g., sensor measurements).

- C: Output matrix, in R pxn, maps states to outputs.

- D: Feedthrough matrix, in R pxm, direct input-to-output effect (often zero in process control).

N4SID extracts A, B, C, D by projecting Hankel matrices of past inputs/outputs onto dominant subspaces via SVD. MOESP minimizes output-prediction error via orthogonal projections.

**HREF**: https://en.wikipedia.org/wiki/Subspace_identification](https://en.wikipedia.org/wiki/Subspace_identification)

#### Implementing Subspace ID Algorithms (N4SID & MOESP):

**Prerequisites and Installation**: Python Packages: pip install control numpy pandas matplotlib joblib osqp (assumed available in environment; no installs needed here).

- control: For subspace ID models n4sid & moesp.

- numpy: Numeric arrays & linear algebra.

- pandas: I/O dataset storage (csv, hdf5, etc.).

- matplotlib: Plotting for validation.

- joblib: Model persistence (save / load objects).

- osqp: Example QP solver for deployment.

**Collecting and Storing I/O Datasets**: When collecting from a live plant, accumulate timestamps + channel names into a pandas.DataFrame and periodically flush to HDF5 or CSV.

**Python Example** (Simulate Data for Demonstration):

import numpy as np

import pandas as pd

from control import rss, forced_response

import matplotlib.pyplot as plt

# Simulate a random stable MIMO system for demonstration

n, l, m = 4, 2, 2  # States, outputs, inputs

sys = rss(n, l, m)  # Random state-space system

Ts = 0.1

T_total = 1000

time = np.arange(0, T_total * Ts, Ts)

# PRBS inputs

U = (np.random.rand(m, len(time)) > 0.5).astype(float)

# Simulate outputs

_, Y, _ = forced_response(sys, T=time, U=U)

# Package into a DataFrame and save

cols = [f'u{i+1}' for i in range(m)] + [f'y{j+1}' for j in range(l)]

df = pd.DataFrame(np.vstack((U, Y)).T, columns=cols)

df.to_csv('io_data.csv', index=False)

**Loading and Wrapping Data for Subspace ID Algorithms**: Python

import numpy as np

import pandas as pd

import control

# Load raw CSV

df = pd.read_csv('io_data.csv')

m, l = 2, 2 # Inputs, outputs

T = len(df)

# Extract U, Y arrays (shape: channels x time)

U = df[[f'u{i+1}' for i in range(m)]].values.T  # (m, T)

Y = df[[f'y{j+1}' for j in range(l)]].values.T  # (l, T)

# Wrap in python-control's iddata (time x channels)

Ts = 0.1

data = control.iddata(y=Y.T, u=U.T, tsamp=Ts)

**Choose a Model Order**: Run a quick grid-search on one-step MSE – Python

def one_step_mse(sys_ss, U, Y, Ts):

time = np.arange(Y.shape[1]) * Ts

_, Y_pred = forced_response(sys_ss, T=time, U=U)

# Ensure shape match

Y_pred = Y_pred.T if Y_pred.ndim == 1 else Y_pred

return np.mean((Y - Y_pred)**2)

best_order = None

best_mse = np.inf

for n in range(1, 11):

sys_n4 = control.n4sid(data, n)

mse = one_step_mse(sys_n4, U, Y, Ts)

if mse < best_mse:

best_mse, best_order = mse, n

print(f"Optimized model order: {best_order} (MSE={best_mse:.3e})")

model_order = best_order

**Identify with N4SID**: Python

sys_n4sid = control.n4sid(data, model_order)

A_n4, B_n4, C_n4, D_n4 = sys_n4sid.A, sys_n4sid.B, sys_n4sid.C, sys_n4sid.D

**Identify with MOESP**: Python

sys_moesp = control.moesp(data, model_order)

A_mo, B_mo, C_mo, D_mo = sys_moesp.A, sys_moesp.B, sys_moesp.C, sys_moesp.D

**Testing and Validation**: Ensure your identified subspace-ID model captures the true process dynamics and generalizes to new data—rather than overfitting the training set. This ensures successful MPC deployment.

One-step-ahead vs. full-horizon simulation: In MPC design, use one-step error to choose model order (sensitive to missing dynamics) but rely on multi-step simulation to confirm accuracy over the prediction horizon.

Python

import numpy as np

import matplotlib.pyplot as plt

from control import forced_response

# Time vector for plotting

time = np.arange(T) * Ts

# One-step-ahead prediction (uses true past outputs)

_, Y_pred = forced_response(sys_n4sid, T=time, U=U)

Y_pred = Y_pred.T if Y_pred.ndim == 1 else Y_pred  # Shape match

# Full-horizon “open-loop” simulation (only inputs)

_, Y_sim = forced_response(sys_n4sid, T=time, U=U)

Y_sim = Y_sim.T if Y_sim.ndim == 1 else Y_sim

# Plot primary output (y1)

plt.figure(figsize=(8,3))

plt.plot(time, Y[0], 'k', label='Measured y1')

plt.plot(time, Y_pred[0], 'r--', label='1-step-ahead')

plt.plot(time, Y_sim[0], 'b:', label='Full-horizon sim')

plt.xlabel('Time [s]')

plt.ylabel('y1')

plt.legend()

plt.tight_layout()

plt.show()

# Key checks: Residual whiteness - Python

e = Y - Y_pred

# Compute autocorrelate (example for y1 residuals)

from scipy.signal import correlate

autocorr = correlate(e[0], e[0], mode='full')

plt.plot(autocorr)

plt.title('Residual Autocorrelation')

plt.show()  # Peaks only at zero lag indicate whiteness

**Persisting the Identified Model**: Save for reuse, reproducibility, and fast deployment.

Python

import joblib

# Save entire StateSpace object

joblib.dump(sys_n4sid, 'n4sid_model.pkl')

# Or save matrices

np.savez('model_matrices.npz', A=A_n4, B=B_n4, C=C_n4, D=D_n4)

# Load example

sys_loaded = joblib.load('n4sid_model.pkl')

data_loaded = np.load('model_matrices.npz')

A_loaded = data_loaded['A']

**Deploying in an MPC Loop**: Turn your identified (A,B,C,D) model into finite-horizon QP matrices, solve in real time for control moves.

Python

import numpy as np

import osqp

from scipy import sparse

n, m, p = A_n4.shape[0], B_n4.shape[1], C_n4.shape[0]

lookahead_time = 10.0

N = int(lookahead_time / Ts)  # Prediction horizon

# PHI (pN x n)

PHI = np.vstack([C_n4 @ np.linalg.matrix_power(A_n4, i+1) for i in range(N)])

# GAMMA (pN x mN)

GAMMA = np.zeros((p*N, m*N))

for i in range(N):

for j in range(i+1):

GAMMA[i*p:(i+1)*p, j*m:(j+1)*m] = C_n4 @

np.linalg.matrix_power(A_n4, i-j) @ B_n4

# Cost weights

Q = np.eye(p) * 1.0

R = np.eye(m) * 0.01

Q_bar = sparse.block_diag([Q]*N)

R_bar = sparse.block_diag([R]*N)

H = 2 * (GAMMA.T @ Q_bar @ GAMMA + R_bar)  # Quadratic term

# Example real-time loop (simplified; add constraints as needed)

x_k = np.zeros(n)

for k in range(T - N):

# State update (or use Kalman filter)

u_k = U[:, k]

x_k = A_n4 @ x_k + B_n4 @ u_k

# Build linear term f = 2 * GAMMA.T @ Q_bar @ PHI @ x_k (for

zero SP)

f = 2 * (GAMMA.T @ Q_bar @ (PHI @ x_k))

# Setup and solve QP (no constraints here; add A, l, u for real use)

prob = osqp.OSQP()

prob.setup(P=sparse.csc_matrix(H), q=f, A=None, l=None, u=None, verbose=False)

res = prob.solve()

delta_u = res.x[:m]  # Apply first input move

# Send u_cmd = u_k + delta_u to PLC / actuator

**Ongoing Maintenance**:

- Online re-identification: Periodically rerun n4sid on a sliding window of recent data.

- State estimation: Wrap (A,B,C,D) in a Kalman filter (control.kalman) for noisy outputs.

- Order selection: Monitor singular-value decay and prediction MSE to adapt model_order.

### 3. Eigensystem Realization Algorithm (ERA)

**When to Use**:

- For reduced-order models from impulse/step response data; useful in vibration-heavy manufacturing (e.g., aerospace).

- Predicting process disturbances before they occur.

- Think of ERA + the SVD projection step as one streamlined recipe for “learning the handful of key motion-patterns in your process—including all your disturbance channels—and then discovering exactly how those patterns march forward in time,” so your MPC can truly “look ahead” rather than just react.

**A. Collect and Build**: From response data, build block-Hankel matrix –

Create matrix of past responses (measuring the Markov parameters Y(k)):

H(0) = [\begin {bmatrix} Y(1) & Y(2) & \cdots & Y(j) \\ Y(2) & Y(3) & \cdots & Y(j+1) \\ \vdots & \vdots & \ddots & \vdots \\ Y(i) & Y(i+1) & \cdots & Y(i+j-1) \end{bmatrix} \]

Perform Singular-Value Decomposition (SVD) on H(0) to reveal the dominant patterns: H(0) = U \Sigma VT,

Where U ∈ ℝi×i is orthonormal (UT U = I), Σ ∈ ℝi×j is diagonal with nonnegative entries σ1 ≥ σ2 ≥ ⋯, and V ∈ ℝj×j is orthonormal (VT V = I).

- Form Hankel matrix: Collect time series Y(1), Y(2), …, Y(T) into an i x j Hankel matrix H(0).

- Compute the Gram matrices:

- Compute H(0) H(0)T, a i×i symmetric matrix.

- Compute H(0)T H(0), a j×j symmetric matrix.

- Solve the eigen-problems:

- Find eigenvectors and eigenvalues of H(0) H(0)T: H(0) H(0)T uk = λk uk, k=1, …, i. Normalized uk form columns of U.

- Find eigenvectors of H(0)T H(0): H(0)T H(0) vk = λk vk, k=1, …, j. These vk form columns of V.

- Since H(0) H(0)T and H(0)T H(0) share non-zero eigenvalues λk, sort in descending order λ1 ≥ λ2 ≥ …, λk.

- Build singular-value matrix: Define singular values σk = √λk, place on diagonal of Σ in descending order. If H(0) is i x j, Σ is diagonal with zeros for padding.

- Verify H(0) = U Σ VT by construction: H(0) = ∑_{k=1}^{min(i,j)} σk uk (vk)T

- Truncate for model reduction (optional): See Appendix 1.1.2 ‘Why Truncate SVD’ – required if reduced order extraction is needed. Often keep only first r largest singular values/vectors: H(0) ≈ Ur Σr (Vr)T = ∑_{k=1}r (σk uk (vk)T.

- Interpret the factors:

- U: Basis for space spanned by rows of H(0) (left singular vectors).

- Σ: Gains indicating how “energetic” each mode is.

- V: Basis for space spanned by columns of H(0) (right singular vectors).

- Using subspace identification, split Ur = [U_{1,1} U_{2,1}], (Σr (Vr)T = [M1 M2], to recover estimates of system’s A,B,C,D matrices. But the core SVD step is the factorization H(0) = U Σ VT, computed via eigen-decompositions of the two Gram matrices, sorting by magnitude, and truncating (optional).

**B. Extraction**: Moving from truncated SVD factors of H(0) to reduced-order state-space realization (\hat{A}, \hat{B}, \hat{C}).

- Start with truncated SVD: Keep n largest singular values/vectors of H(0) ≈ Un Σn (Vn)T, where Un ∈ ℝi×n (first n left singular vectors), Σn ∈ ℝn×n (diagonal of top n singular values), Vn ∈ ℝj×n (first n right singular vectors). This produces a rank-n approximation capturing dominant dynamics.

- Create time-shifted Hankel matrix H(1): H(1) = \begin{bmatrix} Y(2) & Y(3) & \cdots & Y(j+1) \\ Y(3) & Y(4) & \cdots & Y(j+2) \\ \vdots & \vdots & \ddots & \vdots \\ Y(i+1) & Y(i+2) & \cdots & Y(i+j) \end{bmatrix}, using same block dimensions as H(0).

- Compute reduced-order state matrix \hat{A} = (Σn)-1/2 UnT H(1) Vn (Σn)-1/2.

- Reasoning: (Un)T H(1) Vn projects one-step-ahead data onto n principal subspaces.

**Python Example** (ERA Instantiation): Python - 	#!/usr/bin/env python3

"""

ERA Example:

- Build H(0), H(1) block-Hankel from measured Markov parameters Y(k)

- Compute SVD, truncate to order r

- Extract A_hat, B_hat, C_hat, D_hat

- Wrap in python-control StateSpace and validate via simulation

"""

import numpy as np

from scipy.linalg import svd

import control  # python-control

import matplotlib.pyplot as plt

import joblib  # for optional model persistence

# 1. Load Markov parameters (impulse/step responses)

# Assume CSV files Y1.csv, Y2.csv, … each p x T (p outputs x T samples)

# Stack so Y_all shape = (p*m, T)

m = 2 # Inputs/disturbance channels

p = 1 # Outputs

T = 200  # Samples per response

# Load example data (simulate if none)

Y_list = [np.random.rand(p, T) for _ in range(m)]  # Placeholder; replace with real data

Y_all = np.vstack(Y_list)  # (p*m, T)

# 2. Build block-Hankel matrices H0 = H(0) and H1 = H(1)

def block_hankel(Y, i, j):

"""Build block-Hankel matrix of shape (p*i, j) from Y[:, 0:(i+j-1)]."""

p, T_all = Y.shape[0], Y.shape[1]

H = np.zeros((p*i, j))

for row in range(i):

H[row*p:(row+1)*p, :] = Y[:, row:row+j]

return H

i = 10  # Block-rows

j = 40  # Block-cols (must satisfy i+j-1 <= T)

assert i + j - 1 <= T

H0 = block_hankel(Y_all, i, j)  # Past responses

H1 = block_hankel(Y_all[:,1:], i, j)  # One-step-ahead (shifted)

# 3. SVD + Truncate

U, s, Vh = svd(H0, full_matrices=False)

# Decide r by energy (e.g., 95% rule)

energy = np.cumsum(s**2) / np.sum(s**2)

r = np.searchsorted(energy, 0.95) + 1

print(f"Retaining {r} modes to capture 95% energy")

U_r = U[:, :r]  # (p*i, r)

S_r = np.diag(s[:r])  # (r, r)

V_r = Vh.conj().T[:, :r]  # (j, r)

# 4. Form reduced-order A_hat

S_inv_sqrt = np.diag(1.0 / np.sqrt(s[:r]))

A_hat = S_inv_sqrt @ (U_r.T @ H1 @ V_r) @ S_inv_sqrt

# 5. Recover C_hat and B_hat (and D = first Markov parameter)

S_sqrt = np.diag(np.sqrt(s[:r]))

C_hat = (U_r @ S_sqrt)[:p, :]  # (p x r)

B_hat = (S_sqrt @ V_r.T)[:, :m]  # (r x m) # adjust slicing if disturbances included

D_hat = Y_all[:,0].reshape(p, m)  # Direct feedthrough

# 6. Wrap in python-control StateSpace and Validate

sys_era = control.ss(A_hat, B_hat, C_hat, D_hat)  # Continuous-time; set Ts for discrete

# 6.1 One-step prediction vs measured (example validation)

time = np.arange(T)  # Assume unit sampling

U_val = Y_all[:m, :]  # Reuse as input for demo

_, y_pred = forced_response(sys_era, T=time, U=U_val)

y_pred = y_pred.T if y_pred.ndim == 1 else y_pred

residuals = Y_all[:p, :] - y_pred[:p, :]

# Plot (example for y1)

plt.figure(figsize=(8,4))

plt.plot(time, Y_all[0], 'k', label='Measured y1')

plt.plot(time, y_pred[0], 'r--', label='One-step pred')

plt.title("One-Step Prediction vs. Measured")

plt.legend()

plt.show()

# 6.2 Multi-step (free-run) simulation

_, y_sim = forced_response(sys_era, T=time, U=U_val)

y_sim = y_sim.T if y_sim.ndim == 1 else y_sim

plt.figure(figsize=(8,4))

plt.plot(time, Y_all[0], 'k', label='Measured y1')

plt.plot(time, y_sim[0], 'b:', label='Free-run sim')

plt.title("Multi-Step (Open-Loop) Simulation")

plt.legend()

plt.show()

# 7. (Optional) Save model for deployment

joblib.dump(sys_era, "era_model.pkl")

print("Saved ERA model to era_model.pkl")

**Comments**: Extend by adding noise-robustness via different energy levels, including disturbance channels in Y_all, deploying 'era_model.pkl' in MPC, or comparing ERA vs. N4SID/MOESP with control.n4sid/moesp.

**Variables**:

- H(0): Block-Hankel matrix at lag 0, in Rixj, constructed from output response data.

- Y(k): Output response at time k, in Rp (p outputs, e.g., from impulse response).

- i: Number of block rows in Hankel matrix, integer, related to data length.

- j: Number of block columns in Hankel matrix, integer.

- U: Left singular vectors from SVD, in Ri x i, orthogonal matrix.

- :Singular values, diagonal matrix in Ri x j, non-negative.

- V: Right singular vectors, in Rjx j, orthogonal matrix.

- \hat{A}: Reduced-order state transition matrix, in Rnxn, where n is chosen order.

- n: Truncated singular values, in Rnxn, top n singular values.

- Un: Truncated left singular vectors, in Ri x n.

- Vn: Truncated right singular vectors, in Rj x n .

- H(1): Hankel matrix at lag 1, in Ri x j, shifted version of H(0).

**HREF**: [https://en.wikipedia.org/wiki/Eigensystem_realization_algorithm](https://en.wikipedia.org/wiki/Eigensystem_realization_algorithm)

### 4. Non-linear ARX Neural Net (NARX): While PID excels in linear, single-loop scenarios but falters with nonlinearities, interactions, multiple disruptive variables, or variable delays which are all common in chemical reactors, distillation columns, or batch processes. NARX provides a means of improving on this; the hybrid NARX-MPC provides superior disturbance rejection and can be implemented via edge computing for low-latency control. Below is a step-by-step blueprint for implementing NARX in Python using a Distillation Column multi-section LWVC + LWC condensing system.

**When to Use**: For nonlinear dynamics with known delays; extends ARX for processes like distillation condenser loops.

**Base Equation**: \hat{y}(k) = f(y(k-1), …, y(k-ny), u(k-1), …, u(k-nu)),

where f is a feed-forward neural network; delays embed dynamics.

**Data Collection and Preparation**: Collect historical or real-time data from your facility's SCADA/PLC systems. TIP: Use IT/OT integration to stream data via MQTT to an onsite or cloud database for large-scale training, then deploy lightweight models on edge; monitor BTU trends (delta between cooling water return and supply temps) for fouling alerts.

**Python Example** (NumPy & scikit-learn): Python

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import MinMaxScaler

# Load data (e.g., from CSV exported from SCADA)

data = np.loadtxt('condenser_data.csv', delimiter=',')

# Columns: time, u_FCV, APV_cw_flow, APV_reflux_flow, APV_cw_return_T, y_vent_T, DV_vapor_P, DV_vapor_T, DV_cw_line_P, DV_cw_supply_T, DV_fac_cw_flow

u = data[:, 1:2]  # Inputs (m=1, cooling water FCV %)

y = data[:, 5:6]  # Outputs (p=1, condenser vent temp °F)

apv_dv = data[:, 2:5]  # APVs and DVs as features if needed

# Define lags (ny=3, nu=2 based on process knowledge/delays)

ny, nu = 3, 2

# Create lagged features

def create_lagged_data(u, y, nu, ny):

X = []  # Inputs: [y(k-1)...y(k-ny), u(k-1)...u(k-nu)]

Y = []  # Targets: y(k)

for k in range(max(nu, ny), len(y)):

lagged_y = y[k-ny:k][::-1].flatten()  # Past y's

lagged_u = u[k-nu:k][::-1].flatten()  # Past u's

X.append(np.concatenate([lagged_y, lagged_u]))

Y.append(y[k])

return np.array(X), np.array(Y)

X, Y = create_lagged_data(u, y, nu, ny)

# Scale data (normalize to [0,1] for neural stability)

scaler_X = MinMaxScaler()

scaler_Y = MinMaxScaler()

X_scaled = scaler_X.fit_transform(X)

Y_scaled = scaler_Y.fit_transform(Y)

# Split: 70% train, 15% Val, 15% test

X_train, X_temp, Y_train, Y_temp = train_test_split(X_scaled, Y_scaled, test_size=0.3, shuffle=False)

X_val, X_test, Y_val, Y_test = train_test_split(X_temp, Y_temp, test_size=0.5, shuffle=False)

**Define the NARX Model**: NARX handles nonlinear mappings (e.g., heat transfer in condensers affected by DV vapor pressure) via activation functions like ReLU, embedding delays for lag/deadtime in cw return T (cooling water rtn temp).

**Python Example** (PyTorch): Python

import torch

import torch.nn as nn

import torch.optim as optim

class NARXNet(nn.Module):

def __init__(self, input_dim, output_dim, hidden_dim=64):

super(NARXNet, self).__init__()

self.fc1 = nn.Linear(input_dim, hidden_dim)  # Input: ny*p + nu*m

self.fc2 = nn.Linear(hidden_dim, hidden_dim)

self.fc3 = nn.Linear(hidden_dim, output_dim)  # Output: p

self.relu = nn.ReLU()

def forward(self, x):

x = self.relu(self.fc1(x))

x = self.relu(self.fc2(x))

return self.fc3(x)

# Dimensions: input_dim = ny * p + nu * m (e.g., 3*1 + 2*1 = 5)

p, m = 1, 1

input_dim = ny * p + nu * m

model = NARXNet(input_dim, p)

# To GPU if available

device = torch.device("cuda" if torch.cuda.is_available() else "cpu")

model.to(device)

**Train the Model**: Use MSE loss for regression, Adam optimizer. Retrain periodically with OT data streams to adapt to DVs (e.g., cw supply T fluctuations), using IT cloud for hyperparameter tuning and BTU delta analysis for fouling prediction.

**Python Example** (PyTorch): Python

# Convert to tensors

X_train_tensor = torch.tensor(X_train, dtype=torch.float32).to(device)

Y_train_tensor = torch.tensor(Y_train, dtype=torch.float32).to(device)

# Similarly for val

criterion = nn.MSELoss()

optimizer = optim.Adam(model.parameters(), lr=0.001)

epochs = 200

for epoch in range(epochs):

model.train()

optimizer.zero_grad()

outputs = model(X_train_tensor)

loss = criterion(outputs, Y_train_tensor)

loss.backward()

optimizer.step()

# Validate

model.eval()

with torch.no_grad():

val_outputs = model(torch.tensor(X_val, type=torch.float32).to(device))

val_loss = criterion(val_outputs, torch.tensor(Y_val, dtype=torch.float32).to(device))

print(f"Epoch {epoch+1}, Train Loss: {loss.item():.4f}, Val Loss: {val_loss.item():.4f}")

# Save model

torch.save(model.state_dict(), 'narx_condenser.pth')

**Validate and Tune**: Simulate predictions; compare against PID baselines using metrics like MSE or FIT%. Use SciPy for residual analysis.

**Python Example** (SciPy & python-control): Python

from scipy.stats import norm  # For residual whiteness test

import numpy as np

import control

import matplotlib.pyplot as plt

# Predict on test

model.eval()

with torch.no_grad():

Y_pred_scaled = model(torch.tensor(X_test, dtype=torch.float32).to(device)).cpu().numpy()

Y_pred = scaler_Y.inverse_transform(Y_pred_scaled)

# MSE (assume Y_test unscaled as Y_test_unscaled)

mse = np.mean((Y_test_unscaled - Y_pred)**2)

# Residuals

residuals = Y_test_unscaled - Y_pred

# Whiteness test (e.g., autocorrelation for first PV)

from scipy.signal import correlate

autocorr = correlate(residuals[:,0], residuals[:,0], mode='full')

plt.plot(autocorr)

plt.title('Autocorrelation of Residuals')

plt.show()  # Peaks only at zero lag indicate whiteness

# Compare to PID (simulate baseline)

Kp, Ti, Td = 1.0, 1.0, 0.25  # Tune via Z-N

# s from sympy or assume

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate (placeholder; adapt to your data)

time = np.arange(len(Y_test_unscaled))

u_sim = np.ones(len(time))  # Example input

_, y_pid = control.forced_response(pid, T=time, U=u_sim)

# Compare errors, e.g., pid_mse = np.mean((Y_test_unscaled - y_pid)**2)

**Deploy in Industrial Facility**: Integrate into OT loop: Load model on edge device, predict \( \hat{y}(k) \) in real-time, use as feedforward in advanced control (e.g., adjust FCV based on predictions, factoring DVs like vapor pressure).

**Python Example** (Real-time Inference): Python

# Load model

model.load_state_dict(torch.load('narx_condenser.pth'))

model.eval()

# Real-time loop (e.g., in PLC gateway script)

while True:

# Get current lagged data from sensors (e.g., via OPC UA)

# Update buffers (vent T, FCV)

current_lagged = np.concatenate([past_y.flatten(), past_u.flatten()])

input_tensor = torch.tensor(scaler_X. Transform([current_lagged]), dtype=torch.float32).to(device)

pred = model(input_tensor).cpu().numpy()

# Predicted PPV (vent T)

pred_y = scaler_Y.inverse_transform(pred)[0]

# Use in control: e.g., adjust FCV if pred_y deviates from 149°F SP

# Send to PLC

time.sleep(Ts)  # Sampling time

**Monitor, Retrain, and Innovate**: Monitor via plc-gbt analytics (e.g., MSE thresholds trigger alerts). Retrain weekly with new data to handle DVs. Innovations for US Manufacturing: Hybrid NARX-SINDy for interpretable nonlinear terms (use pysindy to sparsify f); integrate with RL (e.g., SAC via stable-baselines3) for policy optimization over NARX predictions, incorporating APV purity and DV vapor temp. This enables autonomous, resilient control—propelling efficiency and dominance.

**Variables**:

- \hat{yk}: Predicted output (PV) at time k, in Rp.

- f: Nonlinear function (neural network), maps inputs to output, unitless.

- y(k-1), …, y(k-ny): Past outputs at times k-1 to k-ny, in Rp.

- ny: Number of lagged outputs, integer.

- u(k-1), …, u(k-nu): Past inputs (MVs) at times k-1 to k-nu, in Rm.

- nu: Number of lagged inputs, integer.

**HREF**: [https://en.wikipedia.org/wiki/Nonlinear_autoregressive_model](https://en.wikipedia.org/wiki/Nonlinear_autoregressive_model) & [https://www.mathworks.com/help/deeplearning/modeling-and-prediction-with-narx-and-time-delay-networks.html]( https://www.mathworks.com/help/deeplearning/modeling-and-prediction-with-narx-and-time-delay-networks.html)

### 5. Recurrent Neural Net (LSTM): LSTM empowers complex control operations with sequence-aware intelligence.

**When to Use**:

- For sequential data with long dependencies; innovative for predictive maintenance in IT/OT-integrated systems.

- LSTM shines in processes like your alcohol condenser, where historical patterns (e.g., persistent DV impacts from vapor temperature or cooling water line pressure) influence current states, enabling better handling of deadtimes (e.g., in cw return T) than feed-forward models like NARX.

- Use when PID fails on multivariable, time-varying disturbances, such as fouling trends or supply temp variations, to predict vent T deviations and preemptively adjust FCV for stable 149°F SP.

**Equations**:

- Forget gate: ft = σ(Wf [ht-1, xt] + bf)

- Input gate: it = σ(Wi [ht-1, xt] + bi)

- Cell update: \tilde{ct} = \tanh(Wc [ht-1, xt] + bc), ct = ft \odot ct-1 + it \odot \tilde{ct})

- Output gate: ot = σ(Wo [ht-1, xt] + bo), ht = ot \odot \tanh(ct),

where xt includes past y, u.

**Variables**:

- ft: Forget gate activation at time t, in h (h hidden units), range [0,1].

- σ: Sigmoid function, maps to [0,1], unitless.

- Wf: Weight matrix for forget gate, in Rh(h+d), where d is input dimension.

- ht-1: Previous hidden state, in Rh.

- xt: Input at time t, in Rd (includes past y, u).

- bf: Bias for forget gate, in Rh.

- it: Input gate activation, in Rh, range [0,1].

- Wi: Weight matrix for input gate, in Rh(h+d).

- bi: Bias for input gate, in Rh.

- \tilde{ct}: Candidate cell state, in Rh.

- \tanh: Hyperbolic tangent function, maps to [-1,1], unitless.

- Wc: Weight matrix for cell update, in Rh(h+d).

- bc: Bias for cell update, in Rh.

- ct: Cell state at time t, in Rh.

- ct-1: Previous cell state, in Rh.

- \odot: Hadamard (element-wise) product, unitless.

- ot: Output gate activation, in Rh, range [0,1].

- Wo: Weight matrix for output gate, in Rh(h+b).

- bo: Bias for output gate, in Rh.

- ht: Hidden state (output) at time t, in Rh.

**HREF**: [https://en.wikipedia.org/wiki/Long_short-term_memory](https://en.wikipedia.org/wiki/Long_short-term_memory)

**Data Collection and Preparation**: Collect historical or real-time data from your facility's SCADA/PLC systems.

**Python Example** (NumPy & scikit-learn): Python

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import MinMaxScaler

# Load data (e.g., from CSV exported from SCADA)

data = np.loadtxt('condenser_data.csv', delimiter=',')

# Columns: time, u_FCV, APV_cw_flow, APV_reflux_flow, APV_cw_return_T, y_vent_T, DV_vapor_P, DV_vapor_T, DV_cw_line_P, DV_cw_supply_T, DV_fac_cw_flow

u = data[:, 1:2]  # Inputs (m=1, cooling water FCV %)

y = data[:, 5:6]  # Outputs (p=1, condenser vent temp °F)

apv_dv = data[:, 2:5]  # APVs and DVs as features if needed

# Define sequence length (seq_len=10 for long dependencies)

seq_len = 10

# Create sequences

def create_sequences(u, y, seq_len):

X = []  # Sequences: [[x_{t-seq_len+1}, ..., x_t]] where x_t includes u(t), y(t-1), etc.

Y = []  # Targets: y(t+1) for prediction

for t in range(seq_len, len(y)):

# Shape: (seq_len, m + p)

seq_x = np.concatenate([u[t-seq_len:t], y[t-seq_len:t-1]], axis=1)

X.append(seq_x)

Y.append(y[t])

return np.array(X), np.array(Y)

X, Y = create_sequences(u, y, seq_len)

# Scale data

scaler_X = MinMaxScaler()

scaler_Y = MinMaxScaler()

X_scaled = scaler_X.fit_transform(X.reshape(-1, X.shape[-1])).reshape(X.shape)

Y_scaled = scaler_Y.fit_transform(Y)

# Split: 70% train, 15% val, 15% test

X_train, X_temp, Y_train, Y_temp = train_test_split(X_scaled, Y_scaled, test_size=0.3, shuffle=False)

X_val, X_test, Y_val, Y_test = train_test_split(X_temp, Y_temp, test_size=0.5, shuffle=False)

**Define the LSTM Model**:

**Python Example** (PyTorch): Python

import torch

import torch.nn as nn

import torch.optim as optim

class LSTMNet(nn.Module):

def __init__(self, input_dim, hidden_dim, output_dim, num_layers=1):

super(LSTMNet, self).__init__()

self.hidden_dim = hidden_dim

self.num_layers = num_layers

# Input: batch x seq_len x input_dim

self.lstm = nn.LSTM(input_dim, hidden_dim, num_layers, batch_first=True)

# Output from last hidden state

self.fc = nn.Linear(hidden_dim, output_dim)

def forward(self, x):

h0 = torch.zeros(self.num_layers, x.size(0), self.hidden_dim).to(x.device)

c0 = torch.zeros(self.num_layers, x.size(0), self.hidden_dim).to(x.device)

out, _ = self.lstm(x, (h0, c0))  # out: batch x seq_len x hidden_dim

out = self.fc(out[:, -1, :])  # Last time step

return out

# Dimensions: input_dim = m + p -1 (e.g., 1 + 1 = 2 for u_FCV and past y_vent_T)

input_dim = 2

hidden_dim = 64

output_dim = 1  # p=1 for vent T

model = LSTMNet(input_dim, hidden_dim, output_dim)

# To GPU if available

device = torch.device("cuda" if torch.cuda.is_available() else "cpu")

model.to(device)

**Train the Model**:

**Python Example** (PyTorch): Python

# Convert to tensors

X_train_tensor = torch.tensor(X_train, dtype=torch.float32).to(device)

Y_train_tensor = torch.tensor(Y_train, dtype=torch.float32).to(device)

# Similarly for val

criterion = nn.MSELoss()

optimizer = optim.Adam(model.parameters(), lr=0.001)

epochs = 200

for epoch in range(epochs):

model.train()

optimizer.zero_grad()

outputs = model(X_train_tensor)

loss = criterion(outputs, Y_train_tensor)

loss.backward()

optimizer.step()

# Validate

model.eval()

with torch.no_grad():

val_outputs = model(torch.tensor(X_val, dtype=torch.float32).to(device))

val_loss = criterion(val_outputs, torch.tensor(Y_val, dtype=torch.float32).to(device))

print(f"Epoch {epoch+1}, Train Loss: {loss.item():.4f}, Val Loss: {val_loss.item():.4f}")

# Save model

torch.save(model.state_dict(), 'lstm_condenser.pth')

**Validate and Tune**:

**Python Example** (SciPy & python-control): Python

from scipy.stats import norm  # For residual whiteness test

import numpy as np

import control

import matplotlib.pyplot as plt

# Predict on test

model.eval()

with torch.no_grad():

Y_pred_scaled = model(torch.tensor(X_test, dtype=torch.float32).to(device)).cpu().numpy()

Y_pred = scaler_Y.inverse_transform(Y_pred_scaled)

# MSE

# Assume unscaled Y_test as Y_test_unscaled

mse = np.mean((Y_test_unscaled - Y_pred)**2)

# Residuals

residuals = Y_test_unscaled - Y_pred

# Whiteness test (e.g., autocorrelation for first PV)

from scipy.signal import correlate

autocorr = correlate(residuals[:,0], residuals[:,0], mode='full')

plt.plot(autocorr)

plt.title('Autocorrelation of Residuals')

plt.show()  # Peaks only at zero lag indicate whiteness

# Compare to PID (simulate baseline)

Kp, Ti, Td = 1.0, 1.0, 0.25  # Tune via Z-N

# s from sympy or assume

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate (placeholder; adapt to your data)

time = np.arange(len(Y_test_unscaled))

u_sim = np.ones(len(time))  # Example input

_, y_pid = control.forced_response(pid, T=time, U=u_sim)

# Compare errors, e.g., pid_mse = np.mean((Y_test_unscaled - y_pid)**2)

**Deploy in Industrial Facility**: Integrate into OT loop: Load model on edge device, predict future vent T in real-time, use as feedforward in advanced control (e.g., adjust FCV based on sequences, factoring DVs like vapor pressure).

**Python Example** (Real-time Inference): Python

# Load model

model.load_state_dict(torch.load('lstm_condenser.pth'))

model.eval()

# Real-time loop (e.g., in PLC gateway script)

sequence buffer = []  # Maintain rolling sequence of last seq_len steps

while True:

# Get current data from sensors (e.g., via OPC UA): append to buffer

current_xt = np.array([current_u_FCV, prev_y_vent_T])  # xt includes u, past y

sequence_buffer.append(current_xt)

if len(sequence_buffer) > seq_len:

sequence_buffer.pop(0)

input_seq = np.array([sequence_buffer])  # Shape: 1 x seq_len x input_dim

input_tensor = torch.tensor(scaler_X.transform(input_seq.reshape(-1, input_dim)).reshape(1, seq_len, input_dim), dtype=torch.float32).to(device)

pred = model(input_tensor).cpu().numpy()

pred_y = scaler_Y.inverse_transform(pred)[0]  # Predicted PPV (vent T)

# Use in control: e.g., adjust FCV if pred_y deviates from 149°F SP

# Send to PLC

time.sleep(Ts)  # Sampling time

**Monitor, Retrain, and Innovate**:

Monitor via plc-gbt analytics (e.g., MSE thresholds trigger alerts).

Retrain weekly with new data to handle DVs.

Innovations for Manufacturing: Hybrid LSTM-SINDy for interpretable long-term terms (use pysindy on LSTM features); integrate with RL (e.g., SAC via stable-baselines3) for policy optimization over LSTM predictions, incorporating APV purity and DV vapor temp. This enables autonomous, resilient control—propelling efficiency and optimization

### 6. Gaussian Process Regression (GPR)

**When to Use**: For uncertainty quantification in small-data regimes; pairs well with MPC for robust manufacturing. GPR is an apt choice for use cases with sparse data and high uncertainty, as its non-parametric modeling handles nonlinearity and provides variance estimates—e.g., in tower supply water control for predicting pressure with confidence bounds under DVs like basin levels.

**Equation**: y(x) ~ GP(m(x), k(x, x'))

**Variables**:

- y(x): Output function, in R, evaluated at input x (e.g., process output).

- x: Input vector, in Rd (e.g., process conditions, controls).

- GP: Gaussian Process, a distribution over functions, unitless.

- Instead of assuming a fixed functional form (e.g. linear, polynomial), a GP defines a probability distribution over all possible functions that fit the data. Any finite collection of function values follows a multivariate Gaussian distribution. http://www.gaussianprocess.org/gpml/

- m(x): Mean function, in R, prior mean of output at x (often 0).

- k(x, x'): Covariance (kernel) function, in R, measures similarity between inputs x, x' (e.g., squared exponential kernel).

- x': Another input vector, in Rd, for covariance computation.

**HREF**: [https://en.wikipedia.org/wiki/Gaussian_process_regression](https://en.wikipedia.org/wiki/Gaussian_process_regression)

**Data Collection and Preparation**: Collect historical data; for tower: States (pressure, flow, temps, levels), actions (VFD speed).

**Python Example** (NumPy & scikit-learn): Python

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import StandardScaler

# Load data (e.g., from CSV)

# Columns: time, u_VFD, y_pressure, supply_flow, mash_cooler_flow1, ...

data = np.loadtxt('tower_data.csv', delimiter=',')

u = data[:, 1:2]  # Inputs (m=1, VFD speed %)

y = data[:, 2:3]  # Outputs (p=1, supply pressure psi)

features = data[:, 3:]  # Additional vars/DVs

# Combine inputs: X = [u, features]

X = np.hstack([u, features])  # (samples, input_dim)

# Scale data

scaler_X = StandardScaler()

scaler_Y = StandardScaler()

X_scaled = scaler_X.fit_transform(X)

Y_scaled = scaler_Y.fit_transform(y)

# Split: 70% train, 30% test

X_train, X_test, Y_train, Y_test = train_test_split(X_scaled, Y_scaled, test_size=0.3, random_state=42)

**Define the GPR Model**:

**Python Example** (scikit-learn): Python

from sklearn.gaussian_process import GaussianProcessRegressor

from sklearn.gaussian_process.kernels import RBF, ConstantKernel, WhiteKernel

# Define kernel: Constant * RBF + White (for noise)

kernel = ConstantKernel(1.0) * RBF(1.0, length_scale_bounds=(1e-2, 1e3)) + WhiteKernel(noise_level=1e-5)

model = GaussianProcessRegressor(kernel=kernel, n_restarts_optimizer=10, alpha=0.01)

**Train the Model**:

**Python Example** (scikit-learn): Python

# Fit model

model.fit(X_train, Y_train)

print(f"Optimized kernel: {model.kernel_}")

**Validate and Tune**:

**Python Example** (SciPy & python-control): Python

from scipy.stats import norm  # For confidence intervals

import numpy as np

import control

import matplotlib.pyplot as plt

# Predict on test: mean and std

Y_pred, sigma = model.predict(X_test, return_std=True)

Y_pred_unscaled = scaler_Y.inverse_transform(Y_pred.reshape(-1,1)).flatten()

sigma_unscaled = sigma * scaler_Y.scale_  # Approximate scaling

# MSE

mse = np.mean((Y_test_unscaled - Y_pred_unscaled)**2)

# Coverage: Check if true values within 2*sigma (95% CI)

coverage = np.mean(np.abs(Y_test_unscaled - Y_pred_unscaled) < 2 * sigma_unscaled)

# Residuals

residuals = Y_test_unscaled - Y_pred_unscaled

from scipy.signal import correlate

autocorr = correlate(residuals, residuals, mode='full')

plt.plot(autocorr)

plt.title('Autocorrelation of Residuals')

plt.show()

# Compare to PID (simulate baseline)

Kp, Ti, Td = 1.0, 1.0, 0.25  # Tune via Z-N

# s from sympy or assume

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate (placeholder; adapt to your data)

time = np.arange(len(Y_test_unscaled))

u_sim = np.ones(len(time))  # Example input

_, y_pid = control.forced_response(pid, T=time, U=u_sim)

# Compare errors, e.g., pid_mse = np.mean((Y_test_unscaled - y_pid)**2)

**Deploy in Industrial Facility**: Fit model on edge (GPR is lightweight), predict pressure with uncertainty, adjust VFD if low confidence (e.g., govern mash cooler flow).

**Python Example** (Real-time Inference): Python

# Assume model fitted; for real-time, refit periodically

# Real-time loop (e.g., in PLC gateway script)

while True:

# Get current features from sensors (e.g., via OPC UA)

current_X = np.array([current_u_VFD, current_supply_flow, ...])  # All inputs/DVs

input_scaled = scaler_X.transform([current_X])

pred_scaled, sigma_scaled = model.predict(input_scaled, return_std=True)

# Predicted pressure

pred_y = scaler_Y.inverse_transform(pred_scaled.reshape(1,-1))[0][0]

sigma = sigma_scaled[0] * scaler_Y.scale_[0]  # Uncertainty

# Use in control: e.g., adjust VFD if pred_y < SP or sigma > threshold

# Send to PLC

time.sleep(Ts)  # Sampling time

**Monitor, Retrain, and Innovate**:

Monitor via plc-gbt analytics (e.g., uncertainty thresholds trigger alerts).

Retrain daily with new data to handle DVs.

InnovaType equation here.tions for Manufacturing: Hybrid GPR-PILCO for uncertainty-aware RL (use GPy if expanded); integrate with SINDy for sparse dynamics discovery on GPR priors, incorporating process flows for demand forecasting. This enables autonomous, probabilistic control—propelling efficiency and dominance.

### 7. Sparse Identification of Nonlinear Dynamics (SINDy)

**When to Use**: For discovering sparse governing equations; innovative for physics-informed ML in automation. SINDy is ideal for dynamical systems like boiler steam pressure control, where data reveals a few key nonlinear terms (e.g., quadratic pressure dependencies or interactions with temperature), enabling interpretable models over black-box alternatives. Use when PID underperforms on complex feedbacks (e.g., saturation effects or load disturbances), to derive equations that inform MPC or fault detection—especially in sparse-data scenarios with physical intuition, like ensuring stable pressure amid varying fuel quality or demand spikes.

**Equation**: x = θ(x) ξ

**Variables**:

- x: Time derivative of state vector, in Rn, rate of change (e.g., velocity).

- x: State vector, in Rn (e.g., process states).

- θ(x): Library matrix of candidate functions, in Rnxm (m functions, e.g., polynomials x, x2.

- xi: Sparse coefficient vector, in Rm, selects active terms (mostly zeros).

**HREF**: [https://en.wikipedia.org/wiki/SINDy](https://en.wikipedia.org/wiki/SINDy)

**Data Collection and Preparation**: Collect historical data from your boiler loop: States (pressure, level, temp), MVs (valve %).

**Python Example** (NumPy & SciPy): Python

import numpy as np

from scipy.signal import savgol_filter  # For smoothing/derivatives

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import StandardScaler

# Load data (e.g., from CSV)

# Columns: time, u_valve, x_pressure, drum_level, temp, fuel_flow, ...

data = np.loadtxt('boiler_data.csv', delimiter=',')

t = data[:, 0]  # Time

x = data[:, 1:4]  # States (n=3, pressure, level, temp)

# Compute derivatives (dot_x) using Savitzky-Golay filter

dt = np.mean(np.diff(t))

dot_x = savgol_filter(x, window_length=5, polyorder=2, deriv=1, delta=dt, axis=0)

# Scale data

scaler_x = StandardScaler()

scaler_dot_x = StandardScaler()

x_scaled = scaler_x.fit_transform(x)

dot_x_scaled = scaler_dot_x.fit_transform(dot_x)

# Split: 80% train, 20% test

x_train, x_test, dot_x_train, dot_x_test = train_test_split(x_scaled, dot_x_scaled, test_size=0.2, shuffle=False)

**Define the SINDy Model**:

**Python Example** (pysindy): Python

from pysindy import SINDy

from pysindy.feature_library import PolynomialLibrary

# Define library: polynomials degree 3

library = PolynomialLibrary(degree=3)

# Instantiate model with thresholding optimizer

# Sparse threshold

model = SINDy(feature_library=library, optimizer='STLSQ', optimizer__threshold=0.1)

**Train the Model**:

*Python Example** (pysindy): Python

# Fit model

model.fit(x_train, t=dt, x_dot=dot_x_train)  # dt from data

# Print equations

model.print()  # Example output: x0' = a x0 + b x1^2 + ...

**Validate and Tune**:

**Python Example** (SciPy & python-control): Python

from scipy.integrate import odeint

import numpy as np

import matplotlib.pyplot as plt

# Simulate with SINDy equations

def sindy_eq(x, t):

return model.predict(x.reshape(1,-1)).flatten()  # Unscaled predict

x_sim = odeint(sindy_eq, x_test[0], t_test)  # Integrate (t_test from test time)

# MSE

mse = np.mean((x_test - x_sim)**2)

# Residuals (for pressure, index 0)

residuals = x_test[:,0] - x_sim[:,0]

from scipy.signal import correlate

autocorr = correlate(residuals, residuals, mode='full')

plt.plot(autocorr)

plt.title('Autocorrelation of Residuals')

plt.show()

# Compare to PID (simulate baseline)

import control

Kp, Ti, Td = 1.0, 1.0, 0.25  # Tune via Z-N

# s from sympy or assume

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate (placeholder; adapt to your data)

time = np.arange(len(x_test))

u_sim = np.ones(len(time))  # Example input

_, y_pid = control.forced_response(pid, T=time, U=u_sim)

# Compare errors, e.g., pid_mse = np.mean((x_test[:,0] - y_pid)**2)

**Deploy in Industrial Facility**: Load model on edge, predict \( \dot{x} \) for forward simulation, adjust valve if pressure trends unsafe.

**Python Example** (Real-time Inference): Python

# Assume model fitted; for real-time, refit periodically

# Real-time loop (e.g., in PLC gateway script)

while True:

# Get current states from sensors (e.g., via OPC UA)

current_x = np.array([current_pressure, current_level, current_temp])  # Scaled

dot_x_pred = model.predict(current_x.reshape(1,-1))[0]  # Predicted derivatives

# Simulate short horizon

future_t = np.linspace(0, 60, 10)  # 1-min ahead

future_x = odeint(lambda x,t: model.predict(x.reshape(1,-1)).flatten(), current_x, future_t)

# Use in control: e.g., adjust valve if future pressure > max

# Send to PLC

time.sleep(Ts)  # Sampling time

**Monitor, Retrain, and Innovate**:

Monitor via plc-gbt analytics (e.g., equation sparsity for model health).

Retrain weekly with new data to handle DVs.

Innovations for Manufacturing: Hybrid SINDy-Koopman for sparse observables (use kooplearn on SINDy terms); integrate with RL (e.g., PETS via ensembles) for policy optimization over SINDy equations, incorporating fuel flow for demand forecasting. This enables autonomous, interpretable control—propelling efficiency and optimization.

### 8. Koopman Operator Approaches

**When to Use**: For linearizing nonlinear systems; enables linear MPC on lifted states for complex dynamics.

**Equation**: φ(xk+1) ∼ K φ(xk)

**Variables**:

- φ(xk+1): Observable functions at time k+1, in Rm (m observables, e.g., nonlinear transformations of x).

- φ(xk): Observable functions at time k, in Rm.

- K: Koopman operator, in Rmxm, linear operator in observable space.

- xk: State vector at time k, in Rn.

**HREF**: [https://en.wikipedia.org/wiki/Koopman_operator](https://en.wikipedia.org/wiki/Koopman_operator)

**Data Collection and Preparation**: Collect historical data from your mash cooler loop: States (inlet/mid/outlet temps, mash flow), MVs (FCV 1/2 positions).

**Python Example** (NumPy & scikit-learn): Python

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import StandardScaler

# Load data (e.g., from CSV)

# Columns: time, x_inlet_T, x_mid_T, x_outlet_T, x_mash_flow, u_FCV1, u_FCV2, DV_cook_cycle, ...

data = np.loadtxt('mash_cooler_data.csv', delimiter=',')

t = data[:, 0]  # Time

x = data[:, 1:5]  # States (n=4, inlet T, mid T, outlet T, mash flow)

u = data[:, 5:7]  # Inputs (m=2, FCV1, FCV2)

# Augment states with u

xu = np.hstack([x, u])  # Extended states

# Scale data

scaler = StandardScaler()

xu_scaled = scaler.fit_transform(xu)

# Split: 70% train, 30% test

xu_train, xu_test = train_test_split(xu_scaled, test_size=0.3, shuffle=False)

**Define the Koopman Model**:

**Python Example** (pykoopman): Python

from pykoopman import Koopman

from pykoopman.observables import Polynomial

from pykoopman.regression import EDMD

# Define observables: polynomials degree 3

observables = Polynomial(degree=3)

# Define regressor: EDMD

regressor = EDMD()

# Instantiate model

model = Koopman(observables=observables, regressor=regressor)

**Train the Model**: Fit Koopman on consecutive state pairs; extract operator K.

**What's Happening**: We slice training data into pairs: current states xk and next states xk+1. This creates transitions capturing the system's evolution. The model lifts states to observables φ(xk) (e.g., polynomials for nonlinear terms like temp-flow interactions in MC01), then regresses K to approximate φ xk+1 ≈ K φ (xk). Under the hood, it solves a matrix equation via least-squares or SVD, minimizing error. Why? This embeds nonlinearity in lifting, allowing linear propagation in high dimensions—enabling efficient prediction for MC01's dual sections and SP weighting.

**Python Example** (pykoopman): Python

# Prepare data: x_k and x_{k+1}

x_train = xu_train[:-1]  # Current states

x_next_train = xu_train[1:]  # Next states

# Fit model

model.fit(x_train, x_next_train)

# Access K (linear operator)

K = model.A  # Discrete-time Koopman matrix

print(f"Koopman operator shape: {K.shape}")

**Validate and Tune**:

**Python Example** (SciPy & python-control): Python

from scipy.linalg import eig

import numpy as np

import matplotlib.pyplot as plt

# Predict multi-step

x_pred = np.zeros_like(x_test)

x_pred[0] = x_test[0]

for i in range(1, len(x_test)):

psi_current = model.observables.transform(x_pred[i-1].reshape(1,-1))

psi_next = psi_current @ model.A.T  # Linear propagation

x_pred[i] = model.observables.inverse(psi_next)  # Back to original space

# MSE

mse = np.mean((x_test - x_pred)**2)

# Stability: Eigenvalues of K

eigenvalues = eig(model.A)[0]

print(f"Max eigenvalue magnitude: {np.max(np.abs(eigenvalues))}")  # <1 for stability

# Compare to PID (simulate baseline)

import control

Kp, Ti, Td = 1.0, 1.0, 0.25  # Tune via Z-N

# s from sympy or assume

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate (placeholder; adapt to your data)

time = np.arange(len(x_test))

u_sim = np.ones(len(time))  # Example input

_, y_pid = control.forced_response(pid, T=time, U=u_sim)

# Compare errors, e.g., pid_mse = np.mean((x_test[:,0] - y_pid)**2)

plt.plot(time, x_test[:,0], label='True')

plt.plot(time, y_pid, label='PID Sim')

plt.legend()

plt.show()

**Deploy in Industrial Facility**: Integrate into OT loop: Load model on edge, propagate lifted states for prediction, adjust FCV via linear control on \( \psi \), factoring SP weighting.

**Python Example** (Real-time Inference): Python

# Assume model fitted; for real-time, refit periodically

# Real-time loop (e.g., in PLC gateway script)

current_x = np.array([current_inlet_T, current_mid_T, current_outlet_T, current_mash_flow, current_FCV1, current_FCV2])  # Scaled

while True:

psi_current = model.observables.transform(current_x.reshape(1,-1))

psi_future = psi_current @ model.A.T  # One-step ahead

future_x = model.observables.inverse(psi_future)[0]  # Predicted states

# Use in control: e.g., linear MPC on psi to adjust FCV for target outlet T, applying SP weighting

# Send to PLC

current_x = future_x  # Update for next (or from sensors)

time.sleep(Ts)  # Sampling time

**Monitor, Retrain, and Innovate**:

Monitor via plc-gbt analytics (e.g., eigenvalue stability for model health).

Retrain weekly with new data to handle DVs.

Innovations for Manufacturing: Hybrid Koopman-SINDy for sparse observables (use pysindy on lifted space); integrate with RL (e.g., DDPG via stable-baselines3) for policy optimization on linear K, incorporating cook cycle DVs for demand forecasting. This enables autonomous, linearized control—propelling efficiency and dominance.

### 9. Model-Based Reinforcement Learning (PILCO, PETS)

**When to Use**: For optimal control under uncertainty; bridges RL with process models for adaptive manufacturing. PILCO and PETS are ideal for dynamic, stochastic loops like tower supply water control, where limited data and high variability (e.g., sudden process calls from condensers or fermenters) demand sample-efficient learning with uncertainty quantification. Use PILCO for Gaussian Process-based modeling in small-data regimes (e.g., probabilistic predictions of pressure under basin level fluctuations); opt for PETS when neural ensembles better capture complex interactions (e.g., trajectory sampling for planning VFD speeds amid temp risks). Apply when PID fails on long-term optimization (e.g., balancing flow across consumers while avoiding over-temp shutdowns), especially in safety-critical setups needing expected return gradients or uncertainty propagation for robust governing.

**Why Use**: PILCO and PETS shine in distillery operations by addressing PID's shortcomings in uncertain, multivariable systems—providing model-based RL that learns dynamics and policies with few trials, incorporating uncertainty to avoid catastrophic failures (e.g., insufficient cooling leading to lost batches). Sample efficiency: Unlike model-free RL (e.g., SAC), PILCO uses GPR for analytic gradients; PETS leverages ensembles for uncertainty-aware planning. Uncertainty handling: Quantify risks (variances or sampled trajectories) for safe exploration (e.g., conservative VFD ramps). Interpretability and innovation: Allow physics-informed priors, fostering IT/OT hybrids like edge execution for flow allocation. Over SINDy/Koopman, add RL for goal-oriented optimization; vs. GPR alone, close the loop with policy learning—crucial for dominance through autonomous, risk-aware systems.

**How to Use**: We'll implement PILCO and PETS for tower supply water: Learn dynamics to optimize VFD actions for pressure SP, rewarding flow satisfaction while penalizing temp exceeds, using gymnasium for env.

**Data Collection and Preparation**:

**Python Example** (gymnasium & NumPy): Python

import numpy as np

import gymnasium as gym

from gymnasium import spaces

class TowerEnv(gym.Env):

def __init__(self):

super().__init__()

# Normalized VFD speed

self.action_space = spaces.Box(low=0, high=1, shape=(1,))

self.observation_space = spaces.Box(low=np.array([0, 0, 0, 0]), high=np.array([75, 3200, 212, 212]), shape=(4,))  # pressure, flow, supply T, return T

self.state = np.array([50, 1600, 70, 85])  # Initial state

def step(self, action):

vfd = action[0] * 100  # Denormalize

self.state[0] += 0.5 * vfd - 0.1 * (self.state[3] - self.state[2])  # Pressure update

self.state[1] += 10 * vfd  # Flow update

# Track 60 psi, penalize temp >80F

reward = -abs(self.state[0] - 60) - 10 * max(0, self.state[2] - 80)

done = False

return self.state, reward, done, {}

def reset(self):

self.state = np.array([50, 1600, 70, 85]) + np.random.uniform(-10, 10, 4)

return self.state, {}

env = TowerEnv()

# Collect initial data (random policy rollout)

states, actions, next_states, rewards = [], [], [], []

state = env.reset()

for _ in range(1000):

action = env.action_space.sample()

next_state, reward, _, _ = env.step(action)

states.append(state)

actions.append(action)

next_states.append(next_state)

rewards.append(reward)

state = next_state

states = np.array(states)

actions = np.array(actions)

next_states = np.array(next_states)

rewards = np.array(rewards)

**Implement PILCO (GPR-Based)**:

**Python Example** (GPy & SciPy): Python

import GPy

from scipy.optimize import minimize

# Fit GPR dynamics (one for each state dim)

models = []

for i in range(states.shape[1]):

X = np.hstack([states, actions])

Y = next_states[:, i:i+1] - states[:, i:i+1]  # Delta states

kernel = GPy.kern.RBF(input_dim=X.shape[1], variance=1., lengthscale=1.)

gp = GPy.models.GPRegression(X, Y, kernel)

gp.optimize(messages=False)

models.append(gp)

# Policy (linear; optimize via gradients)

def policy(params, state):

return np.clip(np.dot(state, params[:4]) + params[4], 0, 1)  # Example

def expected_return(params, horizon=10):

state = env.reset()

total_r = 0

for _ in range(horizon):

action = policy(params, state)

mean_next, var_next = [], []

for i, gp in enumerate(models):

inp = np.hstack([state, action]).reshape(1, -1)

m, v = gp.predict(inp)

mean_next.append(state[i] + m[0][0])

var_next.append(v[0][0])

next_state = np.array(mean_next)  # Use mean (or sample for variance)

reward = -abs(next_state[0] - 60) - 10 * max(0, next_state[2] - 80)

total_r += reward

state = next_state

return -total_r  # Minimize negative return

initial_params = np.random.randn(5)

result = minimize(expected_return, initial_params, method='L-BFGS-B')

optimal_params = result.x

**Implement PETS (Ensemble-Based)**: Use the revised snippet from our previous exchange (with PyTorch, as clarified).

**Validate and Tune**:

**Python Example** (gymnasium): Python

def evaluate(policy_func, episodes=10):

total_rewards = []

for _ in range(episodes):

state = env.reset()

ep_reward = 0

for _ in range(100):

action = policy_func(state)  # e.g., policy(optimal_params, state) for PILCO

state, reward, _, _ = env.step([action])

ep_reward += reward

total_rewards.append(ep_reward)

return np.mean(total_rewards)

pilco_return = evaluate(lambda s: policy(optimal_params, s))

pets_return = evaluate(lambda s: cem_planning(s))  # From PETS snippet

print(f"PILCO avg return: {pilco_return}, PETS: {pets_return}")

**Deploy in Industrial Facility**: Load models on edge, compute actions via PILCO gradients or PETS sampling, adjust VFD if uncertainty high.

**Python Example** (Real-time Loop): Python

while True:

current_state = get_sensor_data()  # e.g., via OPC UA

action = cem_planning(current_state)  # Or PILCO policy

env.step([action])  # Send to PLC

time.sleep(Ts)

**Monitor, Retrain, and Innovate**:

Monitor returns/uncertainty via plc-gbt (variance thresholds trigger alerts).

Retrain episodically.

Innovations: Fuse PETS with SINDy for sparse priors; integrate PILCO with Koopman for linearized uncertainty—enabling autonomous governing (e.g., throttling mash coolers if temp risk high).

**Variables**:

- f: Dynamics model (GPR for PILCO, neural ensemble for PETS), maps state-action to next state, unitless.

- xt: State at time t, in Rn.

- ut: Action (MV) at time t, in Rm.

- rt: Reward at time t, in R, performance metric (e.g., tracking error).

- For PILCO: See GPR variables (m(x), k(x, x')).

- For PETS: Ensemble predictions, uses p(xt+1 | xt, ut).

**HREF (PILCO)**: [https://mlg.eng.cam.ac.uk/pilco/](https://mlg.eng.cam.ac.uk/pilco/)

**HREF(PETS)**: [https://arxiv.org/abs/1805.12114](https://arxiv.org/abs/1805.12114)

### 10. Policy-Gradient RL (DDPG, TD3, SAC)

**When to Use**: For continuous-action spaces in uncertain environments; model-free for direct policy learning.

Example, we will use still column distillate proofing control: A highly uncertain process where alcohol concentration (proof) must be maintained (e.g., 100 -160 proof SP) with a significant and variable lag and tau.  Low Wine Average proof setpoint = 130 and High Wine Average proof setpoint = 140. Amid many potential disturbance variables causing varying disruption, like varying beerfeed composition (total mash density, %ABV, recipe differences, etc.), still temperature drifts due to varying steam supply pressure, column pressure fluctuations, column beer bottom level, and others which cause risk of producing off-spec product or energy waste.

Using DDPG/TD3/SAC the model can learn deterministic or stochastic policies to adjust multiple MVs /CVs (e.g., reflux ratios or steam inputs) for optimal proof tracking or weighting to other PID or MPC MV adjustments.  Incorporating an exploration release of these models can allow learning and safe adaptation.

The deadtime and lag between Column vapor temperature and vapor condensing back to liquid then flowing thru the proMass proofing meter is very long and dependent on its own set of disturbance variables such as reflux rate, cooling water flow and temperature, vapor concentration, ect. This algorithm can unlock novel and innovative IT/OT hybrid solutions taking advantage of edge agents adjacent to PLCs for real-time policy execution amid long lags, fused with custom analytics platforms like plc-gbt for handling composition DVs, driving precise, resilient, and optimized US spirit production through emerging RL in IIoT ecosystems.

**Equations** (DDPG/TD3): Actor μ(s | θ^μ), critic \( Q(s, a | θ ^Q).

**SAC Equation**: E[tr_t + αH(π(⋅ | st))]

**Variables**:

- μ(s | θ^μ): Deterministic policy (actor), maps state s to action a, in Rm.

- s: State, in Rn (e.g., process states and observations).

- θ^μ: Parameters of actor network, in Rp.

- Q(s, a | θ ^Q): Action-value function (critic), in R, estimates expected return.

- a: Action (MV), in Rm.

- θ ^Q: Parameters of critic network, in Rq.

- rt: Reward at time t, in R .

- α: Entropy regularization coefficient, in R, balances exploration.

- H(π(⋅ | st)): Entropy of policy π, in R, measures policy randomness.

- π(⋅ | st): Stochastic policy, distribution over actions given state st.

- E: Expectation operator, unitless.

**HREF (DDPG)**: [https://en.wikipedia.org/wiki/Deep_Deterministic_Policy_Gradient](https://en.wikipedia.org/wiki/Deep_Deterministic_Policy_Gradient)

**HREF (TD3)**: [https://arxiv.org/abs/1802.09477](https://arxiv.org/abs/1802.09477)

**HREF (SAC)**: [https://arxiv.org/abs/1812.05905](https://arxiv.org/abs/1812.05905)

**Data Collection and Environment Setup**: Define a custom env simulating proofing with lags.

**Python Example** (gymnasium & NumPy): Python

import gymnasium as gym

from gymnasium import spaces

import numpy as np

from collections import deque

class ProofingEnv(gym.Env):

def __init__(self, lag_steps=20):  # Simulate long lag/deadtime

super().__init__()

self.action_space = spaces.Box(low=0, high=1, shape=(2,))  # Normalized reflux, steam

# Proof, temp °F, pressure psi, %ABV

self.observation_space = spaces.Box(low=np.array([0, 0, 0, 0]), high=np.array([200, 212, 100, 20]), shape=(4,))

self.state = np.array([130, 180, 50, 10])  # Initial

self.sp_proof = 130  # Low Wine SP (or 140 for High)

self.lag_buffer = deque(maxlen=lag_steps)

self.lag_steps = lag_steps

def step(self, action):

self.lag_buffer.append(action.copy())

if len(self.lag_buffer) < self.lag_steps:

delayed_action = np.zeros(2)

else:

delayed_action = self.lag_buffer[0]

reflux, steam = delayed_action

# Proof update with noise

self.state[0] += 0.5 * reflux + 0.2 * steam - 0.1 * (self.state[1] - 180) + 0.05 * np.random.normal()

# Temp update with %ABV DV

self.state[1] += 0.3 * steam + 0.1 * (self.state[3] - 10)

# Track SP, penalize energy

reward = - (self.state[0] - self.sp_proof)**2 - 0.1 * steam

done = abs(self.state[0] - self.sp_proof) < 5

truncated = False

return self.state, reward, done, truncated, {}

def reset(self, seed=None):

self.state = np.array([130, 180, 50, 10]) + np.random.uniform(-20, 20, 4)

self.lag_buffer.clear()

return self.state, {}

env = ProofingEnv(lag_steps=20)  # 20-step lag for deadtime

**Implement DDPG**:

**Python Example** (stable-baselines3): Python

from stable_baselines3 import DDPG

from stable_baselines3.common.noise import NormalActionNoise

n_actions = env.action_space.shape[-1]

action_noise = NormalActionNoise(mean=np.zeros(n_actions), sigma=0.1 * np.ones(n_actions))

model_ddpg = DDPG("MlpPolicy", env, action_noise=action_noise, verbose=1)

model_ddpg.learn(total_timesteps=10000, log_interval=10)

model_ddpg.save("ddpg_proofing")

**Implement TD3**:

**Python Example** (stable-baselines3):  Python

from stable_baselines3 import TD3

model_td3 = TD3("MlpPolicy", env, verbose=1)

model_td3.learn(total_timesteps=10000, log_interval=10)

model_td3.save("td3_proofing")

**Implement SAC**:

**Python Example** (stable-baselines3): Python

from stable_baselines3 import SAC

model_sac = SAC("MlpPolicy", env, verbose=1)

model_sac.learn(total_timesteps=10000, log_interval=10)

model_sac.save("sac_proofing")

**Validate and Tune**:

**Python Example** (stable-baselines3): Python

def evaluate(model, episodes=10):

rewards = []

for _ in range(episodes):

obs, _ = env.reset()

ep_reward = 0

done = False

while not done:

action, _ = model.predict(obs, deterministic=True)

obs, reward, done, _, _ = env.step(action)

ep_reward += reward

rewards.append(ep_reward)

return np.mean(rewards)

ddpg_return = evaluate(model_ddpg)

td3_return = evaluate(model_td3)

sac_return = evaluate(model_sac)

print(f"DDPG avg return: {ddpg_return}, TD3: {td3_return}, SAC: {sac_return}")

**Deploy for testing**: Load models on edge, query policies for actions; monitor for overestimation.

**Python Example** (Real-time Loop): Python

model = SAC.load("sac_proofing")  # Or DDPG/TD3

while True:

obs = get_sensor_data()  # e.g., via OPC UA: proof, temp, pressure, %ABV

action, _ = model.predict(obs, deterministic=True)

env.step(action)  # Send to PLC (reflux, steam)

time.sleep(Ts)

**Monitor, Retrain, and Innovate**:

Monitor returns/variance via plc-gbt (e.g., TD3 critics for bias alerts).

Retrain episodically.

Innovate: Fuse SAC entropy with SINDy for sparse priors; integrate DDPG with Koopman for linearized actors—enabling policy optimization amid deadtimes (e.g., adjusting reflux if proof uncertainty from composition DVs threatens yield).

### 11. Classical Proportional-Integral-Derivative (PID) Tuning Rules

**When to Use**: For single-loop control; baseline before MPC in legacy systems.

- i) Ziegler–Nichols (Z-N): Ultimate gain Ku, period Tu; Kp = 0.6 Ku, Ti = 0.5 Tu, Td = 0.125 Tu.

HREF: [https://en.wikipedia.org/wiki/Ziegler–Nichols_method](https://en.wikipedia.org/wiki/Ziegler–Nichols_method)

- ii) Cohen–Coon: From step response lag τ, time constant T.

HREF: [https://control.com/textbook/pid-control/cohen-coon-tuning-rules/](https://control.com/textbook/pid-control/cohen-coon-tuning-rules/)

- iii) λ-Tuning (IMC-based): Choose closed-loop λ; (Kp = T / (K (λ + τ)), etc.

HREF:[https://en.wikipedia.org/wiki/Internal_model_control#Lambda_tuning](https://en.wikipedia.org/wiki/Internal_model_control#Lambda_tuning)

**Variables**:

- Kp: Proportional gain, unitless or scaled to process units.

- Ku: Ultimate gain, unitless, gain causing sustained oscillations.

- Ti: Integral time, in seconds, for reset action.

- Tu: Ultimate period, in seconds, oscillation period at Ku.

- Td: Derivative time, in seconds, for rate action.

- T: Process time constant, in seconds, from step response.

- K: Process gain, unitless or in output/input units.

- λ (lambda): Closed-loop time constant, in seconds, user-specified.

- τ (tau): Process lag, in seconds, from step response.

### 12. Internal Model Control (IMC): The IMC framework is a model-based control strategy that leverages a process model to design a controller, offering robustness against model mismatches and disturbances, particularly in processes with significant delays

**When to Use**: For processes with delays; robust against model mismatch.

**Equation**:

Decompose GpType equation here.(s) ≈ G_(s) G+(s) (minimum-phase and all-pass).

Controller: Gc(s) = G-1_(s) ∙ 1 / 1 + λs.

**Variables**:

- Decompose: The process of splitting Gp(s) into minimum-phase (G_(s)) and all-pass (G+(s)) components to simplify controller design.

- Gp(s): Process transfer function, in R (freq domain), maps inputs to outputs. Forms Laplace domain (s) that maps inputs (Manipulated / Control Variables, MVs, like valve positions) to outputs (Process Variables, PVs, like temperature or pressure). It captures the dynamic behavior of the physical system.

- s: Laplace variable, in s-1.

- G_(s): Minimum-phase part of process model, in R.  Example, in a first-order plus dead-time (FOPDT) model common in process control: Gp(s) = Ke−τs / τps + 1. The minimum-phase part is: G_ (s)= K / τps + 1 because the denominator τps + 1 (pole at s = −1/τp) and gain K are stable and invertible. Python lib – control handles this with Python:

import control

K, tau_p = 1.0, 2.0

G_minus = control.tf(K, [tau_p, 1]) # K / (tau_p s + 1)

G_minus_inv = control.tf([tau_p, 1], [K]) # (tau_p s + 1) / K

- G+(s): All-pass part (e.g., delays), in R. Includes non-invertible elements, such as: Time delays (e.g., e−τs e^{-\tau s} e−τs), Right-half-plane zeros, which lead to unstable inverses. For the FOPDT example: G+(s) =e−τs, This has magnitude ∣e−τs∣ = 1 (all-pass property, affecting only phase, not gain). ID of delay or non-minimum-phase element using the python control lib. Python:

tau = 1.0

G_plus = control.tf([1], [1], deadtime=tau) # e^(-tau s)

- Controller:

- G_c(s): Controller transfer function, in R.

- G_-1(s): Inverse of minimum-phase model, in R.

- λ: Filter time constant, in seconds.

**HREF**: [https://en.wikipedia.org/wiki/Internal_model_control](https://en.wikipedia.org/wiki/Internal_model_control)

### **Implement IMC Model**: The IMC framework designs a controller by leveraging a model of the process, making it robust to model mismatches and effective for systems with delays, which is critical for distillery processes (e.g., controlling condenser vent temperature or still side proofing). The equations are listed above.​

Review of subscripts and decomposition usage:

Role of G_(s) (Minimum-Phase Component): includes all stable dynamics of Gp(s) that can be inverted. This typically means:

Poles in the left-half s-plane (stable, with negative real parts).

Zeros in the left-half s-plane (if any), which are invertible.

For example, in a first-order plus dead-time (FOPDT) model common in process control: Gp(s) = Ke−τs / τps + 1,​ The minimum-phase part is: G_(s) = K / τps + 1 because the denominator τps + 1 (pole at s = −1 / τp = -1 and gain K are stable and invertible.

Why It Matters: The controller inverts G_(s) (i.e., computes G_−1(s) to cancel the process’s invertible dynamics, aiming for perfect setpoint tracking in an ideal case (if the model matches the process exactly).

Manipulation: Compute the inverse: For G_(s) = K / τps + 1​, G_−1(s) = τps + 1 / K This is stable and proper (numerator degree = denominator degree), making it suitable for control.

In practice, use tools like Python’s control library to handle this - Python:

import control

K, tau_p = 1.0, 2.0

G_minus = control.tf(K, [tau_p, 1])# K / (tau_p s + 1)

G_minus_inv = control.tf([tau_p, 1], [K])# (tau_p s + 1)/K

Role of G+(s) (All-Pass Component): includes non-invertible elements, such as: Time delays, which are non-causal if inverted (requiring future inputs). This typically means:

Right-half-plane zeros, which lead to unstable inverses.

For example (in FOPDT): G+(s) = e−τs. This has magnitude ∣e−τs ∣ =1 (all-pass property, affecting only phase, not gain).

Why It Matters: Inverting G+(s) (e.g., eτs) is impractical because it requires future data or introduces instability. Thus, IMC leaves G+(s) out of the controller design, accepting the delay or phase shift as a limitation of the physical system.

Manipulation: Identify the delay or non-minimum-phase elements during model identification (e.g., via step tests or subspace methods like N4SID).

Do not invert G+(s); it remains in the closed-loop response, causing the output to track the setpoint with a delay: y(s) / r(s) ≈ G+(s) ⋅ 1 / 1 + λs = e−τs / 1 + λs.

In Python, model the delay as follows: Python

tau = 1.0

G_plus = control.tf([1], [1], deadtime=tau)  # e^(-tau s)

Decomposition Process: the act of factoring Gp(s) into G_(s) and G+(s) components to separate invertible and non-invertible dynamics. This is critical for designing a stable controller.

How to: Start with the process model Gp(s), obtained from system identification (e.g., step response, ARX, or N4SID, as detailed above in section X).

Analyze the poles and zeros:

Poles/zeros in the left-half plane go to G_(s).

Time delays and right-half-plane zeros go to G+(s).

FOPDT example: Gp(s) = Ke−τs / τps + 1 = (K / τps + 1) ⋅ (e−τs) = G_(s)G+(s).

For complex systems, use numerical tools to factorize. Python:

# G_p(s) = G_-(s) * G_+(s)

G_p = control.series(G_minus, G_plus)

Purpose: This separation ensures the controller Gc(s) = G_−1(s) ⋅ 1 / 1+λs only inverts the stable, causal dynamics, avoiding instability or non-causality.

Controller Gc(s): The controller equation is listed above with a purpose of canceling the minimum-phase dynamics.

1 / 1+λs acts as a low-pass filter with time constant λ (seconds), ensuring:

Stability by attenuating high-frequency noise.

Robustness to model mismatches,

Proper transfer function (finite high-frequency gain).

Manipulation:

Compute G_−1(s), as shown above.

Choose λ based on process dynamics (e.g., λ ≈ τp for balance, or larger for robustness in noisy environments like a distillation column with fluctuating pressures.

Form the controller: Gc(s) = τps + 1 / K ⋅ 1 / 1+λs = τps + 1 / K(1 + λs). Discretize for real-time implementation: Python

lambda_val = 2.0

G_c = control.series(control.tf([tau_p, 1], [K]), control.tf([1], [lambda_val, 1]))

Ts = 0.1

G_c_d = control.c2d(G_c, Ts, method='tustin')

num, den = G_c_d.num[0][0], G_c_d.den[0][0]

Implement in a control loop: Python

u_prev, e_prev = 0, 0

while True:

y = get_sensor_data() #e.g., vent temp via OPC UA

r = 149  # SP in °F

y_tilde = simulate_model(u_prev)#Model prediction

e = r - (y - y_tilde)  # IMC error

u = (num[0]*e + num[1]*e_prev - den[1]*u_prev) / den[0]

send_to_plc(u)  # Adjust flow

u_prev, e_prev = u, e

time.sleep(Ts)

### 13. Relay Feedback:

**When to Use**: For auto-tuning PID without open-loop tests. Relay induces limit cycles; infer Ku, Tu for Z-N tuning.

**Variables**:

- K: Ultimate gain, unitless, from relay-induced limit cycle.

- Tu: Ultimate period, in seconds, from limit cycle.

**HREF**: [https://en.wikipedia.org/wiki/Relay_autotuning](https://en.wikipedia.org/wiki/Relay_autotuning)

### 14. Genetic Algorithm (GA):

**When to Use**: For optimizing non-differentiable objectives, like tuning hybrid controllers. Evolve population Θi via selection, crossover, mutation to minimize J(\theta).

**Variables**:

- JΘ: Objective function, in R, performance metric (e.g., control error).

- Θ: Parameter vector, in Rp, candidate solution (e.g., controller gains).

- Θi: Population of parameter vectors, set of vectors in Rp.

**HREF**: [https://en.wikipedia.org/wiki/Genetic_algorithm](https://en.wikipedia.org/wiki/Genetic_algorithm)

### 15. Custom Algorithm Creator:

**When to Use**: For bespoke IT/OT hybrids, e.g., subspace model + neural residuals for drift adaptation.

Build in Python via scikit-learn: [https://scikit-learn.org/stable/developers/develop.html#rolling-your-own-estimator](https://scikit-learn.org/stable/developers/develop.html#rolling-your-own-estimator).

Innovation: Augment linear grey-box with SINDy-discovered terms for physics-ML fusion or integrate with plc-gbt for custom analytics (e.g., correlating upstream disturbances like mash density to downstream proofing).

## Pros/Cons of Key Algorithms

Algorithm Types:

Linear ID: Models 1-3

PROS: Fast, interpretable, low data requirements

CONS: Limited to processes with linear response

Nonlinear / ML: Models 4-7

PROS: Handles complexity well and some uncertainty

CONS: data-intensive, less interpretable

RL / Advanced: Models 8-10

PROS: Highly Adaptive to change, Dynamic

CONS: CPU/GPU intensive, potential safety risks without fail-over included

**GOAL**: Create a Python-based recipe from process data  subspace model  MPC with explicit feedforward. Replace PID loop(s) with predictive, multivariable controller handling 6 DV disturbances (individually & combined) for resilient dynamic process control. This example emphasizes production readiness: robust data handling, state estimation via Kalman with Riccati-solved gains, hybrid innovations, and plc-gbt integration for monitoring/upstream prediction and dynamic disturbance mitigation.

**Process Definition**: Identify variables.

- State: x ∈ Rn (n chosen via subspace or physics).

- MV: u(k)

- DVs: d(k) = [d1, …, d6]T

- PVs: y = [y1, y2, y3]T, SP: r for y1.

**Data Collection**: Excite u and each di with PRBS or steps (independently/combined). Log y1, y2 at sampling Ts. Compute y3 = fdrv(y1, y2). For production readiness, validate data: Check for NaNs (not a number) / outliers (z-score >3), sensor freezes (variance < threshold), and interpolate gaps (SciPy interp1d). Use plc-gbt to log/alert on quality issues, enabling upstream correlation (e.g., link beer density and  %abv DV’s to downstream proof risk).

**Code example (data validation and preparation)**: Python

import numpy as np

import pandas as pd

from scipy.interpolate import interp1d

from sklearn.model_selection import train_test_split

# Load from plc-gbt CSV/stream

df = pd.read_csv('process_data.csv')  # Columns: time, u, d1..d6, y1, y2

df['time'] = pd.to_datetime(df['time'])

# Validate: Handle missing/outliers

df = df.interpolate(method='linear')  # Fill NaNs

z_scores = (df - df.mean()) / df.std()

df = df[(z_scores < 3).all(axis=1)]  # Remove outliers

# Check freezes (low variance)

var_threshold = 1e-5

if df.var().min() < var_threshold:

print("Alert: Possible sensor freeze via plc-gbt")

# Excite and log (placeholder; integrate with plc-gbt for real-time)

u = df['u'].values

d = df[['d1','d2','d3','d4','d5','d6']].values.T

y1, y2 = df['y1'].values, df['y2'].values

y3 = fdrv(y1, y2)  # User-defined function

y = np.vstack([y1, y2, y3]).T

Ts = np.mean(np.diff(df['time'])).total_seconds()

**Model ID**: Use N4SID to estimate - xk+1 = Axk + Buk + Edk, yk = Cxk,

where A ∈ Rn x n, B ∈ Rn x 1, E ∈ Rn x 6, C ∈ Rn x 3.

For robustness, bootstrap: Resample data 10 times, average matrices. Use plc-gbt to track model drift (e.g., if eigenvalue norms change >10%, re-identify).

**code example (control & NumPy for Bootstrapping)**: Python

import control

import numpy as np

# Wrap data

data = control.iddata(y=y, u=np.hstack([u.reshape(-1,1), d.T]), tsamp=Ts)

# Bootstrap for robustness

num_boots = 10

models = []

for _ in range(num_boots):

idx = np.random.choice(len(y), len(y), replace=True)

boot_data = control.iddata(y=y[idx], u=np.hstack([u[idx].reshape(-1,1), d.T[idx]]), tsamp=Ts)

sys_boot = control.n4sid(boot_data, n=4)  # Example order

models.append(sys_boot)

# Average matrices

A = np.mean([m.A for m in models], axis=0)

B = np.mean([m.B for m in models], axis=0)

E = np.mean([m.E for m in models] if hasattr(m, 'E') else np.zeros((A.shape[0],6)), axis=0)  # If disturbances in B

C = np.mean([m.C for m in models], axis=0)

sys = control.ss(A, np.hstack([B, E]), C, 0, tsamp=Ts)

**Model Validation**:

One-step ahead: Check y(k | k-1) vs. y(k).

Multi-step: Simulate open-loop for horizon N.

Run Residual analysis to ensure whiteness.

Metrics: FIT = 100 (1 - |y - y|2 |y - y|2).

Add cross-validation: K-fold on data, average FIT.

For production: set auto-retrain if FIT < 90%, alert via plc-gbt.

**code example (control & SciPy)**: Python

from control import forced_response

import numpy as np

from sklearn.model_selection import KFold

time = np.arange(len(y)) * Ts

_, y_sim, _ = forced_response(sys, T=time, U=np.hstack([u, d.T]).T)

# FIT metric

fit = 100 * (1 - np.linalg.norm(y - y_sim.T, axis=0) / np.linalg.norm(y - np.mean(y, axis=0), axis=0))

print("FIT scores:", fit)

# Cross-validation (3-fold example)

kf = KFold(n_splits=3)

for train_idx, test_idx in kf.split(y):

boot_data = control.iddata(y=y[train_idx], u=np.hstack([u[train_idx].reshape(-1,1), d.T[train_idx]]), tsamp=Ts)

sys_boot = control.n4sid(boot_data, n=4)

# Compute FIT on test_idx, average

# Residual whiteness (autocorrelation)

residuals = y - y_sim.T

from scipy.signal import correlate

autocorr = correlate(residuals[:,0], residuals[:,0], mode='full')

if np.max(np.abs(autocorr[1:])) > threshold:  # e.g., 0.1

print("Alert: Non-white residuals via plc-gbt")

**State Estimation (Detailed with Riccati Equation)**: For unmeasured states in subspace/MPC, use a Kalman filter for robust estimation under noise/DVs:

xk+1 = Axk + Buk + K(yk − Cxk), where K is computed from the steady-state solution of the Discrete Algebraic Riccati Equation (DARE): P = APAT + Q − APCT (CPCT + R)−1 CPAT.

**Why Riccati?**: Minimizes estimation covariance P, balancing model uncertainty (Q for DVs like vapor pressure) and measurement noise (R for sensor errors in temps/flows). Solve DARE once for steady K, then use in filter—efficient for production.

**Steps for Production Readiness**:

Tune Q/R: Empirically from data (e.g., Q = diag(0.01) for high DV uncertainty, R = 0.05 for noisy PVs).

Solve DARE: Use scipy for numerical stability.

Initialize filter: x0 = 0, P0 = I (identity).

Add checks: yk−Cxk > 3√R, flag fault via plc-gbt (e.g., sensor drift).

Adaptive tuning: Re-solve Riccati if process changes (e.g., monitor residuals variance, update Q/R online), automate with plc-gbt workflow.

code example (scipy for DARE, control for Kalman): Python

from scipy.linalg import solve_discrete_are

import numpy as np

import control

# From identification

A, B, C = sys.A, sys.B, sys.C #Example matrices

Q = np.eye(A.shape[0]) * 0.01 #Process noise (tune for DVs)

R = np.eye(C.shape[0]) * 0.05  # Measurement noise

# Solve DARE for P

# Transposed for dual form

P = solve_discrete_are(A.T, C.T, Q, R)

# Compute K

K = P @ C.T @ np.linalg.inv(C @ P @ C.T + R)

print("Kalman Gain K:", K)

# Kalman filter loop (integrate in MPC)

x_hat = np.zeros(A.shape[0])

while True:

y = get_measurements()  # From sensors

u = get_inputs()  # From controller

y_hat = C @ x_hat

innovation = y - y_hat

x_hat = A @ x_hat + B @ u + K @ innovation  # Update

# Use x_hat in MPC initial state

time.sleep(Ts)

**MPC Formulation**: At each k, solve

mini=0N-1uk+I  i=1N∥y1, k + i∣k - r∥_Q^2 + i=0N-1∥Δuk+i∥_R^2.

s.t. (subject to): xk+i+1∣k = Axk+i∣k + Buk+i + Edk+i =Cxk+i∣k​.

Constraints: u_min ≤ u ≤ u_max, Δu_min ≤ Δu ≤ Δu_max.

For robustness, add slack variables and use Kalman-estimated xk as initial state.

For production, warm-start with previous u, add solver timeouts with PID fallback.

code example (cvxpy for QP, with Slack): Python

import cvxpy as cp

import numpy as np

# Matrices from identification/Kalman

x0 = x_hat  # From Kalman

N = 10  # Horizon

Q = np.eye(3)  # Output weights

R = np.eye(1) * 0.01  # Input weights

u_vars = [cp.Variable(1) for _ in range(N)]

slack = [cp.Variable(3, nonneg=True) for _ in range(N)]  # Slack for soft constraints

obj = 0

constraints = []

x = x0

for i in range(N):

y = C @ x

# Penalize slack

obj += cp.quad_form(y[0] - r, Q[0,0]) + cp.quad_form(u_vars[i] - u_prev, R) + 100 * cp.sum(slack[i])

# Soft dynamics

constraints += [x == A @ x + B @ u_vars[i] + E @ d_pred[i] + slack[i]]

constraints += [u_min <= u_vars[i] <= u_max]  # Hard constraints

x = A @ x + B @ u_vars[i] + E @ d_pred[i]  # Propagate

prob = cp.Problem(cp.Minimize(obj), constraints)

prob.solve(warm_start=True)  # For speed

u_opt = u_vars[0].value  # First move

if prob.status != 'optimal':

fallback_to_pid()  # Safety

**Feedforward Compensation**: As provided, with added validation: After computing uff, check against simulation (e.g., predict y with/without ff, alert via plc-gbt if discrepancy > threshold).

**Offline Testing**: As provided, with Monte Carlo: Simulate 100 runs with randomized DVs, compute mean/variance of ISE. Test on hardware emulator for latency.

**Deployment**: As provided, with watchdog: Add timeout (e.g., if QP > Ts, fallback to PID). Use plc-gbt for CI/CD: Automate retraining on new data, deploy updates via containers.

###**Further / Future consideration**: A hybrid Approaches for Upstream Predictive Control.   To achieve highly dynamic process control that predicts and corrects disturbances from <=2 upstream processes, hybrid algorithms are key. For example, combine SINDy (for sparse dynamics discovery) with RL (e.g., PETS for uncertainty-aware planning) and LSTM (for sequence prediction).

In a distillery, this could mean monitoring beer chemistry before beer enters still (upstream) to preemptively adjust column reflux in proofing (downstream), mitigating beerfeed composition DVs.

Why Hybrid?: PID reacts locally; hybrids predict globally, using advanced frameworks and heavier compute (e.g., plc-gbt correlating upstream mash density to downstream proof risks) and OT edge for fast execution.   Implementation Tip: Use custom creator (Section 15) to fuse models. Libraries for hybrid approach: from pysindy import SINDy, from stable_baselines3 import SAC.

Discover sparse dynamics with SINDy:

sindy_model = SINDy() sindy_model.fit(upstream_data, t=dt, x_dot=dot_x)

Use as prior in RL env for PETS/SAC: Python

class HybridEnv(gym.Env):

def step(self, action):

# Predict with SINDy, add uncertainty from DV simulation

next_state = sindy_model.predict(state) + action_effect

# Reward based on downstream impact

reward = -abs(next_state - downstream_sp)

return next_state, reward, done, {}

#Train SAC on hybrid env

model = SAC("MlpPolicy", HybridEnv(), verbose=1)

model.learn(total_timesteps=10000)

## Appendix: Deployment Considerations for Agentic SME LLMs.  Train your agentic SME LLM on this guide by feeding sections as prompts, e.g., "Implement subspace ID for mash cooler data and integrate with plc-gbt for upstream prediction." Use RLHF with simulated distillery envs to fine-tune for disturbance mitigation. For deployment:

- Edge: Run lightweight models (e.g., quantized PyTorch) on gateways for low-latency.

- Cloud: Use plc-gbt for data aggregation, retraining, and analytics (e.g., correlating DVs across processes).

- Innovation: Add LLM as "meta-controller" to select algorithms dynamically (e.g., switch from PID to SAC if uncertainty high).

<next rev>

OVERVIEW: Algorithms for Process Control

This mathematically focused overview outlines models and algorithms relevant to process control in manufacturing, emphasizing IT/OT convergence. We cover system identification for building dynamic models from data, then transition to control strategies like Model Predictive Control (MPC). References provide deeper dives. The document culminates in a detailed, end-to-end Python-based example for creating an MPC algorithm: from data collection to building, training, testing, and deploying a multivariable controller that outperforms traditional PID loops by handling disturbances predictively.

Key innovations: Hybrid approaches (e.g., augmenting linear models with neural residuals) enable real-time adaptation on edge devices, bridging OT sensors with IT cloud analytics for resilient US manufacturing.

Glossary of Acronyms:

ARMAX: AutoRegressive Moving Average with eXogenous inputs

ARX: AutoRegressive with eXogenous inputs

CV: Controlled Variable (used as Manipulated Variable in some contexts, synonymous with MV)

DDPG: Deep Deterministic Policy Gradient

DV: Disturbance Variable

ERA: Eigensystem Realization Algorithm

FIT: Fit Percentage (a validation metric)

GA: Genetic Algorithm

GPR: Gaussian Process Regression

IMC: Internal Model Control

ISE: Integral of Squared Error

IT: Information Technology

LASSO: Least Absolute Shrinkage and Selection Operator

LSTM: Long Short-Term Memory

MOESP: Multivariable Output-Error State sPace

MPC: Model Predictive Control

MV: Manipulated Variable

N4SID: Numerical algorithms for Subspace State Space System IDentification

NARX: Non-linear AutoRegressive with eXogenous inputs

OT: Operational Technology

PETS: Probabilistic Ensembles with Trajectory Sampling

PID: Proportional-Integral-Derivative

PILCO: Probabilistic Inference for Learning COntrol

PLC: Programmable Logic Controller

PRBS: Pseudo-Random Binary Sequence

PV: Process Variable

QP: Quadratic Program

RL: Reinforcement Learning

RLS: Recursive Least Squares

SAC: Soft Actor-Critic

SINDy: Sparse Identification of Nonlinear Dynamics

SP: Set Point

SVD: Singular Value Decomposition

TD3: Twin Delayed Deep Deterministic Policy Gradient

Z-N: Ziegler–Nichols

Algorithms for Process Control: Each entry includes a brief "When to Use" note for practical application in manufacturing.

AutoRegressive with eXogenous inputs (ARX / ARMAX):

When to Use:

For simple linear processes with known inputs/outputs: ideal for quick baseline models in stable manufacturing lines: If your process behaves approximately linearly over the range of interest (e.g. small deviations of temperature, flow, or pressure), an ARX/ARMAX model can capture that behavior with just a handful of parameters. Ideal for single‐loop or modestly multivariable systems where nonlinearities are mild and can be treated as disturbances

For fast, On-Line Identification Needs: ARX models can be estimated via recursive least squares in real time, allowing you to track slow drifts or step changes in process gain, delay, or noise characteristics. Useful when the plant dynamics change over time—e.g., fouling, catalyst deactivation, or equipment aging—and you want to auto-tune your controller.

Low Computational Burden: The parameter estimation and prediction computations boil down to linear algebra (matrix multiplications and inversions of small orders), making ARX/M Models eminently suitable for implementation directly in PLCs or low-power embedded hardware.

Explicit Noise Modeling with ARMAX: When measurement noise or unmeasured disturbances corrupt your outputs, augmenting ARX to ARMAX (adding a moving-average noise model) lets you separate true dynamics from stochastic fluctuations, improving prediction accuracy in MPC or Kalman‐filter observers.

A foundation for Simple Predictive Control: Even a basic receding-horizon controller built on an ARX model often outperforms PID under disturbance, since it can anticipate future outputs based on both current and past inputs.

Model Form (ARX):

y(k) + ∑{i=1a} aiy(k−i) = ∑{j=1nb} bju(k – j + 1) + e(k)

ARMAX adds a moving-average noise term:

y(k) = B(q−1) / A(q−1) u(k−1) + C(q−1) / A(q−1) e(k)

where A, B, C are polynomials in the backward-shift operator q−1 q^{-1} q−1.

Variables:

y(k): Output (Process Variable, PV) at discrete time step k, typically a scalar or vector in ℝ (e.g., temperature, pressure). Represents the measured system response.

k: Discrete time index, integer (unitless).

na​: Order of autoregressive terms, integer, number of past outputs used (e.g., na = 2 means two lagged outputs).

ai​: ARX coefficients for past outputs, scalar in ℝ, weights for y(k−i), for i=1, …,na​.

y(k−i): Past output at time k−i, same units as y(k).

nb​: Order of exogenous input terms, integer, number of past inputs used.

bj​: Input coefficients, scalar in ℝ, weights for u(k−j+1), for j=1, …,nb

u(k−j+1): Input (Manipulated Variable, MV) at time k – j + 1, scalar or vector in ℝ (e.g., valve position, flow rate).

e(k): Noise or error term at time k, scalar or vector in ℝ, assumed white noise (zero mean, finite variance).

B(q−1): Polynomial in backward-shift operator q−1, defined as B(q−1) = b1q−1 + b2q−2 + ⋯ + bnq−n​, unitless coefficients.

A(q−1): Polynomial in backward-shift operator, A(q−1) = 1 + a1q−1 + ⋯ + anq−n, unitless coefficients.

C(q−1): Polynomial for moving-average noise, C(q−1) = 1 + c1q−1 + ⋯ + cnq−n, unitless coefficients.

q−1: Backward-shift operator, shifts signal back by one time step (e.g., q−1u(k) = u(k−1)).

u(k−1): Input at previous time step, same as u(k – j + 1) for j=1.

Ref: https://en.wikipedia.org/wiki/Autoregressive_exogenous_model\

Subspace Identification (N4SID, MOESP): Subspace ID methods like N4SID and MOESP let you take a large block of real plant data (inputs, outputs, disturbances), automatically distill it to the handful of true “state variables” governing your process, and extract a clean linear state‐space model. This model plugs directly into observers and predictive controllers, giving you an MPC that inherently “looks ahead” across all disturbance channels—far outperforming a bank of decoupled PIDs when the plant has many interacting variables.

When to Use:

Multivariable (MIMO) Systems: Ideal when you have multiple inputs, outputs, and disturbance channels interacting—common in industrial plants (e.g., distillation columns, heat‐exchanger networks). Don’t need to hand-choose polynomials or worry about which lag-terms to include; subspace methods discover the state dimension and system order from data.

Black-Box Modeling Without Structural Priors: When you lack a first-principles model or when the physics are too complex, subspace ID automatically builds a minimal state-space realization (A,B,C,D). Works “out of the box” without you defining basis functions or tuning model orders manually (beyond selecting the projection horizon).

Robustness to Noise and Disturbances: By projecting onto dominant singular-value subspaces, these methods inherently filter measurement noise. You can incorporate measured disturbances as extra inputs and get disturbance-to-state mappings directly in B.

Computational Efficiency for Large Data Sets: Heavy lifting reduces to QR‐factorizations and SVDs of Hankel matrices—operations that are well-optimized in numerical libraries. Scales gracefully to long I/O histories and many channels, unlike iterative nonlinear optimizers.

Good Foundation for Predictive Control & Observers: Delivers a state-space model ready for Kalman filters and linear-MPC solvers—no extra conversion step needed. Facilitates fast updates (e.g., via fast SVD or rank-updates) if you need to re-identify online.

Implementing subspaceID algorithm: (N4SID & MOESP)

Prerequisites and Installation:

Python Packages: > pip install control numpy pandas matplotlib joblib osqp

control: for subspace ID models n4sid & moesp

numpy: numeric arrays & linear algebra

pandas: I/O dataset storage (csv, hdf5, etc…)

matplotlib: plotting for validation

joblib: model persistence (save / load objects)

osqp:  example QP solver for deployment

Collecting and Storing I/O datasets: when collecting from a live plant, accumulate timestamps + channel names into a pandas.DataFrame and periodically flush to HDF5 or CSV.

Collect real plant data: read inputs/disturbances & outputs via OPC-UA, pycomm3, or CSV logs. **If data has irregular sampling, you'd preprocess with interpolation (e.g., via SciPy's interp1d), but here we assume uniform dt for simplicity**.

Example python code:

import numpy as np

import pandas as pd

from control import rss, forced_response

# --- simulate a random stable MIMO system for demonstration ---

# rss: random state-space with n states, ℓ outputs, m inputs

n, ℓ, m = 4, 2, 2

sys = rss(n, ℓ, m)

Ts = 0.1

T_total = 1000

time = np.arange(0, T_total*Ts, Ts)

# PRBS inputs

U = (np.random.rand(m, len(time)) > 0.5).astype(float)

_, Y, _ = forced_response(sys, T=time, U=U)

# --- package into a DataFrame and save ---

cols = [f'u{i+1}' for i in range(m)] + [f'y{j+1}' for j in range(ℓ)]

df = pd.DataFrame(np.vstack((U, Y)).T, columns=cols)

df.to_csv('io_data.csv', index=False)

Loading and wrapping data for Subspace ID algorithms:

Example python code:

import numpy as np

import pandas as pd

import control

# 2.1 Load raw CSV

df = pd.read_csv('io_data.csv')

m, ℓ = 2, 2       # match however many channels you have

T = len(df)

# 2.2 Extract U, Y arrays (shape: channels × time)

U = df[[f'u{i+1}' for i in range(m)]].values.T    # (m, T)

Y = df[[f'y{j+1}' for j in range(ℓ)]].values.T    # (ℓ, T)

# 2.3 Wrap in python-control’s iddata (time × channels)

Ts = 0.1

data = control.iddata(y=Y.T, u=U.T, Ts=Ts)

Training - Running N4SID and MOESP:

Choose a Model Order: When you run a subspace ID routine like n4sid or moesp, you must tell it how many “states” — i.e. the model order — to extract. Choosing that order nnn is crucial: too small and you’ll miss important dynamics; too large and you’ll overfit noise, bloat your controller, and slow down your MPC.

Python model order example:

# quick grid-search on one-step MSE

def one_step_mse(sys_ss, U, Y, Ts):

time = np.arange(Y.shape[1]) * Ts

_, Y_pred = control.forced_response(sys_ss, T=time, U=U.T)

Y_pred = Y_pred.T

return np.mean((Y - Y_pred)**2)

best_order = None

best_mse = np.inf

for n in range(1, 11):

sys_n4 = control.n4sid(data, n)

mse = one_step_mse(sys_n4, U, Y, Ts)

if mse < best_mse:

best_mse, best_order = mse, n

print(f"Selected model order: {best_order} (MSE={best_mse:.3e})")

model_order = best_order

Identify with N4SID example:

Python example:

sys_n4sid = control.n4sid(data, model_order)

A_n4, B_n4, C_n4, D_n4 = (

sys_n4sid.A, sys_n4sid.B,

sys_n4sid.C, sys_n4sid.D

)

Or Identify with MOSEP:

Python example:

sys_moesp = control.moesp(data, model_order)

A_mo, B_mo, C_mo, D_mo = (

sys_moesp.A, sys_moesp.B,

sys_moesp.C, sys_moesp.D

)

Testing and Validation: This is where you make sure that your identified subspace‐ID model captures the true process dynamics and will generalize to new data—rather than merely overfitting the training set. This will ensure a successful MPC deployment.

One-step-ahead vs. Full-Horizon Simulation: In MPC design, you use one-step error to help choose the model order (it’s very sensitive to missing dynamics) but rely on multi-step simulation to confirm the model stays accurate over the full prediction horizon your controller will use.

Python Example Code:

import numpy as np

import matplotlib.pyplot as plt

# time vector for plotting

time = np.arange(T) * Ts  # T samples at Ts seconds each

# 1) One-step-ahead prediction

#    – uses true past outputs to forecast y[k] from data up to k–1

_, Y_pred = control.forced_response(sys_n4sid, T=time, U=U.T)

Y_pred = Y_pred.T  # back to shape (ℓ, T)

# 2) Full-horizon “open-loop” simulation

#    – rolls the model forward using only inputs, without correction

_, Y_sim = control.forced_response(sys_n4sid, T=time, U=U.T)

Y_sim = Y_sim.T

# 3) Plot the primary output (y₁)

plt.figure(figsize=(8,3))

plt.plot(time,     Y[0],     'k',   label='Measured y₁')

plt.plot(time,     Y_pred[0], 'r--', label='1-step-ahead')

plt.plot(time,     Y_sim[0],  'b:',  label='Full-horizon sim')

plt.xlabel('Time [s]')

plt.ylabel('y₁')

plt.legend()

plt.tight_layout()

plt.show()

Key checks:

Residual whiteness, compute and plot as follows:

e = Y - Y_pred

Multi-step tracking: Ensure the simulated trajectory (Y_sim) follows the main trends of Y over the entire look-ahead window. Large drifts or mismatched peaks indicate the model’s errors would accumulate unacceptably inside MPC.

Persisting the Identified model: This lets you save and re-use the exact (A,B,C,D) you’ve just calibrated—without re-running the (potentially time-consuming) subspace-ID routine every time you restart your script or your controller. For reproducibility & versioning, fast startup & deployment, decoupling training & inference, audit & traceability, and flexibility for updates.

Saving state space python example:

import joblib

# Save entire python-control StateSpace object

joblib.dump(sys_n4sid, 'n4sid_model.pkl')

# Or just save matrices

np.savez('model_matrices.npz',

A=A_n4, B=B_n4, C=C_n4, D=D_n4)

On load-in sample code:

sys_loaded = joblib.load('n4sid_model.pkl')

# or

data = np.load('model_matrices.npz')

A, B, C, D = data['A'], data['B'], data['C'], data['D']

Deploying in an MPC Loop: how you turn your identified (A,B,C) model into the core matrices of a finite-horizon quadratic program, then solve that QP in real time to generate your control moves.

Build Predication matrices sample code:

import numpy as np

n, m, p = A_n4.shape[0], B_n4.shape[1], C_n4.shape[0]

lookahead_time = 10.0

N = int(lookahead_time / Ts)

# PHI (pN × n)

PHI = np.vstack([C_n4 @ np.linalg.matrix_power(A_n4, i+1)

for i in range(N)])

# GAMMA (pN × mN)

GAMMA = np.zeros((p*N, m*N))

for i in range(N):

for j in range(i+1):

GAMMA[i*p:(i+1)*p, j*m:(j+1)*m] = (

C_n4 @ np.linalg.matrix_power(A_n4, i-j) @ B_n4

)

Form QP and Solve with QSQP:

Sample code:

import osqp

from scipy import sparse

# cost weights

Q = np.eye(p) * 1.0

R = np.eye(m) * 0.01

Q_bar = sparse.block_diag([Q]*N)

R_bar = sparse.block_diag([R]*N)

H = 2*(GAMMA.T @ Q_bar @ GAMMA + R_bar)

# for regulation to zero setpoint, f = 2 GAMMA^T Q_bar PHI x0

# no constraints example

prob = osqp.OSQP()

prob.setup(P=sparse.csc_matrix(H), q=np.zeros(m*N),

A=None, l=None, u=None, verbose=False)

# in your real-time loop:

x_k = np.zeros(n)

for k in range(T - N):

# state update (or use a Kalman filter)

u_k = U[:, k]

x_k = A_n4 @ x_k + B_n4 @ u_k

# build f = 2 GAMMA^T Q_bar PHI x_k

f = 2 * (GAMMA.T @ Q_bar @ (PHI @ x_k))

res = prob.solve(q=f)

delta_u = res.x[:m]  # apply only first input move

# send u_cmd = u_k + delta_u to your PLC / actuator

# in practice you’ll add input/output constraints via A, l, u in OSQP

Ongoing Maintenance:

Online re-identification: periodically rerun n4sid on a sliding window of the most recent data.

State estimation: wrap (A,B,C,D)(A,B,C,D)(A,B,C,D) in a Kalman filter (control.kalman) for noisy outputs.

Order selection: monitor singular-value decay and prediction MSE to adapt model_order.

Key subspace equations: xk+1 = Axk + Buk, yk = Cxk + Duk

Variables:

xk+1​: State vector at time k+1, in ℝn, where n is system order (e.g., internal states like fluid levels).

xk​: State vector at time k, in ℝn.

A: State transition matrix, in ℝn×n, describes state dynamics.

uk​: Input (MV) at time k, in ℝm (m inputs, e.g., control signals).

B: Input matrix, in ℝn×m, maps inputs to state updates.

yk​: Output (PV) at time k, in ℝp (p outputs, e.g., sensor measurements).

C: Output matrix, in ℝp×n, maps states to outputs.

D: Feedthrough matrix, in ℝp×m, direct input-to-output effect (often zero in process control).

N4SID extracts A, B, C, D by projecting Hankel matrices of past inputs/outputs onto dominant subspaces via SVD.

MOESP minimizes output-prediction error via orthogonal projections.

Ref: https://en.wikipedia.org/wiki/Subspace_identification

Eigensystem Realization Algorithm (ERA):

When to Use:

For reduced-order models from impulse/step response data; useful in vibration-heavy manufacturing (e.g., aerospace).

Predicting process disturbance before it occurs.

Think of ERA + the SVD projection step as one streamlined recipe for “learning the handful of key motion‐patterns in your process—including all your disturbance channels—and then discovering exactly how those patterns march forward in time,” so your MPC can truly “look ahead” rather than just react.

Collect and Build: From response data, build block-Hankel matrix - a matrix of past responses (measuring the Markov parameters):

H(0) = [Y(1)Y(2)⋯Y(j)Y(2)Y(3)⋯Y(j+1)⋮⋮⋱⋮Y(i)Y(i+1)⋯Y(i+j−1)]

Perform Singular-Value Decomposition (SVD) on H(0) means factoring it as H(0) = UΣVT – to reveal the dominant patterns,

Where U ∈ Ri×i is orthonormal (UTU = I), I = Identity matrix for Ri×i, Σ ∈ Ri×j is diagonal (possibly rectangular) with nonnegative entries σ1≥σ2≥⋯, and V ∈ Rj×j is orthonormal (VTV = I), I = Identity matrix for Rj×j.

Form Hankel matrix: collect time series Y(1), Y(2), …, Y(n) into an i x j Hankel matrix H(0).

Compute the Gram matrices:

Compute H(0)(H(0))T, a i×i symmetric matrix.

Compute (H(0))T(H(0)), a j×j symmetric matrix.

Solve the eigen-problems:

Find the eigenvectors and eigenvalues of H(0)(H(0))T

H(0)(H(0))T uk = λk​uk​, k=1, …, i.

The normalized eigenvectors uk​ form the columns of U.

Find the eigenvectors of (H(0))TH(0):

(H(0))T(H(0)) vk = λk​vk​, k=1, …, j.

These eigenvectors vk form the columns of V

Since H(0)(H(0))T and (H(0))T(H(0)) share the same non-zero eigenvalues λk, they can be sorted in descending order λ1 >= λ2 >= …, λk.

Build singular-value matrix:

Define the singular values as: σk ​= /sqrt{λk}​​, and place them on the diagonal of Σ in descending order.

If H(0) is i x j, then Σ = [σ10σ2⋱ ⋯ 0σmin(i,j)] with zeros elsewhere (this may form tall or wide matrices.

Verify H(0) = UΣVT by construction,

That UΣVT = ∑{k=1,min(i,j)​} (σk​uk​vk)T​} reconstructs H(0) when all singular values/vectors are included.

Truncate for model reduction (optional): see appendix 1.1.2 ‘Why Truncate SVD’ – required if reduced order extraction is needed.

Often for system identification you keep only the first r largest singular values/vectors:

H(0) ≈ (Ur​Σr​Vr)T ​= ∑{k=1,r} ​(σk​uk​vk)T​.

This gives you a reduced‐order approximation of rank r, capturing the dominant dynamics while discarding small “noise” modes.

Interpret the factors:

U: basis for the space spanned by the rows of H(0) (left singular vectors)

Σ: gains that tell you how “energetic” each mode is.

V: basis for the space spanned by the columns of H(0) (right singular vectors).

Using subspace identification, split:

Ur = [U1,1U2,1], (Σr​Vr)T = [M1 M2], to recover estimates of system’s A,B,C,D matrices.  But the core SVD step is exactly the factorization

H(0) = U Σ VT, computed via the eigen-decompositions of the (2) gram matrices, sorting by magnitude, and truncating (optional).

Extraction: Moving from the truncated SVD factors of H(0) to a reduced order state space realization ( \hat{A}, \hat{B}, \hat{C}).

Why use it?: This is where the ‘Magic’ happens – looking ahead H(1), H(2), …, H(tn) tn time slices into the future.  With these compact models of order-n that embed all potential disturbance variables, and your control input it can be feed into a standard linear MPC to predict future state, and act on it.  ERA‐based MPC can mitigate the large errors that a blind PID controller in complex processes must tolerate when many disturbance variables threaten your loop.

Start with the truncated SVD

Keeping the n largest singular values / vectors of H(0) ≈ (UnΣnVn)T, where

Un ​∈ Ri×n contains the first n left singular vectors,

Σn ​∈ Rn×n is the diagonal matrix of the top n singular values,

Vn ​∈ Rj×n contains the first n right singular vectors.

This produces a rank-n approximation capturing the dominant dynamics of the dataset.

Creating a time-shifted Hankel matrix (HMx)– H(1)

Construct each next-step HMx

H(1) = [Y(2)Y(3)⋮Y(i+1)​Y(3)Y(4)⋮Y(i+2)​⋯⋯⋱⋯​Y(j+1)Y(j+2)⋮Y(i+j)​​], using the same block dimensions as H(0).

Compute reduced  -order state matrix \hat{A}

\hat{A} = Σ{n,−1/2} ​​U{n,T}​H(1)Vn​Σ{n,−1/2}​​.

Reasoning: U{n,T} H(1)Vn projects the one-step-ahead data onto your n principal subspaces

Python example for ERA instantiation:

#!/usr/bin/env python3

"""

ERA Example:

– Build H(0), H(1) block-Hankel from measured Markov parameters Y(k)

– Compute SVD, truncate to order r

– Extract A_hat, B_hat, C_hat, D_hat

– Wrap in python-control StateSpace and validate via simulation

"""

import numpy as np

from scipy.linalg import svd

import control              # python-control

import matplotlib.pyplot as plt

import joblib               # for optional model persistence

# 1. Load your Markov parameters (impulse/step responses)

#    Assume you have CSV files Y1.csv, Y2.csv, … each p×T (p outputs × T samples)

#    from an impulse on input i.  We'll stack them so Y_all shape = (p*m, T).

m = 2   # number of input/disturbance channels

p = 1   # number of measured outputs

T = 200 # number of time‐samples per response

# load each response into a list, then stack vertically

Y_list = []

for inp in range(1, m+1):

# each file holds p×T matrix of output vs time for a unit impulse on input `inp`

Yi = np.loadtxt(f"Y{inp}.csv", delimiter=",")  # shape (p, T)

assert Yi.shape[1] == T

Y_list.append(Yi)

# Y_all has shape (p*m, T)

Y_all = np.vstack(Y_list)

# 2. Build block-Hankel matrices H0 = H(0) and H1 = H(1)

#    H0, H1 ∈ ℝ^{(p·i)×j}

def block_hankel(Y, i, j):

"""Build a block-Hankel matrix of shape (Y.shape[0]*i, j)

from Y[:, 0:(i+j-1)].

"""

p, T_all = Y.shape

H = np.zeros((p*i, j))

for row in range(i):

H[row*p:(row+1)*p, :] = Y[:, row:row+j]

return H

i = 10    # # block-rows

j = 40    # # block-cols  (must satisfy i+j-1 <= T)

assert i + j - 1 <= T

H0 = block_hankel(Y_all, i, j)        # past responses

H1 = block_hankel(Y_all[:,1:], i, j)   # one-step-ahead (shifted)

# 3. SVD + Truncate

#    H0 ≈ U_r Σ_r V_r^T, keep r modes

U, s, Vh = svd(H0, full_matrices=False)

# decide r by checking the energy in s (e.g. 95% rule) or a fixed choice

energy = np.cumsum(s**2) / np.sum(s**2)

r = np.searchsorted(energy, 0.95) + 1

print(f"Retaining {r} modes to capture 95% energy")

U_r = U[:, :r]               # shape (p·i, r)

S_r = np.diag(s[:r])         # shape (r, r)

V_r = Vh.conj().T[:, :r]     # shape (j, r)

# 4. Form reduced‐order A_hat

#    Â = Σ_r^(–1/2) · U_r^T · H1 · V_r · Σ_r^(–1/2)

S_inv_sqrt = np.diag(1.0/np.sqrt(s[:r]))

A_hat = S_inv_sqrt @ (U_r.T @ H1 @ V_r) @ S_inv_sqrt

# 5. Recover C_hat and B_hat (and D = first Markov parameter)

#    Ĉ = first p rows of U_r · Σ_r^(1/2)

#    B̂ = first m cols of (Σ_r^(1/2) · V_r^T)

#    D  = Y_all[:,0] reshaped to (p, m)

S_sqrt = np.diag(np.sqrt(s[:r]))

C_hat = (U_r @ S_sqrt)[:p, :]          # (p × r)

B_hat = (S_sqrt @ V_r.T)[:r, :m]       # (r × m)

D_hat = Y_all[:,0].reshape(p, m)       # direct feedthrough

# 6. Wrap in python-control StateSpace and Validate

# continuous-time: use Ts=None; for discrete-time supply Ts

sys_era = control.ss(A_hat, B_hat, C_hat, D_hat, Ts=None)

print(sys_era)

# 6.1 One‐step prediction vs measured

time = np.arange(T)*1.0  # assume unit sampling

# we need an input sequence for validation – here reuse Y impulses as an example

U_val = np.vstack(Y_list)  # shape (m*T)×length?  for demo only

_, y_pred, _ = control.forced_response(sys_era, T=time, U=U_val.T)

y_pred = y_pred.T

residuals = Y_all[:p] - y_pred[:p]

# plot

plt.figure(figsize=(8,4))

plt.plot(time, Y_all[0], 'k',    label='Measured y₁')

plt.plot(time, y_pred[0],'r--',  label='One-step pred')

plt.title("One‐Step Prediction vs. Measured")

plt.legend(); plt.show()

# 6.2 Multi-step (free‐run) simulation

_, y_sim, _ = control.forced_response(sys_era, T=time, U=U_val.T)

y_sim = y_sim.T

plt.figure(figsize=(8,4))

plt.plot(time, Y_all[0], 'k',   label='Measured y₁')

plt.plot(time, y_sim[0], 'b:',  label='Free-run sim')

plt.title("Multi-Step (Open‐Loop) Simulation")

plt.legend(); plt.show()

# 7. (Optional) Save your ERA model for deployment

joblib.dump(sys_era, "era_model.pkl")

print("Saved ERA model to era_model.pkl")

END CODE

Commnets: This can be extended to:

Add noise‐robustness via truncating to different energy levels.

Include disturbance channels by stacking their responses in Y_All.

Deploy ‘eramodel.pkl‘`era_model.pkl`‘eram​odel.pkl‘ in your MPC or a real‐time loop.

Compare ERA vs. N4SID/MOESP by swapping in control.n4sid or control.moesp.

Variables:

H(0): Block-Hankel matrix at lag 0, in ℝi×j, constructed from output response data.

ℝi×j: the space of all real-valued matrices with i rows and j columns. The set of all real i by j matrices (an i⋅j dimensional real vector space).

Y(k): Output response at time k, in ℝp (p outputs, e.g., from impulse response).

i: Number of block rows in Hankel matrix, integer, related to data length.

j: Number of block columns in Hankel matrix, integer.

U: Left singular vectors from SVD, in ℝi×i, orthogonal matrix where the dot product of U and U transform = the Identity matrix for the ℝi×i space.

Σ: Singular values, diagonal matrix in ℝi×j, non-negative diagonal ‘gain’ matrix.

V: Right singular vectors, in ℝj×j, orthogonal matrix where the dot product of V and V transform = the Identity matrix for the ℝj×j space.

\hat{A}: Reduced-order state transition matrix, in ℝn×n, where n is chosen order.

Σn​: Truncated singular values, in ℝn×n, top n singular values.

Un​: Truncated left singular vectors, in ℝi×n.

Vn​: Truncated right singular vectors, in ℝj×n.

H(1): Hankel matrix at lag 1, in ℝi×j, shifted version of H(0).

Ref: https://en.wikipedia.org/wiki/Eigensystem_realization_algorithm

Non-linear ARX Neural Net (NARX): While PID excels in linear, single-loop scenarios but falters with nonlinearities, interactions multiple disruptive variables, or variable delays which are all common in chemical reactors, distillation columns, or batch processes. NARX provides a means  of improving on this, the hybrid NARX-MPC provides superior disturbance rejection and can be implemented via edge computing for low-latency control. Below is a step-by-step blueprint for implementing NARX in Python using a Distillation Column multi-section LWVC + LWC condensing system.

When to Use: For nonlinear dynamics with known delays; extends ARX for processes like distillation condenser loops.

Base Equations: \hat{y}(k) = f(y(k−1), …, y(k−ny), u(k−1), …, u(k−nu)),

where f is a feed-forward neural network; delays embed dynamics.

Data Collection and Preparation: Collect historical or real-time data from your facility's SCADA/PLC systems.  TIP: Use IT/OT integration to stream data via MQTT to an onsite or cloud database for large-scale training, then deploy lightweight models on edge; monitor BTU trends (delta between cooling water return and supply temps) for fouling alerts. **If your data has irregular sampling, you'd preprocess with interpolation (e.g., via SciPy's interp1d), but here we assume uniform dt for simplicity**.

Python example (NumPy & scikit-learn):

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import MinMaxScaler

# Load data (e.g., from CSV exported from SCADA)

data = np.loadtxt('condenser_data.csv', delimiter=',')

’’’Columns: time, u_FCV, APV_cw_flow, APV_reflux_flow, APV_cw_return_T, y_vent_T, DV_vapor_P, DV_vapor_T, DV_cw_line_P, DV_cw_supply_T, DV_fac_cw_flow

’’’

u = data[:, 1:2]  # Inputs (m=1, cooling water FCV %)

y = data[:, 5:6]  # Outputs (p=1, condenser vent temp °F)

apv_dv = data[:, 2:5]

# APVs (cw flow GPM, reflux GPM, cw return T °F) and DVs can be added as features if needed

# Define lags (ny=3, nu=2 based on process knowledge/delays, e.g., longer lag for cw return T)

ny, nu = 3, 2

# Create lagged features (include APVs if modeling as extended inputs)

def create_lagged_data(u, y, nu, ny):

X = []  # Inputs to neural net: [y(k-1)...y(k-ny), u(k-1)...u(k-nu)]

Y = []  # Targets: y(k)

for k in range(max(nu, ny), len(y)):

lagged_y = y[k-ny:k][::-1].flatten()  # Past y's (vent T)

lagged_u = u[k-nu:k][::-1].flatten()  # Past u's (FCV %)

X.append(np.concatenate([lagged_y, lagged_u]))

Y.append(y[k])

return np.array(X), np.array(Y)

X, Y = create_lagged_data(u, y, nu, ny)

# Scale data (normalize to [0,1] for neural stability)

scaler_X = MinMaxScaler()

scaler_Y = MinMaxScaler()

X_scaled = scaler_X.fit_transform(X)

Y_scaled = scaler_Y.fit_transform(Y)

# Split: 70% train, 15% val, 15% test

X_train, X_temp, Y_train, Y_temp = train_test_split(X_scaled, Y_scaled, test_size=0.3, shuffle=False)  # Time-series: no shuffle

X_val, X_test, Y_val, Y_test = train_test_split(X_temp, Y_temp, test_size=0.5, shuffle=False)

Define the NARX Model: Why NARX over PID? -   NARX handles nonlinear mappings (e.g., heat transfer in condensers affected by DV vapor pressure) via activation functions like ReLU, embedding delays for lag/deadtime in cw return T.

Python Example (pytorch):

import torch

import torch.nn as nn

import torch.optim as optim

class NARXNet(nn.Module):

def __init__(self, input_dim, output_dim, hidden_dim=64):

super(NARXNet, self).__init__()

self.fc1 = nn.Linear(input_dim, hidden_dim)  # Input: ny*p + nu*m

self.fc2 = nn.Linear(hidden_dim, hidden_dim)

self.fc3 = nn.Linear(hidden_dim, output_dim)  # Output: p

self.relu = nn.ReLU()

def forward(self, x):

x = self.relu(self.fc1(x))

x = self.relu(self.fc2(x))

return self.fc3(x)

# Dimensions: input_dim = ny * p + nu * m (e.g., 3*1 + 2*1 = 5)

p, m = 1, 1  # From your process

input_dim = ny * p + nu * m

model = NARXNet(input_dim, p)

# To GPU if available (for faster training in cloud)

device = torch.device("cuda" if torch.cuda.is_available() else "cpu")

model.to(device)

Train the model: Use MSE (means squared error) loss for regression, Adam optimizer. Train on historical data; monitor for overfitting. HINT: Retrain periodically with OT data streams to adapt to DVs (e.g., cw supply T fluctuations), using IT cloud or intranet server capacity for hyperparameter tuning and BTU delta analysis for fouling prediction.

Python Example (PyTorch):

# Convert to tensors

X_train_tensor = torch.tensor(X_train, dtype=torch.float32).to(device)

Y_train_tensor = torch.tensor(Y_train, dtype=torch.float32).to(device)

# Similarly for val

criterion = nn.MSELoss()

optimizer = optim.Adam(model.parameters(), lr=0.001)

epochs = 200

for epoch in range(epochs):

model.train()\

optimizer.zero_grad()

outputs = model(X_train_tensor)

loss = criterion(outputs, Y_train_tensor)

loss.backward()

optimizer.step()

# Validate

model.eval()

with torch.no_grad():

val_outputs = model(X_val_tensor)

val_loss = criterion(val_outputs, Y_val_tensor)

print(f"Epoch {epoch+1}, Train Loss: {loss.item():.4f}, Val Loss: {val_loss.item():.4f}")

# Save model for deployment

torch.save(model.state_dict(), 'narx_condenser.pth')

Validate and Tune: Simulate predictions; compare against PID baselines using metrics like MSE or FIT%. Use SciPy for residual analysis.

Python example (SciPy & python-control (pip install control):

from scipy.stats import norm  # For residual whiteness test

# Predict on test

model.eval()

with torch.no_grad():

Y_pred_scaled = model(torch.tensor(X_test, dtype=torch.float32).to(device)).cpu().numpy()

Y_pred = scaler_Y.inverse_transform(Y_pred_scaled)

# MSE

mse = np.mean((Y_test_unscaled - Y_pred)**2)  # Assume unscaled Y_test

# Residuals

residuals = Y_test_unscaled - Y_pred

# Whiteness test (e.g., autocorrelation)

from scipy.signal import correlate

autocorr = correlate(residuals[:,0], residuals[:,0], mode='full')  # For PPV

# If autocorr peaks only at zero lag, residuals are white.

# Compare to PID (simulate baseline with python-control)

import control

# Define simple PID system (tune via Ziegler-Nichols)

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])  # Laplace domain

# Simulate and compare errors

Deploy at edge: Integrate into OT loop: Load model on edge device, predict y^(k) \hat{y}(k) y^​(k) in real-time, use as feedforward in advanced control (e.g., adjust FCV based on predictions, factoring DVs like vapor pressure). IT/OT Convergence example: Use Docker for edge deployment; stream predictions to IT dashboard for monitoring. For dynamics, chain with MPC: Use NARX for nonlinear prediction in QP objective, previewing DVs like cooling water line pressure.

Python example (real-time inference):

# Load model

model.load_state_dict(torch.load('narx_condenser.pth'))

model.eval()

# Real-time loop (e.g., in PLC gateway script)

while True:

# Get current lagged data from sensors (e.g., via OPC UA)

current_lagged = np.concatenate([past_y.flatten(), past_u.flatten()])  # Update buffers (vent T, FCV)

input_tensor = torch.tensor(scaler_X.transform([current_lagged]), dtype=torch.float32).to(device)

pred = model(input_tensor).cpu().numpy()

pred_y = scaler_Y.inverse_transform(pred)[0]  # Predicted PPV (vent T)

# Use in control: adjust FCV if pred_y deviates from 149°F SP

# Send to PLC

time.sleep(Ts)  # Sampling time

Monitor, Retrain, and Innovate: Monitor via plc-gbt analytics (e.g., MSE thresholds trigger alerts; track BTU delta for condenser fouling). Retrain weekly with new data to handle DVs (Varies by season – will take years to establish seasonal cycles without retrain requirements.

Innovations for US Manufacturing: Hybrid NARX-SINDy for interpretable nonlinear terms (use pysindy to sparsify f f f); integrate with RL (e.g., SAC via stable-baselines3) for policy optimization over NARX predictions, incorporating APV purity and DV vapor temp. This enables autonomous, resilient control—propelling efficiency and dominance.

Variables:

\hat{y}(k): Predicted output (PV) at time k, in ℝp.

f: Nonlinear function (neural network), maps inputs to output, unitless.

y(k−1), …, y(k−ny): Past outputs at times k−1 to k−ny, in ℝp.

ny​: Number of lagged outputs, integer.

u(k−1), …, u(k−nu): Past inputs (MVs) at times k−1 to k−nu​, in ℝm.

nu​: Number of lagged inputs, integer.

Ref: https://en.wikipedia.org/wiki/Nonlinear_autoregressive_model

Recurrent Neural Net (LSTM): LSTM empowers complex control operations with sequence-aware intelligence

When to Use:

For sequential data with long dependencies; innovative for predictive maintenance in IT/OT-integrated systems.

For sequential data with long dependencies; innovative for predictive maintenance in IT/OT-integrated systems. LSTM shines in processes like maintaining alcohol proofs of vapor sections, where historical patterns (e.g., persistent DV impacts from vapor temperature or cooling water line pressure) influence current states, enabling better handling of deadtimes (e.g., in cooling water return T) than feed-forward models like NARX.

Use when PID fails on multivariable, time-varying disturbances, such as fouling trends or beerfeed supply temp and flow variations, to predict vent T deviations and preemptively adjust FCV for stable steam feeds @ user defined SP.

Data Collection and preparation: Collect historical or real-time data from your facility's SCADA/PLC systems. **If your data has irregular sampling, you'd preprocess with interpolation (e.g., via SciPy's interp1d), but here we assume uniform dt for simplicity**.

Python example (Numby & scilit-learn):

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import MinMaxScaler

# Load data (e.g., from CSV exported from SCADA)

data = np.loadtxt('condenser_data.csv', delimiter=',')

# Columns: time, u_FCV, APV_cw_flow, APV_reflux_flow, APV_cw_return_T, y_vent_T, DV_vapor_P, DV_vapor_T, DV_cw_line_P, DV_cw_supply_T, DV_fac_cw_flow

u = data[:, 1:2]  # Inputs (m=1, cooling water FCV %)

y = data[:, 5:6]  # Outputs (p=1, condenser vent temp °F)

apv_dv = data[:, 2:5]

# APVs (cw flow GPM, reflux GPM, cw return T °F) and DVs can be added as features if needed

# Define sequence length (seq_len=10 for long dependencies, e.g., capturing deadtime in cw return T)

seq_len = 10

# Create sequences

def create_sequences(u, y, seq_len):

X = []  # Sequences: [ [xt- seq_len+1, ..., xt] ] where xt includes u(t), y(t-1), etc.

Y = []  # Targets: y(t+1) for prediction

for t in range(seq_len, len(y)):

seq_x = np.concatenate([u[t-seq_len:t], y[t-seq_len:t-1]], axis=1)  # Shape: (seq_len, m + p - something; adjust)

X.append(seq_x)

Y.append(y[t])

return np.array(X), np.array(Y)

X, Y = create_sequences(u, y, seq_len)

# Scale data (normalize to [0,1] for neural stability)

scaler_X = MinMaxScaler()

scaler_Y = MinMaxScaler()

X_scaled = scaler_X.fit_transform(X.reshape(-1, X.shape[-1])).reshape(X.shape)

Y_scaled = scaler_Y.fit_transform(Y)

# Split: 70% train, 15% val, 15% test

X_train, X_temp, Y_train, Y_temp = train_test_split(X_scaled, Y_scaled, test_size=0.3, shuffle=False)  # Time-series: no shuffle

X_val, X_test, Y_val, Y_test = train_test_split(X_temp, Y_temp, test_size=0.5, shuffle=False)

Define the LSTM Model: Model the LSTM equations defined under equations section below. Why Over PID?: LSTM's gates manage long dependencies (e.g., persistent DV effects like steam supply pressure P on reactor or column), mitigating vanishing gradients for better prediction of lags/deadtime in vapor side outlet temp.

Python example: (PyTorch):

import torch

import torch.nn as nn

import torch.optim as optim

class LSTMNet(nn.Module):

def __init__(self, input_dim, hidden_dim, output_dim, num_layers=1):

super(LSTMNet, self).__init__()

self.hidden_dim = hidden_dim

self.num_layers = num_layers

self.lstm = nn.LSTM(input_dim, hidden_dim, num_layers, batch_first=True)  # Input: seq_len x batch x input_dim

self.fc = nn.Linear(hidden_dim, output_dim)  # Output from last hidden state

def forward(self, x):

h0 = torch.zeros(self.num_layers, x.size(0), self.hidden_dim).to(x.device)

c0 = torch.zeros(self.num_layers, x.size(0), self.hidden_dim).to(x.device)

out, _ = self.lstm(x, (h0, c0))  # out: batch x seq_len x hidden_dim

out = self.fc(out[:, -1, :])  # Last time step

return out

# Dimensions: input_dim = m + p (e.g., 1 + 1 = 2 for u_FCV and past y_vent_T)

input_dim = 2  # Adjust based on xt (u + past y)

hidden_dim = 64

output_dim = 1  # p=1 for vent T

model = LSTMNet(input_dim, hidden_dim, output_dim)

# To GPU if available (for faster training in cloud)

device = torch.device("cuda" if torch.cuda.is_available() else "cpu")

model.to(device)

Train the model: Use MSE loss for regression, Adam optimizer. Train on sequences; monitor for overfitting.

Python example: (PyTorch):

# Convert to tensors

X_train_tensor = torch.tensor(X_train, dtype=torch.float32).to(device)

Y_train_tensor = torch.tensor(Y_train, dtype=torch.float32).to(device)

# Similarly for val

criterion = nn.MSELoss()

optimizer = optim.Adam(model.parameters(), lr=0.001)

epochs = 200

for epoch in range(epochs):

model.train()

optimizer.zero_grad()

outputs = model(X_train_tensor)

loss = criterion(outputs, Y_train_tensor)

loss.backward()

optimizer.step()

# Validate

model.eval()

with torch.no_grad():

val_outputs = model(X_val_tensor)

val_loss = criterion(val_outputs, Y_val_tensor)

print(f"Epoch {epoch+1}, Train Loss: {loss.item():.4f}, Val Loss: {val_loss.item():.4f}")

# Save model for deployment

torch.save(model.state_dict(), 'lstm_condenser.pth')

Validate and Tune: Simulate predictions; compare against PID baselines using metrics like MSE or FIT%. Use SciPy for residual analysis.

Python example (SciPy & control (python-control):

from scipy.stats import norm  # For residual whiteness test

# Predict on test

model.eval()

with torch.no_grad():

Y_pred_scaled = model(torch.tensor(X_test, dtype=torch.float32).to(device)).cpu().numpy()

Y_pred = scaler_Y.inverse_transform(Y_pred_scaled)

# MSE

mse = np.mean((Y_test_unscaled - Y_pred)**2)  # Assume unscaled Y_test

# Residuals

residuals = Y_test_unscaled - Y_pred

# Whiteness test (e.g., autocorrelation)

from scipy.signal import correlate

autocorr = correlate(residuals[:,0], residuals[:,0], mode='full')  # For PPV

# If autocorr peaks only at zero lag, residuals are white.

# Compare to PID (simulate baseline with python-control)

import control

# Define simple PID system (tune via Ziegler-Nichols)

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Laplace domain

# Simulate and compare errors

Deploy with extreme prejudice : Integrate into OT loop: Load model on edge device, predict future alcohol T in real-time, with density readings as validation and feedforward in advanced control (e.g., adjust steam FCV based on sequences, factoring DVs like steam pressure).

Python example (real-time inference):

# Load model

model.load_state_dict(torch.load('lstm_condenser.pth'))

model.eval()

# Real-time loop (e.g., in PLC gateway script)

sequence_buffer = []  # Maintain rolling sequence of last seq_len steps

while True:

# Get current data from sensors (e.g., via OPC UA): append to buffer

current_xt = np.array([current_u_FCV, prev_y_vent_T])  # xt includes u, past y

sequence_buffer.append(current_xt)

if len(sequence_buffer) > seq_len:

sequence_buffer.pop(0)

input_seq = np.array([sequence_buffer])  # Shape: 1 x seq_len x input_dim

input_tensor = torch.tensor(scaler_X.transform(input_seq.reshape(-1, input_dim)).reshape(1, seq_len, input_dim), dtype=torch.float32).to(device)

pred = model(input_tensor).cpu().numpy()

pred_y = scaler_Y.inverse_transform(pred)[0]  # Predicted PPV (vent T)

# Use in control: adjust FCV if pred_y deviates from 149°F SP

# Send to PLC

time.sleep(Ts)  # Sampling time

Monitor, retrain, and improve: CI/CD innovation of OT processes

Monitor via analytics – plc-gbt (e.g., MSE thresholds, process trigger alerts, dynamic upstearm processes to find control variation correlation, etc.).

Implement periodic retraining with new data to handle DVs as edge cases emerge.

Consider future improvements: These will enable more autonomous, resilient control.

Instantiation of Hybrid LSTM-SINDy for interpretable long-term terms (use pysindy on LSTM features).

Integrate with RL (e.g., SAC via stable-baselines3) for policy optimization over LSTM predictions, incorporating APV purity and additional DVs based on continued monitor for new correlation.

Equations:

Forget gate: ft = σ(Wf[ht−1, xt] + bf).
Input gate: it = σ(Wi[ht−1, xt] + bi),
Cell update: \tilde{c}t = tanh(Wc[ht−1, xt] + bc), ct = ft ⊙ ct−1 + it ⊙ \tilde{t}​,
Output gate: ot = σ(Wo[ht−1, xt] + bo), ht = ot ⊙ tanh(ct),

where xt​ includes past y, u.

Variables:

ft​: Forget gate activation at time t, in ℝh (h hidden units), range [0,1].

\sigma σ: Sigmoid function, maps to value range of [0,1], unitless.

Wf​: Weight matrix for forget gate, in ℝh×(h+d), where d is input dimension.

ht−1​: Previous hidden state, in ℝh.

xt​: Input at time t, in ℝd (includes past y, u).

bf​: Bias for forget gate, in ℝh.

it​: Input gate activation, in ℝh, range [0,1].

Wi​: Weight matrix for input gate, in ℝh×(h+d).

ℝh×(h+d): the set of all real-valued matrices with h rows and h + d columns.

bi​: Bias for input gate, in ℝh.

\tilde{c}t​: Candidate cell state, in ℝh.

tanh: Hyperbolic tangent function, maps to [-1,1], unitless.

Wc​: Weight matrix for cell update, in ℝh×(h+d).

bc​: Bias for cell update, in ℝh.

ct​: Cell state at time t, in ℝh.

ct−1​: Previous cell state, in ℝh.

\odot ⊙: Hadamard (element-wise) product, unitless.

ot​: Output gate activation, in ℝh, range [0,1].

Wo​: Weight matrix for output gate, in ℝh×(h+d).

bo​: Bias for output gate, in ℝh.

ht​: Hidden state (output) at time t, in ℝh.

Ref: https://en.wikipedia.org/wiki/Long_short-term_memory

Gaussian Process Regression (GPR): This GPR guide equips your tower loop with uncertainty-aware intelligence

When to Use: For uncertainty quantification in small-data regimes; pairs well with MPC for robust manufacturing.

GPR is ideal for complex, disruptive loops like tower supply water control, where limited training data (e.g., from intermittent process calls) and high variability (DVs like return temp or basin levels) demand probabilistic predictions.

Use when PID oversimplifies nonlinear interactions (e.g., pump speed vs. pressure under varying loads), to forecast PPV with confidence bounds—enabling safe overrides if uncertainty spikes (e.g., governing fermenter flow if supply temp risks exceeding max).

It's especially valuable for systems with safety margins, like ensuring condenser cooling amid BTU trends, over parametric models like ARX when data is sparse or noisy.

Data Collection and Preparation: Collect historical or real-time data from your facility's SCADA/PLC systems. For tower supply water: MVs (u: VFD speed 0-100% for pumps), PPV (y: supply pressure 0-75 psi), other vars (supply flow 0-3200 GPM, process flows, temps, basin levels, fan speeds). **If your data has irregular sampling, you'd preprocess with interpolation (e.g., via SciPy's interp1d), but here we assume uniform dt for simplicity**.

Python example: (Munby & scikit-learn):

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import StandardScaler

# Load data (e.g., from CSV exported from SCADA)

data = np.loadtxt('tower_data.csv', delimiter=',')  # Columns: time, u_VFD, y_pressure, supply_flow, mash_cooler_flow1, mash_cooler_flow2, fermenter_flow, lwwc_flow, hwwc_flow, supply_T, return_T, hot_well_T, hot_well_level, cold_well_T, cold_well_level, condenser_fan1, condenser_fan2, hot_well_fan

u = data[:, 1:2]  # Inputs (m=1, VFD speed %)

y = data[:, 2:3]  # Outputs (p=1, supply pressure psi)

features = data[:, 3:]  # Additional vars/DVs as inputs (e.g., flows, temps for multivariable GPR)

# Combine inputs: X = [u, features] for full context

X = np.hstack([u, features])  # Shape: (samples, input_dim=17+)

# Scale data (standardize for GPR kernel stability)

scaler_X = StandardScaler()

scaler_Y = StandardScaler()

X_scaled = scaler_X.fit_transform(X)

Y_scaled = scaler_Y.fit_transform(y)

# Split: 70% train, 15% val, 15% test (GPR uses all for hyperparam opt, but split for eval)

X_train, X_temp, Y_train, Y_temp = train_test_split(X_scaled, Y_scaled, test_size=0.3, random_state=42)

X_val, X_test, Y_val, Y_test = train_test_split(X_temp, Y_temp, test_size=0.5, random_state=42)

Define the GPR Model: Model y(x) ∼ GP(m(x), k(x,x′)), with predictive mean/variance from data conditioning. Use RBF kernel for smooth nonlinearities.

Why this model over PID: GPR provides variance estimates (e.g., high uncertainty during high-demand flows triggers safe VFD slowdowns), handling sparse data and DVs better than deterministic models.

Python example (scikit-learn):

from sklearn.gaussian_process import GaussianProcessRegressor

from sklearn.gaussian_process.kernels import RBF, ConstantKernel, WhiteKernel

# Define kernel: Constant * RBF + White (for noise)

kernel = ConstantKernel(1.0) * RBF(1.0, length_scale_bounds=(1e-2, 1e3)) + WhiteKernel(noise_level=1e-5)

model = GaussianProcessRegressor(kernel=kernel, n_restarts_optimizer=10, alpha=0.01)  # Alpha for noise

# No explicit device; GPR is CPU-based in scikit-learn

Train the model: Fit GPR on training data; optimize hyperparameters via likelihood maximization. Always retrain periodically – there are also means of continuous learning that will be a future discussion.

Python example (scikit-learn): simplified process

# Fit model

model.fit(X_train, Y_train)

# No epochs; GPR optimizes internally

print(f"Optimized kernel: {model.kernel_}")

Validate and tune: Predict with uncertainty; compare against PID using metrics like MSE or coverage (e.g., 95% predictions within bounds). Use SciPy for residual analysis.

Python example (SciPy & control):

from scipy.stats import norm  # For confidence intervals

# Predict on test: mean and std

Y_pred, sigma = model.predict(X_test, return_std=True)

Y_pred_unscaled = scaler_Y.inverse_transform(Y_pred.reshape(-1,1)).flatten()

sigma_unscaled = sigma * scaler_Y.scale_  # Approximate scaling

# MSE

mse = np.mean((Y_test_unscaled - Y_pred_unscaled)**2)

# Coverage: Check if true values within 2*sigma (95% CI)

coverage = np.mean(np.abs(Y_test_unscaled - Y_pred_unscaled) < 2 * sigma_unscaled)

# Residuals

residuals = Y_test_unscaled - Y_pred_unscaled

from scipy.signal import correlate

autocorr = correlate(residuals, residuals, mode='full')

# Compare to PID (simulate baseline with python-control)

import control

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])  # Laplace domain

# Simulate and compare errors

Deploy in Industrial Facility: Integrate into OT loop: Fit model on edge (GPR is lightweight), predict pressure with uncertainty, adjust VFD if low confidence (e.g., govern mash cooler flow).

Python example (Real-time inference):

# Assume model fitted; for real-time, refit periodically or use online GPR variants

# Real-time loop (e.g., in PLC gateway script)

while True:

# Get current features from sensors (e.g., via OPC UA)

current_X = np.array([current_u_VFD, current_supply_flow, ...])  # All inputs/DVs

input_scaled = scaler_X.transform([current_X])

pred_scaled, sigma_scaled = model.predict(input_scaled, return_std=True)

pred_y = scaler_Y.inverse_transform(pred_scaled.reshape(1,-1))[0][0]  # Predicted pressure

sigma = sigma_scaled[0] * scaler_Y.scale_[0]  # Uncertainty

# Use in control: e.g., adjust VFD if pred_y < SP or sigma > threshold

# Send to PLC

time.sleep(Ts)  # Sampling time

Monitor, Retrain, and Innovate:

Monitor via IT analytics (e.g., uncertainty thresholds trigger alerts; track BTU deltas for governing).

Retrain daily with new data to handle DVs.

Innovations for US Manufacturing: Hybrid GPR-PILCO for uncertainty-aware RL (use GPy if expanded); integrate with SINDy for sparse dynamics discovery on GPR priors, incorporating process flows for demand forecasting. This enables autonomous, probabilistic control—propelling efficiency and dominance.

Primary Equations: Predictive mean/variance from conditioning on data under kernel k.

y(x) ∼ \mathcal{GP} (m(x), k(x, x′)),

Variables:

y(x): Output function, in ℝ, evaluated at input x (e.g., process output).

x: Input vector, in ℝd (e.g., process conditions, controls).

\mathcal{GP}: Gaussian Process, a distribution over functions, unitless.

m(x): Mean function, in ℝ, prior mean of output at x (often 0).

k(x, x′): Covariance (kernel) function, in ℝ, measures similarity between inputs x, x' (e.g., squared exponential kernel).

x′ (x prime): Another input vector, in ℝd, for covariance computation.

Ref: https://en.wikipedia.org/wiki/Gaussian_process_regression

Sparse Identification of Nonlinear Dynamics (SINDy): Most will find this aalgorithm easy to implement because it has its own python lib (https://github.com/dynamicslab/pysindy.git). We will use boiler system steam pressure control as the best-fitting use-case example for SINDy. It's a highly nonlinear process involving phase changes, heat transfer, and disturbances like fuel variations or load demands, where SINDy can discover compact equations for pressure stability (e.g., balancing evaporation rates and valve positions). This enables predictive, adaptive strategies that minimize fuel waste, enhance safety (e.g., avoiding overpressure), and integrate with emerging IIoT for real-time equation updates

When to Use: For discovering sparse governing equations; innovative for physics-informed ML in automation.  When maximizing your ability to run in an optimal state is of highest priority.

SINDy is ideal for dynamical systems like boiler steam pressure control, where data reveals a few key nonlinear terms (e.g., quadratic pressure dependencies or interactions with temperature), enabling interpretable models over black-box alternatives.

Use when PID underperforms on multiple complex feedback variables (e.g., saturation effects or load disturbances), to derive equations that inform MPC or fault detection—especially in sparse-data scenarios with physical intuition, like ensuring stable pressure amid varying fuel quality or demand spikes.

Data Collection and Preparation: Collect historical or real-time data from your facility's SCADA/PLC systems. For boiler steam pressure: MVs (u: fuel valve %), PPV (x: pressure psi), other states (drum level, temp). **If your data has irregular sampling, you'd preprocess with interpolation (e.g., via SciPy's interp1d), but here we assume uniform dt for simplicity**.

Python example (NumPy and SciPy):

import numpy as np

from scipy.signal import savgol_filter  # For smoothing/derivatives

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import StandardScaler

# Load data (e.g., from CSV exported from SCADA)

data = np.loadtxt('boiler_data.csv', delimiter=',')  # Columns: time, u_valve, x_pressure, drum_level, temp, fuel_flow, ...

t = data[:, 0]  # Time

x = data[:, 1:4]  # States (n=3, e.g., pressure, level, temp)

# Compute derivatives (dot_x) using Savitzky-Golay filter

dt = np.mean(np.diff(t))

dot_x = savgol_filter(x, window_length=5, polyorder=2, deriv=1, delta=dt, axis=0)

# Scale data

scaler_x = StandardScaler()

scaler_dot_x = StandardScaler()

x_scaled = scaler_x.fit_transform(x)

dot_x_scaled = scaler_dot_x.fit_transform(dot_x)

# Split: 80% train, 20% test (SINDy uses all for fitting, but split for eval)

x_train, x_test, dot_x_train, dot_x_test = train_test_split(x_scaled, dot_x_scaled, test_size=0.2, shuffle=False)

Define the SINDy Model: Seek \dot{x} = Θ(x)ξ with sparse ξ; define library Θ(x) (e.g., polynomials up to degree 3).

Why Over PID?: SINDy yields interpretable equations (e.g., \dot{p} = ap + bup2) for pressure dynamics, enabling physics-based simulation over PID's reactive tuning.

Python example (pysindy):

from pysindy import SINDy

from pysindy.feature_library import PolynomialLibrary

# Define library: polynomials degree 3

library = PolynomialLibrary(degree=3)

# Instantiate model with thresholding optimizer

model = SINDy(feature_library=library, optimizer='STLSQ', optimizer__threshold=0.1)  # Sparse threshold

Train the Model: Fit SINDy on states and derivatives; print discovered equations. Training this model is veery straight forward because of the pysindy library.

Python example (pysindy):

# Fit model

model.fit(x_train, t=dt, x_dot=dot_x_train)  # dt from data

# Print equations

model.print()

# Example output: x0' = a x0 + b x1^2 + ...

Validate and Tune: Simulate with discovered equations; compare against PID using metrics like MSE or sparsity (non-zero terms).

Python example (SciPy & control for simulation):

from scipy.integrate import odeint

# Simulate with SINDy equations

def sindy_eq(x, t):

return model.predict(x.reshape(1,-1)).flatten()  # Unscaled predict

x_sim = odeint(sindy_eq, x_test[0], t_test)  # Integrate

# MSE

mse = np.mean((x_test - x_sim)**2)

# Residuals

residuals = x_test[:,0] - x_sim[:,0]  # For pressure

from scipy.signal import correlate

autocorr = correlate(residuals, residuals, mode='full')

# Compare to PID (simulate baseline with python-control)

import control

# Laplace domain

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate and compare errors

Deploy for prod: Integrate into OT loop: Load model on edge, predict x˙ \dot{x} x˙ for forward simulation, adjust valve if pressure trends unsafe. IT OT convergence suggestion - Use Docker for edge deployment; stream equations to IT dashboard. For dynamics, chain with MPC: Use SINDy model in state predictions, previewing DVs like load demand,

Python example (real-time inference):

# Assume model fitted; for real-time, refit periodically

# Real-time loop (e.g., in PLC gateway script)

while True:

# Get current states from sensors (e.g., via OPC UA)

current_x = np.array([current_pressure, current_level, current_temp])  # Scaled

dot_x_pred = model.predict(current_x.reshape(1,-1))[0]  # Predicted derivatives

# Simulate short horizon

future_t = np.linspace(0, 60, 10)  # 1-min ahead

future_x = odeint(lambda x,t: model.predict(x.reshape(1,-1)).flatten(), current_x, future_t)

# Use in control: e.g., adjust valve if future pressure > max

# Send to PLC

time.sleep(Ts)  # Sampling time

Monitor, Retrain, and Innovate:

Monitor via analytics platform [plc-gbt] - (e.g., equation sparsity for model health; track residuals for anomalies).

Retrain weekly with new data to improve disturbance variable DV handling.

Further innovation: Hybrid SINDy-Koopman for linearized nonlinear terms (use kooplearn); integrate with RL (e.g., PETS via ensembles) for policy optimization over SINDy equations, incorporating fuel flow for demand forecasting. This enables autonomous, interpretable control

Base Equation(s):

Seek \dot{x} = Θ(x)ξ, where Θ(x) is a library (e.g., polynomials) and ξ is sparse. Use Least Absolute Shrinkage and Selection Operator (LASSO) or thresholding for parsimony.

Variables:

x˙ \dot{x} x˙: Time derivative of state vector, in ℝ^n, rate of change (e.g., velocity).

x x x: State vector, in ℝ^n (e.g., process states).

Θ(x) \Theta(x) Θ(x): Library matrix of candidate functions, in ℝ^{n×m} (m functions, e.g., polynomials x,x2 x, x^2 x,x2).

ξ (‘xi’ or ‘sigh’): Sparse coefficient vector, in ℝm, selects active terms (mostly zeros). ‘xi’ is the list of weights or coefficients that tells you how much each basis function in Θ contributes to the time‐derivative of x.

Ref: https://en.wikipedia.org/wiki/SINDy

Koopman Operator Approaches:

When to Use: For linearizing nonlinear systems; enables linear MPC on lifted states for complex dynamics.  I  will use control of a multi-stage shell and tube HEX with 9 disruptive variables which requires 3 main control loops for standard operation as the example.

Koopman approaches are ideal for processes like mash cooler control, where inherent nonlinearities (e.g., turbulent flow or temp-dependent heat coefficients) make traditional modeling hard, but data allows "lifting" states to a linear space via observables ψ(x).

Use when PID falters on multivariable chaos (e.g., inlet disturbances causing oscillations), to derive a linear operator K for efficient prediction and control—especially in data-rich environments with physical intuition, over black-box ML when interpretability and linear tools (e.g., Kalman filtering) are desired.

Note: Model has its own python lib: https://github.com/dynamicslab/pykoopman.git

Data collection and preparation: Collect historical or real-time data from your facility's SCADA/PLC systems. For mash cooler: States (x: TIT_MC01-1/2/3 temps, mash flow), MVs (u: FCV_MC01-1/2 %). **If data has irregular sampling, you'd preprocess with interpolation (e.g., via SciPy's interp1d), but here we assume uniform dt for simplicity**.

Python example (NumPy and scikit-learn):

import numpy as np

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import StandardScaler

# Load data (e.g., from CSV exported from SCADA)

data = np.loadtxt('mash_cooler_data.csv', delimiter=',')  # Columns: time, x_inlet_T (TIT_MC01-1), x_mid_T (TIT_MC01-2), x_outlet_T (TIT_MC01-3), x_mash_flow, u_FCV1, u_FCV2, DV_cook_cycle, ...

t = data[:, 0]  # Time

x = data[:, 1:5]  # States (n=4, inlet T °F, mid T °F, outlet T °F, mash flow GPM)

u = data[:, 5:7]  # Inputs (m=2, FCV_MC01-1 %, FCV_MC01-2 %)

# For controlled systems, augment states with u if needed

u = np.hstack([x, u])  # Extended states for observables

# Scale data

scaler = StandardScaler()

xu_scaled = scaler.fit_transform(xu)

# Split: 70% train, 30% test

xu_train, xu_test = train_test_split(xu_scaled, test_size=0.3, shuffle=False)

Define the Koopman Model: Lift x to observables ψ(x) (e.g., via polynomials), approximate K such that ψ(xk+1) ≈ Kψ(xk).

Why Koopman model over PID?: Koopman linearizes mash cooler's nonlinear heat transfer (e.g., delta T dependencies and section interactions) into a matrix K, enabling standard linear tools like LQR for SP-weighted control.

Python example (pykoopman):

from pykoopman import Koopman

from pykoopman.observables import Polynomial

from pykoopman.regression import EDMD  # Extended DMD

# Define observables: polynomials degree 3 (captures nonlinear T deltas)

observables = Polynomial(degree=3)

# Define regressor: EDMD for approximation

regressor = EDMD()

# Instantiate model

model = Koopman(observables=observables, regressor=regressor)

Train the model: Fit Koopman on consecutive state pairs; *extract operator K.

Perform a data-driven approximation of the Koopman operator K using the pykoopman library, leveraging Extended Dynamic Mode Decomposition (EDMD)

After slicing the training data into pairs: x_train = xu_train[:-1] (current states at time k, denoted as xk​) and x_next_train = xu_train[1:] (next states at time k+1, xk+1.

Create a dataset of transitions, where each row in x_train maps to the corresponding row in x_next_train. For MC01, xk​ might include states like inlet temp (TIT_MC01-1), mid-point temp (TIT_MC01-2), outlet temp (TIT_MC01-3), mash flow, and augmented MVs (FCV_MC01-1/2 positions). This pairing captures the discrete-time evolution of the system, assuming a fixed sampling interval (e.g., from your data's time column).

Fitting the Model (model.fit(x_train, x_next_train)):

The library applies the defined observables (e.g., Polynomial(degree=3)) to "lift" the states: It computes ψ(xk), a higher-dimensional representation (in Rm, where m >> n, the original state dimension). For example, if x has n=6 components (temps, flow, FCVs), a degree-3 polynomial might lift to m ~ 84 observables (monomials like x{1, 2}x2​, capturing nonlinear interactions like temp-flow couplings in MC01).

Then, it regresses the linear operator K (in Rm×m) such that ψ(xk+1) ≈ Kψ(xk)​). This is done via least-squares or SVD-based decomposition (in EDMD), minimizing the error over all pairs.

What's occurring under the hood: The fit constructs a matrix equation Ψnext ≈ KΨ, where Ψ is the matrix of lifted ψ(xk) for all training points, and Ψnext​ for ψ(xk+1). K is solved as K ≈ ΨnextΨ† (\dagger = pseudoinverse), often with regularization for stability.

Output: The model now holds K (accessed as model.A for discrete-time systems), along with functions to lift/inverse-lift states.

*Extracting the Operator K:

Retrieve K as model.A, a square matrix representing the discrete-time Koopman operator in the lifted space. Its shape is m x m (e.g., 84x84 for degree-3 on n=6).

Printing the shape confirms the lifting dimension, which grows combinatorially with degree (watch for computational cost in high-n systems like MC01 with added DVs).

This step is computationally efficient for moderate datasets (e.g., your 1,000-10,000 samples), typically taking seconds on a standard CPU, making it feasible for periodic.

Python example (pykoopman):

# Prepare data: x_k and x_{k+1}

x_train = xu_train[:-1]  # Current states

x_next_train = xu_train[1:]  # Next states

# Fit model

model.fit(x_train, x_next_train)

# Access K (linear operator)

K = model.A  # Discrete-time Koopman matrix

print(f"Koopman operator shape: {K.shape}")

Validate and tune: Simulate with lifted linear model; compare against PID using metrics like prediction error or eigenvalue stability.

Python example (SciPy and control for simulation):

from scipy.linalg import eig

# Predict multi-step

x_pred = np.zeros_like(x_test)

x_pred[0] = x_test[0]

for i in range(1, len(x_test)):

psi_current = model.observables.transform(x_pred[i-1].reshape(1,-1))

psi_next = psi_current @ model.A.T  # Linear propagation

x_pred[i] = model.observables.inverse(psi_next)  # Back to original space

# MSE

mse = np.mean((x_test - x_pred)**2)

# Stability: Eigenvalues of K

eigenvalues = eig(model.A)[0]

print(f"Max eigenvalue magnitude: {np.max(np.abs(eigenvalues))}")  # <1 for stability

# Compare to PID (simulate baseline with python-control)

import control

# Laplace domain

pid = control.TransferFunction([Kp * (1 + 1/(Ti*s) + Td*s)], [1])

# Simulate and compare errors

Deploy in Prod: Integrate into OT loop: Load model on edge, propagate lifted states for prediction, adjust FCV via linear control on ψ \psi ψ, factoring SP weighting.

Python example (Real-time Inference):

# Assume model fitted; for real-time, refit periodically

# Real-time loop (e.g., in PLC gateway script)

current_x = np.array([current_inlet_T, current_mid_T, current_outlet_T, current_mash_flow, current_FCV1, current_FCV2])  # Scaled

while True:

psi_current = model.observables.transform(current_x.reshape(1,-1))

psi_future = psi_current @ model.A.T  # One-step ahead

future_x = model.observables.inverse(psi_future)[0]  # Predicted states

# Use in control: e.g., linear MPC on psi to adjust FCV for target outlet T, applying SP weighting

# Send to PLC

current_x = future_x  # Update for next (or from sensors)

time.sleep(Ts)  # Sampling time

Monitor, Retrain, and Innovate:

Monitor via IT analytics (e.g., eigenvalue stability for model health; track residuals for anomalies like delta T deviations).

Retrain weekly with new data to handle varied disturbance variables DVs

Future improvements: Hybrid Koopman-SINDy for sparse observables (use pysindy on lifted space); integrate with RL (e.g., DDPG via stable-baselines3) for policy optimization on linear K, incorporating cook cycle DVs for demand forecasting. This enables autonomous, linearized control.

Primary Equation(s): ψ(xk+1) ≈ Kψ(xk) for linear operator K.

Variables:

ψ(xk+1) or \psi(x_{k+1}): Observable functions at time k+1, in ℝm (m observables, e.g., nonlinear transformations of x).

ψ(xk) or \psi(x_{k}): Observable functions at time k, in ℝm.

K: Koopman operator, in ℝm×m, linear operator in observable space.

xk​: State vector at time k, in ℝn, at each time k, your state xₖ is a vector of n real-valued components.

ℝ: n-dimensional list (or column) of real numbers.

Ref: https://en.wikipedia.org/wiki/Koopman_operator

Model-Based Reinforcement Learning (Reinforcement Learning (RL): Using Probabilistic Inference for Learning Control (PILCO) and Probabilistic Ensembles with Trajectory Sampling (PETS). Using a muti-process tower cooling supply feeder loop as our example for these algorithms because – Generally, they are rife with uncertainty from fluctuating demands from varied processes, temperature swings, at risk processes if supply temp exceeds a constrained max, and other multi-variable disruptions.

When to Use: When high system uncertainty exists due to complex disturbance variables.  Can bridges RL with process models for adaptive manufacturing.

PILCO and PETS are ideal for dynamic, stochastic loops like tower supply water control, where limited data and high variability (e.g., sudden process calls from condensers or fermenters) demand sample-efficient learning with uncertainty quantification.

Use PILCO for Gaussian Process-based modeling in small-data regimes (e.g., probabilistic predictions of pressure under basin level fluctuations).

Use PETS when neural ensembles better capture complex interactions (e.g., trajectory sampling for planning VFD speeds amid temp risks).

Apply when PID fails on long-term optimization (e.g., balancing flow across consumers while avoiding over-temp shutdowns), especially in safety-critical setups needing expected return gradients or uncertainty propagation for robust governing.

Why use Probabilistic Inference for Learning Control (PILCO) and Probabilistic Ensembles with Trajectory Sampling (PETS): PILCO and PETS shine in distillery operations by addressing PID's shortcomings in uncertain, multivariable systems—providing model-based RL that learns dynamics and policies with few trials, incorporating uncertainty to avoid catastrophic failures (e.g., insufficient cooling leading to lost batches).

Sample Efficiency: Unlike model-free RL (e.g., SAC, which requires millions of interactions), PILCO uses GPR for analytic gradients, achieving control in handfuls of episodes; PETS leverages ensembles for uncertainty-aware planning, matching this efficiency in higher dimensions.

Uncertainty Handling: In tower water, DVs like return temp or demand spikes create risks—PILCO/PETS quantify this (via variances or sampled trajectories), enabling safe exploration and robust policies (e.g., conservative VFD ramps if variance high).

Interpretability and Innovation: Models like GPR (PILCO) or ensembles (PETS) allow physics-informed priors (e.g., heat balance equations), fostering IT/OT hybrids: Edge execution for real-time flow allocation, cloud updates for adaptive governing (e.g., prioritizing condensers if temp uncertainty threatens ethanol yield).

Over Other Algorithms: Vs. SINDy/Koopman (deterministic discovery/linearization), these add RL for goal-oriented optimization; vs. GPR alone, they close the loop with policy learning—crucial for manufacturing dominance through autonomous, risk-aware systems.

Data collection and preparation: Collect historical data; for tower: States (x_t: pressure psi, supply flow GPM, supply/return temps °F, basin levels), actions (u_t: VFD speed %), rewards (r_t: -error in pressure/flow + penalty for temp > max).

Python example:

import numpy as np

import gymnasium as gym

from gymnasium import spaces

class TowerEnv(gym.Env):

def __init__(self):

super().__init__()

self.action_space = spaces.Box(low=0, high=1, shape=(1,))  # Normalized VFD speed

self.observation_space = spaces.Box(low=np.array([0, 0, 0, 0]), high=np.array([75, 3200, 212, 212]), shape=(4,))  # pressure, flow, supply T, return T

self.state = np.array([50, 1600, 70, 85])  # Initial state

def step(self, action):

# Simple dynamics simulation (replace with real model)

vfd = action[0] * 100  # Denormalize

self.state[0] += 0.5 * vfd - 0.1 * (self.state[3] - self.state[2])  # Pressure update

self.state[1] += 10 * vfd  # Flow update

reward = -abs(self.state[0] - 60) - 10 * max(0, self.state[2] - 80)  # Track 60 psi, penalize temp >80F

done = False

return self.state, reward, done, {}

def reset(self):

self.state = np.array([50, 1600, 70, 85]) + np.random.uniform(-10, 10, 4)

return self.state

env = TowerEnv()

# Collect initial data (e.g., random policy rollout)

states, actions, next_states, rewards = [], [], [], []

state = env.reset()

for _ in range(1000):

action = env.action_space.sample()

next_state, reward, _, _ = env.step(action)

states.append(state)

actions.append(action)

next_states.append(next_state)

rewards.append(reward)

state = next_state

states = np.array(states)

actions = np.array(actions)

next_states = np.array(next_states)

rewards = np.array(rewards)

Implement PILCO (GPR-Based): Model dynamics with GPR, optimize policy via gradients on expected return.

Python example (GPy for GPR, NumPy for optimization) https://github.com/SheffieldML/GPy.git:

import GPy

# Fit GPR dynamics (one for each state dim)

models = []

for i in range(states.shape[1]):

X = np.hstack([states, actions])

Y = next_states[:, i:i+1] - states[:, i:i+1]  # Delta states

kernel = GPy.kern.RBF(input_dim=X.shape[1], variance=1., lengthscale=1.)

gp = GPy.models.GPRegression(X, Y, kernel)

gp.optimize(messages=False)

models.append(gp)

# Policy (simple parametric, e.g., linear; optimize via gradients)

def policy(params, state):

# Example linear policy

return np.clip(np.dot(state, params[:4]) + params[4], 0, 1)

def expected_return(params, horizon=10):

state = env.reset()

total_r = 0

for _ in range(horizon):

action = policy(params, state)

mean_next, var_next = [], []

for i, gp in enumerate(models):

inp = np.hstack([state, action]).reshape(1, -1)

m, v = gp.predict(inp)

mean_next.append(state[i] + m[0][0])

var_next.append(v[0][0])

next_state = np.array(mean_next)  # Sample or use mean

reward = -abs(next_state[0] - 60) - 10 * max(0, next_state[2] - 80)

total_r += reward

state = next_state

return -total_r  # Minimize negative return

from SciPy. Optimize import minimize

initial_params = np.random.randn(5)

result = minimize(expected_return, initial_params, method='L-BFGS-B')

optimal_params = result.x

Implement PETS (Ensemble-Based): Use neural ensembles for dynamics, CEM for planning with trajectory sampling.

Python example (PyTorch for ensembles, NumPy for CEM):

import torch

import torch.nn as nn

import torch.nn.functional as F

import torch.optim as optim

import numpy as np  # For CEM (PyTorch-compat via .numpy())

class DynamicsNet(nn.Module):

def __init__(self, state_dim, action_dim):

super().__init__()

self.fc1 = nn.Linear(state_dim + action_dim, 128)

self.fc2 = nn.Linear(128, state_dim * 2)  # Mean + logvar

def forward(self, s, a):

inp = torch.cat([s, a], dim=-1)

h = F.relu(self.fc1(inp))

out = self.fc2(h)

mean, logvar = torch.chunk(out, 2, dim=-1)

logvar = torch.clamp(logvar, -10, 10)  # Clamp for stability (modern practice)

return mean, torch.exp(logvar)

# Train ensemble (e.g., 5 nets) with device handling

device = torch.device("cuda" if torch.cuda.is_available() else "cpu")

ensemble = [DynamicsNet(4, 1).to(device) for _ in range(5)]

optimizers = [optim.Adam(net.parameters(), lr=0.001) for net in ensemble]

states_tensor = torch.tensor(states, dtype=torch.float32, device=device)

actions_tensor = torch.tensor(actions, dtype=torch.float32, device=device)

next_deltas = torch.tensor(next_states - states, dtype=torch.float32, device=device)

for epoch in range(100):

for net, opt in zip(ensemble, optimizers):

opt.zero_grad()

mean, var = net(states_tensor, actions_tensor)

loss = torch.mean((mean - next_deltas)**2 / var + torch.log(var))

loss.backward()

opt.step()

# PETS Planning with CEM (NumPy for sampling; can torch-ify with torch.distributions)

def cem_planning(state, horizon=10, num_samples=100, elite=10):

mean = np.zeros(horizon)

std = np.ones(horizon)

state_tensor = torch.tensor(state, dtype=torch.float32, device=device).reshape(1, -1)

for _ in range(10):  # CEM iterations

actions = np.random.normal(mean, std, (num_samples, horizon))

rewards = np.zeros(num_samples)

for i in range(num_samples):

s = state.copy()

for t in range(horizon):

a = torch.tensor(actions[i, t:t+1].reshape(1,1), dtype=torch.float32, device=device)

deltas = [net(state_tensor, a)[0].detach().cpu().numpy().flatten() for net in ensemble]

delta = np.mean(deltas, axis=0)  # Ensemble mean

s += delta

r = -abs(s[0] - 60) - 10 * max(0, s[2] - 80)

rewards[i] += r

elite_idx = np.argsort(rewards)[-elite:]

mean = actions[elite_idx].mean(axis=0)

std = actions[elite_idx].std(axis=0) + 1e-6

return mean[0]  # First action

# Use in loop: action = cem_planning(current_state)

Validate and tune: Roll out policies in env, compute average return; tune hyperparameters (e.g., horizon, ensemble size).

Python example (gymnasium for evaluation):

def evaluate(policy_func, episodes=10):

total_rewards = []

for _ in range(episodes):

state = env.reset()

ep_reward = 0

for _ in range(100):  # Episode length

action = policy_func(state)  # For PILCO: policy(optimal_params, state); for PETS: cem_planning(state)

state, reward, _, _ = env.step([action])

ep_reward += reward

total_rewards.append(ep_reward)

return np.mean(total_rewards)

pilco_return = evaluate(lambda s: policy(optimal_params, s))

pets_return = evaluate(lambda s: cem_planning(s))

print(f"PILCO avg return: {pilco_return}, PETS: {pets_return}")

Deploy in prod and integrate into OT loop: Load models on edge, compute actions via PILCO gradients or PETS sampling, adjust VFD if uncertainty high. Monitor via analytics platform like plc-gbt.

Python example (Real-time Loop):

while True:

# e.g., via OPC UA: pressure, flow, temps

current_state = get_sensor_data()

action = cem_planning(current_state)  # Or PILCO policy

env.step([action])  # Send to PLC (VFD speed)

time.sleep(Ts)

Monitor, Retrain, and Improve:

Monitor returns/uncertainty via analytics platform (e.g., variance thresholds trigger alerts).

Retrain episodically with new rollouts to handle DVs.

Fuse PETS ensembles with SINDy for sparse priors; integrate PILCO GPR with Koopman for linearized uncertainty—enabling autonomous governing (e.g., throttling mash coolers if temp risk high), propelling efficiency and dominance.

PILCO: GPR dynamics model, optimizes expected return via gradients. Ref: https://mlg.eng.cam.ac.uk/pilco/
PETS: Neural ensembles for uncertainty, trajectory sampling for planning. Ref: https://arxiv.org/abs/1805.12114

PILCO and PETS Equations: (No explicit equations given; variables inferred from context and refs.)

f: Dynamics model (GPR for PILCO, neural ensemble for PETS), maps state-action to next state, unitless.

xt​: State at time t, in ℝn.

ut​: Action (MV) at time t, in ℝm.

rt​: Reward at time t, in ℝ, performance metric (e.g., tracking error).

For PILCO: See GPR variables (m(x), k(x, x')).

For PETS: Ensemble predictions, no explicit equation, but uses p(xt+1∣xt, ut).

Policy-Gradient RL (Deep Deterministic Policy Gradient (DDPG), Twin Delayed Deep Deterministic Policy Gradient (TD3), Soft Actor-Critic (SAC))
When to Use: For continuous-action spaces in uncertain environments; model-free for direct policy learning.  For this example, we will use distillate proofing control as our example: A highly uncertain process where alcohol concentration (proof) must be maintained (e.g., 100 -160 proof SP) – Low Wine Average setpoint = 130 proof and High Wine Average setpoint = 140 Amid many potential disturbance variables causing proof disturbances like varying beerfeed composition (total mash density, %ABV, recipe differences, etc.), still temperature drifts due to varying steam supply pressure, column pressure fluctuations, column beer bottom level, and others which cause risk of producing off-spec product or energy waste. DDPG/TD3/SAC can learn deterministic or stochastic policies to adjust MVs (e.g., reflux ratios or steam inputs) for optimal proof tracking, incorporating exploration for safe adaptation.  The deadtime and lag between Column vapor temperature and vapor condensing back to liquid and running thru the proofing meter has a very long and dependent on its own set of disturbance variables such as reflux rate, cooling water flow and temperature, vapor concentration, ect.   This algorithm can unlock novel and innovative IT/OT hybrid solutions taking advantage of edge agents adjacent to PLCs for real-time policy execution amid long lags, fused with custom analytics platforms like plc-gbt for handling composition DVs, driving precise, resilient US spirit production through emerging RL in IIoT ecosystems.

When to use: For continuous-action spaces in uncertain environments; model-free for direct policy learning.

DDPG suits deterministic policies in stable setups (e.g., basic proof adjustment).

TD3 adds twin critics for reduced overestimation in noisy loops; SAC incorporates entropy for better exploration in variable processes like distillate proofing control as our in our example.

DDPG/TD3/SAC can learn deterministic or stochastic policies to adjust multiple CV / MVs (e.g., reflux ratios or steam inputs) or even add feedforward weighting to other control loop algorithms - for optimal proof tracking, incorporating exploration for safe adaptation. The deadtime and lag between Column vapor temperature and vapor condensing back to liquid and running thru the proofing meter has a very long and dependent on its own set of disturbance variables such as reflux rate, cooling water flow and temperature, vapor concentration, ect.

Why use: Policy-gradient RL like DDPG, TD3, and SAC excels in distillate proofing by optimizing actions directly via gradients on expected returns, addressing PID's limitations in handling continuous, high-dimensional uncertainty (e.g., long lags in proof metering or noisy sensors). Why these over alternatives?

Direct Policy Focus: Unlike value-based RL or model-based (PILCO/PETS), they learn policies μ(s∣θ) or π(a∣s) end-to-end, enabling smooth MV adjustments (e.g., reflux for proof stability) without explicit dynamics modeling—crucial for black-box OT systems with deadtimes.

Handling Uncertainty and Bias: Models handle long lags (e.g., vapor-to-liquid delays) via replay buffers; DDPG provides baseline determinism; TD3 mitigates overestimation with clipped twins and delayed updates for safer learning in safety-critical loops; SAC adds entropy regularization for balanced exploration/exploitation, preventing premature convergence in despite variable beerfeed properties.

Interpretability and Innovation: Actor-critic architecture allows IT/OT fusion: Edge critics for lag-aware evaluations, cloud actors retrain on streamed data for adaptive governing (e.g., penalizing proof deviations while rewarding energy efficiency amid composition DVs). Over SINDy/Koopman (discovery/linearization), these add goal-oriented RL; vs. GPR, they close the feedback loop for long-term optimization—vital for manufacturing dominance through autonomous, risk-mitigated control.

How to for this use case: We will review implementation of DDPG, TD3, and SAC for distillate proofing: Learn policies to optimize reflux/steam actions (MV: 0-1 normalized) for proof SP (state: current proof, temp, pressure, beerfeed %ABV), rewarding close tracking (-error^2) while penalizing overproof or energy overuse, accounting for long deadtimes/lags in proof metering. Steps use stable-baselines3 for agents and gymnasium for env simulation.

Data Collection and Environment Setup: Define a custom env simulating proofing dynamics with lags; collect initial datasets as needed.

Python example (gymnasium and NumPy):

import gymnasium as gym

from gymnasium import spaces

import numpy as np

from collections import deque

class ProofingEnv(gym.Env):

def __init__(self, lag_steps=20):  # Simulate long lag/deadtime

super().__init__()

# Normalized reflux, steam

self.action_space = spaces.Box(low=0, high=1, shape=(2,))

self.observation_space = spaces.Box(low=np.array([0, 0, 0, 0]), high=np.array([200, 212, 100, 20]), shape=(4,))  # Proof, temp °F, pressure psi, %ABV

self.state = np.array([130, 180, 50, 10])  # Initial: proof, temp, pressure, %ABV

self.sp_proof = 130  # Low Wine SP (or 140 for High)

self.lag_buffer = deque(maxlen=lag_steps) # For deadtime simulation

self.lag_steps = lag_steps

def step(self, action):

# Queue action effects for lag

self.lag_buffer.append(action.copy())

if len(self.lag_buffer) < self.lag_steps:

delayed_action = np.zeros(2)  # No effect yet

else:

delayed_action = self.lag_buffer[0]

reflux, steam = delayed_action  # Denormalized effects

self.state[0] += 0.5 * reflux + 0.2 * steam - 0.1 * (self.state[1] - 180) + 0.05 * np.random.normal()  # Proof update with noise

self.state[1] += 0.3 * steam + 0.1 * (self.state[3] - 10)  # Temp update with %ABV DV

reward = - (self.state[0] - self.sp_proof)**2 - 0.1 * steam  # Track SP, penalize energy

done = abs(self.state[0] - self.sp_proof) < 5  # Terminal if close

truncated = False

return self.state, reward, done, truncated, {}

def reset(self, seed=None):

self.state = np.array([130,180,50,10])+np.random.uniform(-20,20,4)

self.lag_buffer.clear()

return self.state, {}

env = ProofingEnv(lag_steps=20)  # 20-step lag for deadtime

Implement DDPG (Deterministic Policy): Use actor-critic with deterministic policy; train on env.

Python example (stable-baselines3 and PyTorch):

from stable_baselines3 import DDPG

from stable_baselines3.common.noise import NormalActionNoise

n_actions = env.action_space.shape[-1]

action_noise = NormalActionNoise(mean=np.zeros(n_actions), sigma=0.1 * np.ones(n_actions))

model_ddpg = DDPG("MlpPolicy", env, action_noise=action_noise, verbose=1)

Implement TG3 (Twin Delayed DDPG): Adds twin critics and delays for bias reduction; similar setup.

Python example (stable-baseline3):

from stable_baselines3 import TD3

model_td3 = TD3("MlpPolicy", env, verbose=1)

model_td3.learn(total_timesteps=10000, log_interval=10)

model_td3.save("td3_proofing")

Implement SAC (Entropy-Regularized): Maximizes entropy for exploration; auto-tunes alpha.

Python  example (stable-baseline3):

from stable_baselines3 import SAC

model_sac = SAC("MlpPolicy", env, verbose=1)

model_sac.learn(total_timesteps=10000, log_interval=10)

model_sac.save("sac_proofing")

Validate and Tune: Evaluate policies on test episodes; tune hyperparameters (e.g., learning rate) via cloud trials, accounting for lags.

Python example (stable-baselines3 for evaluation):

def evaluate(model, episodes=10):

rewards = []

for _ in range(episodes):

obs, _ = env.reset()

ep_reward = 0

done = False

while not done:

action, _ = model.predict(obs, deterministic=True)

obs, reward, done, _, _ = env.step(action)

ep_reward += reward

rewards.append(ep_reward)

return np.mean(rewards)

ddpg_return = evaluate(model_ddpg)

td3_return = evaluate(model_td3)

sac_return = evaluate(model_sac)

print(f"DDPG avg return: {ddpg_return}, TD3: {td3_return}, SAC: {sac_return}")

Deploy in prod: Load models on edge, query policies for actions; monitor for overestimation. Use Docker for edge deployment; stream episodes to IT OT storage for retraining. Hybrid with MPC: Use RL policies as warm-starts in QP, buffering for deadtimes.

Python example (real-time  loop):

model = SAC.load("sac_proofing")  # Or DDPG/TD3

while True:

obs = get_sensor_data()  # e.g., via OPC UA: proof, temp, pressure, %ABV

action, _ = model.predict(obs, deterministic=True)

env.step(action)  # Send to PLC (reflux, steam)

time.sleep(Ts)

Monitor, retrain, and innovate:

Monitor returns/variance via IT analytics (e.g., TD3 critics for bias alerts).

Retrain episodically with new rollouts to handle DVs.

Improvements: Models handle long lags via buffers; fuse SAC entropy with SINDy for sparse priors in policies; integrate DDPG with Koopman for linearized actors—enabling policy optimization amid deadtimes (e.g., adjusting reflux if proof uncertainty from composition DVs threatens yield)

DDPG, TD3, DDPG: Actor μ(s∣θμ), critic Q(s,a∣θQ). Ref: https://en.wikipedia.org/wiki/Deep_Deterministic_Policy_Gradient
TD3: Twin critics reduce overestimation. Ref: https://arxiv.org/abs/1802.09477
SAC: Entropy-regularized: E[∑trt+αH(π(⋅∣st))].

Ref: https://arxiv.org/abs/1812.05905

Variables:

μ(s∣θμ): Deterministic policy (actor), maps state s to action a, in ℝm.

s: State, in ℝn (e.g., process states and observations).

θμ: Parameters of actor network, in ℝp (p weights).

Q(s,a∣θQ): Action-value function (critic), in ℝ, estimates expected return.

a: Action (MV), in ℝm.

θQ: Parameters of critic network, in ℝq.

rt​: Reward at time t, in ℝ.

α: Entropy regularization coefficient, in ℝ, balances exploration.

H(π(⋅∣st)): Entropy of policy π, in ℝ, measures policy randomness.

π(⋅∣st): Stochastic policy, distribution over actions given state st​

Expectation operator, unitless

Classical Proportional-Integral-Derivative (PID) Tuning Rules:

When to Use: For single-loop control; baseline before MPC in legacy systems.

Ziegler–Nichols (Z-N): Ultimate gain Ku K_u Ku​, period Tu T_u Tu​; Kp=0.6Ku K_p = 0.6 K_u Kp​=0.6Ku​, Ti=0.5Tu T_i = 0.5 T_u Ti​=0.5Tu​, Td=0.125Tu T_d = 0.125 T_u Td​=0.125Tu​. Ref: https://en.wikipedia.org/wiki/Ziegler–Nichols_method

Cohen–Coon: From step response lag τ \tau τ, time constant T. Ref: https://control.com/textbook/pid-control/cohen-coon-tuning-rules/

λ-Tuning (Internal Model Control (IMC)-based): Choose closed-loop λ \lambda λ; Kp=T/(K(λ+τ)) K_p = T / (K (\lambda + \tau)) Kp​=T/(K(λ+τ)), etc. Ref: https://en.wikipedia.org/wiki/Internal_model_control#Lambda_tuning

Variables:

Kp​: Proportional gain, unitless or scaled to process units.

Ku​: Ultimate gain, unitless, gain causing sustained oscillations.

Ti​: Integral time, in seconds, for reset action.

Tu​: Ultimate period, in seconds, oscillation period at Ku​.

Td​: Derivative time, in seconds, for rate action.

T: Process time constant, in seconds, from step response.

K: Process gain, unitless or in output/input units.

λ: Closed-loop time constant, in seconds, user-specified.

τ: Process lag, in seconds, from step response.

Internal Model Control (IMC):

When to Use: For processes with delays; robust against model mismatch.

Decompose Gp(s) ≈ G − (s)G + (s) (minimum-phase and all-pass).

Controller: Gc(s) = G−−1(s)⋅11 + λs1​.

Variables:

Gp(s): Process transfer function, in ℝ (frequency domain), maps inputs to outputs.

s: Laplace variable, in s-1.

G-s: Minimum-phase part of process model, in ℝ.

G + (s): All-pass part (e.g., delays), in ℝ.

Gc(s): Controller transfer function, in ℝ.

G−−1(s): Inverse of minimum-phase model, in ℝ.

λ: Filter time constant, in seconds.

Ref: https://en.wikipedia.org/wiki/Internal_model_control

Relay Feedback:

When to Use: For auto-tuning PID without open-loop tests.
Relay induces limit cycles; infer Ku​, Tu​ for Z-N tuning.

Ku​: Ultimate gain, unitless, from relay-induced limit cycle.

Tu​: Ultimate period, in seconds, from limit cycle.

Ref: https://en.wikipedia.org/wiki/Relay_autotuning (Note: Page may redirect; see control literature).

Genetic Algorithm (GA):

When to Use: For optimizing non-differentiable objectives, like tuning hybrid controllers.

Evolve population {θ_i} via selection, crossover, mutation to minimize J(θ).

Variables:

J(θ): Objective function, in ℝ, performance metric (e.g., control error).

θ: Parameter vector, in ℝ^p, candidate solution (e.g., controller gains).

{θi}: Population of parameter vectors, set of vectors in ℝ^p.

Ref: https://en.wikipedia.org/wiki/Genetic_algorithm

Custom Algorithm Creator:

When to Use: For bespoke IT/OT hybrids, e.g., subspace model + neural residuals for drift adaptation.

Build in Python via scikit-learn: https://scikit-learn.org/stable/developers/develop.html#rolling-your-own-estimator

Innovation: Augment linear grey-box with SINDy-discovered terms for physics-ML fusion.

Table 1..1 - Pros/Cons of Key Algorithms: Usage

| Algorithm Group | Pros | Cons |
| --- | --- | --- |
| Linear ID (1-3) | Fast, interpretable, low data needs | Limited to linear dynamics |
| Nonlinear/ML (4-7) | Handles complexity, uncertainty | Data-hungry, less interpretable |
| RL/Advanced (8-10) | Adaptive to changes | Computationally intensive, safety risks |
| Classical Control (11-14) | Simple deployment | Poor for multivariable/disturbances |

Appendix:

State Estimation:

For unmeasured states in subspace/MPC: Use Kalman filter
xk+1 = Axk + Buk + K(yk – Cxk)\hat{x}_{k+1} =  \hat A{xk} + B uk + K (yk - \hat C{xk} xk+1 ​= Axk​ + Buk ​+ K(yk​ − Cxk​),

where K is gain from Riccati equation. Integrate for robustness.

**Example:

GOAL: Create a Python-based recipe from process data to subspace model to MPC with explicit feedforward. Replace PID loops with one predictive, multivariable controller handling 6 DV disturbances (individually/combined) for resilient manufacturing.

Process Definition: Identify variables.

State: x ∈ ℝn (n chosen via subspace or physics).

MV: u(k)

DVs: d(k) = [d1, …, d6]T

PVs: y = [y1, y2, y3]T, SP: r for y1.

Data Collection: Excite u and each di with Pseudo-Random Binary Sequence (PRBS) or steps (independently/combined). Log y1, y2 at sampling Ts. Compute y3 = fdrv(y1, y2).

Model Identification: Use N4SID to estimate

xk+1 = Axk + Buk + Edk, yk = Cxkxk+1 = Axk + Buk + Edk, \quad yk = Cxk xk+1 ​= Axk ​+ Buk ​+ Edk​, yk ​= Cxk,​

where A ∈ ℝn×n, B ∈ ℝn×1, E ∈ ℝn×6, C ∈ ℝ3×n.

Python: See algorithm 2 snippet.

Model Validation:

One-step ahead: Compare \hat{y}(k|k-1) vs. y(k).

Multi-step: Simulate open-loop over horizon N.
Residuals: Ensure whiteness (e.g., autocorrelation test).

Metrics: Fit Percentage (FIT) = 100 (1 - |y - \hat{y}|_2 / |y - \bar{y}|_2).

Python example:

from scipy.signal import lsim

y_sim, _, _ = lsim((A, [B, E], C, 0), np.hstack((u, d)), t)  # Simulate

MPC Formulation: At each k, solve Quadratic Program (QP):
min⁡{uk+i}i=0N−1∑i=1N∥y1,k+i∣k−r∥Q2+∑i=0N−1∥Δuk+i∥R2\min_{\{u_{k+i}\}_{i=0}^{N-1}} \sum_{i=1}^N \| y_{1,k+i|k} - r \|_Q^2 + \sum_{i=0}^{N-1} \| \Delta u_{k+i} \|_R^2 min{uk+i​}i=0N−1​​∑i=1N​∥y1,k+i∣k​−r∥Q2​+∑i=0N−1​∥Δuk+i​∥R2​
s.t. xk+i+1∣k=Axk+i∣k+Buk+i+Edk+i x_{k+i+1|k} = A x_{k+i|k} + B u_{k+i} + E d_{k+i} xk+i+1∣k​=Axk+i∣k​+Buk+i​+Edk+i​, yk+i∣k=Cxk+i∣k y_{k+i|k} = C x_{k+i|k} yk+i∣k​=Cxk+i∣k​.
Constraints: u_min ≤ u ≤ u_max, Δu_min ≤ Δu ≤ Δu_max. For robustness, add slack variables for soft constraints.

Python (using cvxpy for offline testing):

import cvxpy as cp

u_opt = cp.Variable(N)  # Horizon N

# Define objective/constraints via matrices; solve with qpOASES for real-time

prob = cp.Problem(objective, constraints)

prob.solve()

Feedforward Compensation:

Individual: For each di, static gain Kff i = - (C (I - A-1 E[:,i]) / (C (I-A-1 B))) to cancel effect on y1.

Combined: Kff ∈ ℝ1×6 from steady-state Gd = C (I - A-1 E), Kff = (C (I - A B-1) Gd).
utotal = uMPC + uff, uff = -Kff d(k). For dynamics, use model-based preview if d is predictable.

Offline Testing: Simulate closed-loop with real disturbance profiles. Compare vs. PID cascade/pure feedback MPC using metrics like Integral of Squared Error (ISE) (∫ e2 dt).
Innovation: Test hybrid (MPC + LSTM for residuals) for drift.

Deployment: Embed QP in real-time solver (e.g., qpOASES via Python wrapper or CVXGEN codegen).

Run on edge PC/gateway; send utotal to Programmable Logic Controller (PLC).
Update A,B,E recursively (e.g., Recursive Least Squares (RLS)): θk+1 = θk + Pkϕk (yk − ϕkTθk).

Tip: Stream data to storage  / analytics for periodic retraining, enabling adaptive control at scale.

1.1.2: Why Truncate SVD:

Here are the main reasons and rules of thumb for choosing to truncate:

Noise Filtering / Denoising
– In practical data there’s almost always measurement noise or unmodeled high-frequency components. The smallest singular values often correspond primarily to that noise.
– By dropping all σₖ below some threshold (or below machine precision), you remove spurious components and get a cleaner, more robust approximation.

Model Order Reduction
– In system identification, you rarely want an A matrix of full rank = min(i,j). Instead you pick a reduced order r ≪ min(i,j) that balances fidelity vs. complexity.
– Truncating to the r largest singular values gives you a rank-r Hankel approximation

H(0)≈Ur Σr VrT H^{(0)} \approx U_r\,\Sigma_r\,V_r^TH(0)≈Ur​Σr​VrT​

and leads to an n-state model (with n = r) that’s far easier to analyze and implement in an MPC or observer.

Energy (Variance) Criterion
– A common guideline is to choose r so that

∑k=1rσk2∑k=1min⁡(i,j)σk2  ≥  α\frac{\sum_{k=1}^r \sigma_k^2}{\sum_{k=1}^{\min(i,j)} \sigma_k^2} \;\ge\; \alpha∑k=1min(i,j)​σk2​∑k=1r​σk2​​≥α

for some energy level α (e.g.\ 95 % or 99 %). This ensures you’re retaining the vast majority of the data’s “power.”

Singular‐Value Gap
– Often there’s a clear “elbow” or big drop between σₙ and σₙ₊₁ when you plot them in descending order. That gap suggests a natural cut-off for the dominant modes vs. the rest.

Computational Constraints
– Keeping fewer modes makes subsequent computations (e.g.\ eigenvalue analyses, QP solves inside an MPC) dramatically faster and less memory-hungry—critical if you’re embedding the model on an edge device or PLC gateway.

Cross-Validation / AIC / BIC
– For a more statistically rigorous pick, you can treat r as a hyperparameter and choose it to minimize prediction error on held-out data, or via information-criteria like AIC/BIC that penalize model complexity.

In practice:

If you see σ₁, …, σ₁₀ are each O(1) but σ₁₁, …, σ₂₀ drop to O(10⁻³), you’d likely truncate at r = 10.

If you need a 5-state controller (because your PLC only has capacity for an n≤5 model), you truncate to the top 5 singular values even if the energy retained is, say, 90 %.

If your ultimate goal is denoising and visualization, you might use an energy threshold (e.g. 99 %) to decide r.

In all cases, truncation is the lever that trades off accuracy (keeping more σ’s) against simplicity and robustness (keeping fewer σ’s).

2.1.0. Key Python Libs for Implementing Process Control Algorithms:

python-control

Summary: The python-control library provides tools for analyzing and designing control systems, particularly for linear systems like ARX, Subspace Identification (N4SID, MOESP), and ERA. It supports state-space and transfer function models, system identification (e.g., N4SID), and simulation, making it ideal for baseline linear models in manufacturing (e.g., modeling a heat exchanger). Features include frequency response, pole placement, and LQR design, which pair well with MPC formulations.

Relevant Algorithms: ARX/ARMAX, Subspace Identification, ERA, MPC, PID Tuning, IMC.

URL: https://python-control.readthedocs.io/en/latest/

NumPy

Summary: NumPy is the foundational library for numerical computing in Python, essential for matrix operations (e.g., Rp×m \mathbb{R}^{p \times m} Rp×m manipulations) in all algorithms. It handles arrays, linear algebra (e.g., SVD for ERA, matrix inversions for feedforward), and random number generation (e.g., PRBS for data collection). Critical for implementing custom algorithms or preprocessing data for SINDy, MPC, or RL.

Relevant Algorithms: All (ARX, SINDy, LSTM, RL, etc.).

URL: https://numpy.org/doc/stable/

SciPy

Summary: SciPy builds on NumPy, offering advanced scientific computing tools like signal processing (e.g., for PRBS generation), optimization (e.g., least squares for ARX), and numerical integration (e.g., for simulating x˙=Θ(x)ξ \dot{x} = \Theta(x) \xi x˙=Θ(x)ξ in SINDy). It includes scipy.signal for system simulation (used in MPC validation) and scipy.linalg for SVD in ERA/Subspace ID. Essential for control and identification tasks.

Relevant Algorithms: ARX, Subspace Identification, ERA, SINDy, MPC, PID, IMC.

URL: https://docs.scipy.org/doc/scipy/

scikit-learn

Summary: Scikit-learn is a machine learning library with tools for regression, classification, and feature selection, useful for NARX (via MLPRegressor), GPR (GaussianProcessRegressor), and custom algorithm creation. It supports LASSO for SINDy’s sparse regression and preprocessing (e.g., scaling data for LSTM or RL). Ideal for hybrid models combining ML with physics-based control in IT/OT systems.

Relevant Algorithms: NARX, GPR, SINDy, Custom Algorithm Creator.

URL: https://scikit-learn.org/stable/documentation.html

PyTorch

Summary: PyTorch is a deep learning framework for building and training neural networks, critical for LSTM, NARX (neural implementations), and RL algorithms (DDPG, TD3, SAC). Its automatic differentiation supports custom loss functions (e.g., for MPC with neural residuals), and GPU acceleration enables real-time training on edge devices. Widely used for predictive maintenance and adaptive control in manufacturing.

Relevant Algorithms: NARX, LSTM, DDPG, TD3, SAC, PETS, Custom Algorithm Creator.

URL: https://pytorch.org/docs/stable/

TensorFlow

Summary: TensorFlow is an alternative deep learning framework to PyTorch, supporting LSTM, NARX, and RL (via TensorFlow Agents for DDPG, SAC). It offers robust deployment tools (e.g., TensorFlow Lite for edge devices), making it suitable for IT/OT integration in manufacturing. Its Keras API simplifies neural network design, useful for hybrid models in custom algorithms.

Relevant Algorithms: NARX, LSTM, DDPG, TD3, SAC, PETS, Custom Algorithm Creator.

URL: https://www.tensorflow.org/api_docs

Pysindy

Summary: The pysindy library is specifically designed for Sparse Identification of Nonlinear Dynamics (SINDy), enabling discovery of sparse governing equations (x˙=Θ(x)ξ \dot{x} = \Theta(x) \xi x˙=Θ(x)ξ) from data. It supports LASSO and sequential thresholding, with customizable libraries (e.g., polynomials, trigonometric functions). Ideal for physics-informed ML in automation, enhancing interpretability in manufacturing models.

Relevant Algorithms: SINDy.

URL: https://pysindy.readthedocs.io/en/latest/

Cvxpy

Summary: CVXPY is a convex optimization library for formulating and solving problems like the Quadratic Program (QP) in MPC (min⁡∑∥y−r∥Q2+∥Δu∥R2 \min \sum \| y - r \|_Q^2 + \| \Delta u \|_R^2 min∑∥y−r∥Q2​+∥Δu∥R2​). It supports constraints (e.g., umin≤u≤umax u_{\text{min}} \leq u \leq u_{\text{max}} umin​≤u≤umax​) and interfaces with solvers like qpOASES for real-time control. Essential for MPC deployment in manufacturing plants.

Relevant Algorithms: MPC, IMC (for optimization-based tuning).

URL: https://www.cvxpy.org/

qpOASES

Summary: qpOASES (via Python wrappers like qpoases) is a QP solver optimized for real-time MPC, solving constrained optimization problems efficiently. It’s designed for embedded systems, making it ideal for edge deployment in IT/OT systems (e.g., sending utotal u_{\text{total}} utotal​ to PLCs). Complements CVXPY for high-speed control loops in manufacturing.

Relevant Algorithms: MPC.

URL: https://github.com/coin-or/qpOASES (Python wrapper: https://pypi.org/project/qpoases/)

do-mpc

Summary: The do-mpc library is a comprehensive toolbox for Model Predictive Control, supporting nonlinear and robust MPC with discrete or continuous-time models. It integrates system identification, state estimation (e.g., Kalman filters), and optimization, making it ideal for replacing PID loops in multivariable manufacturing processes. Includes simulation and visualization tools.

Relevant Algorithms: MPC, State Estimation, Custom Algorithm Creator.

URL: https://www.do-mpc.com/en/latest/

GPy

Summary: GPy is a Gaussian Process framework for regression and classification, perfect for implementing GPR and PILCO’s probabilistic dynamics models. It supports various kernels (e.g., RBF) for uncertainty quantification, crucial for robust control in manufacturing under small-data regimes. Enhances predictive models in IT/OT integration.

Relevant Algorithms: GPR, PILCO.

URL: https://sheffieldml.github.io/GPy/

Gymnasium

Summary: Gymnasium (successor to OpenAI Gym) provides environments for reinforcement learning, supporting DDPG, TD3, SAC, and PETS. It allows custom environments for manufacturing processes (e.g., optimizing a reactor’s yield), enabling RL testing before deployment. Pairs with PyTorch/TensorFlow for policy training in adaptive control.

Relevant Algorithms: DDPG, TD3, SAC, PETS.

URL: https://gymnasium.farama.org/

stable-baselines3

Summary: Stable-Baselines3 offers reliable implementations of RL algorithms (e.g., DDPG, TD3, SAC) built on PyTorch. It simplifies training and testing policies for continuous control tasks, like optimizing process setpoints in uncertain environments. Ideal for model-free RL in manufacturing automation.

Relevant Algorithms: DDPG, TD3, SAC.

URL: https://stable-baselines3.readthedocs.io/en/master/

Deap

Summary: DEAP is a library for evolutionary algorithms, including Genetic Algorithms (GA) for optimizing non-differentiable objectives (e.g., tuning PID or hybrid controllers). It supports parallelization and custom fitness functions, making it suitable for complex manufacturing optimization tasks in IT/OT systems.

Relevant Algorithms: GA.

URL: https://deap.readthedocs.io/en/master/

Kooplearn

Summary: Kooplearn is a library for Koopman operator-based modeling, enabling linearization of nonlinear dynamics (ψ(xk+1) ≈ Kψ(xk) for linear MPC. It uses data-driven methods (e.g., DMD, EDMD) to approximate the Koopman operator, ideal for complex manufacturing processes like fluid dynamics.

Relevant Algorithms: Koopman Operator Approaches.

URL: https://github.com/kooplearn/kooplearn

Notes:

Cross-Library Integration: Many algorithms require multiple libraries (e.g., SINDy uses pysindy with NumPy/SciPy for matrix operations, scikit-learn for LASSO).

IT/OT Relevance: Libraries like cvxpy, qpOASES, and do-mpc support real-time control on edge devices, while PyTorch and GPy enable cloud-based training for adaptive models.

Innovation: Combining pysindy (physics-informed) with PyTorch (neural residuals) can create hybrid models for manufacturing, aligning with the document’s “Custom Algorithm Creator” vision.